SA WG2 Meeting #170	S2-2506643

25 – 29 Aug, 2025, Goteborg, SE

Source:	vivo

Title:	[WT#8, 6G IoT] Unified Architecture for 6G IoT and eMBB

Document for:	Agreement

Agenda Item:	20.6.8

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposals a unified design for 6G IOT and eMBB in 6G.

1. Justifications

  - Architectural Motivation & Gap Analysis:

In the era of 6G, the mobile system will provide its services to a myriad of devices, from low-end IoT devices to high-end advanced eMBB devices.

Table-1 Three device types with mandatory baseband and RF capability

|  | Device type A (Smart phone, immersive eMBB device, CPE) | Device type B (Reduced capability eMBB device, wearable/XR) | Device type C (6G IoT device) |
| --- | --- | --- | --- |
| Typical Device/service | Smartphones, immersive eMBB, CPE, | Reduced capability eMBB, e.g. wearable/XR, etc | IoT |
| Downlink peak data rate | ~10Gbps | 200Mbps ~ 1Gbps | ~10Mbps |
| Uplink peak data rate | ~2Gbps | 50Mbps ~ 200Mbps | ~5Mbps |
| Supported maximum downlink channel bandwidth | At least 200 MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Supported maximum uplink channel bandwidth | [100/200] MHz | 100 MHz | ~5MHz for FDD ~20MHz for TDD |
| Supported maximum Downlink MIMO layer | At least 4 layers | 1~2 layers | 1~2 layers |
| Supported maximum Uplink MIMO layer | At least 2 layers | 1 layer | 1 layer |

Three device types shown in Table 1, from high-tier device type A including Smartphones, immersive eMBB and CPE, to low-tier device type C for IoT, and device type B in-between for reduced capability eMBB, can be considered. It is noted that, ambient IoT is a different type of IoT evolved in a separate track, thus is not considered here.

For each device type, a minimum set of mandatory features for baseband and RF can be defined, e.g., channel bandwidth, number of MIMO layer as well as peak data rate as shown in Table 1.

Considering lower cost and complexity, and also extreme coverage, type C device (6G IoT device) can be categorized as the following aspects:

- bandwidth around 5MHz: According to cost reduction analysis in LTE and NR, bandwidth around 5MHz for low band in FR1 (i.e. FDD bands) achieves sufficiently low cost and complexity, and further reduction of bandwidth would only bring marginal benefit. Meanwhile, with 5MHz bandwidth for device type C, common design for common channels and signals compatible with all three UE types can be easily supported. For higher band in FR1 (i.e. TDD bands), if supported for IoT, minimum bandwidth around 20MHz would be reasonable choice, scaling with the subcarrier spacing, to facilitate unified design for different bands in FR1.

- single Tx & Rx: Similar as in LTE and NR IoT device as well as Redcap/eRedcap, type C device will typically support single Tx & Rx antenna (and optionally two Rx antennas), and may have smaller antenna gain due to complexity and smaller device form factors restriction. Thus, downlink MIMO layer no larger than 2 is supported by type C device.

- up to 10 Mbps: To meet various service requirements/scenarios, e.g., from smart cities, smart metering to immersive multimedia, different device types supporting different data rates shall be considered in the first release of 6GR. With limited bandwidth and limited number of Rx/Tx antennas, type C device can support up to 10 Mbps.

Proposal 1: 6GS shall consider the 6G IoT device as above.

<Lessons on different device types definition in 5GS>

In 5GS, the new device types (e.g., (e)Redcap, (e)MTC, NB-IoT) are not supported in 1st release(release 15) of 5GS and has been introduced in later releases, causing too many different sub-categories serving similar service, resulting in serious dilemma in deployment e.g.:

- NB-IoT with category NB1 & NB2, and MTC with category M1 & M2.

- DRX and eDRX.

- CIoT service request and common Service Request, CM idle and CM suspend etc.

- CIoT UP and common UP.

- CP CIoT PDU Session and common PDU Session.

Prpposal2: 6G IoT device should be introduced together with 6G eMBB device in the first release to avoid compatibility issues in later releases.

In 5GS, the definition of IoT and eMBB stem from different technical origins, follow independent standardization paths, and there is no in-depth collaboration between the core network and access network. e.g.:

- Separate definition will cause fragmented standardization and restricted technical evolution.

- Separated definition will cause fragmented RATs and Core network, and will cause high network deployment/operation costs and low resource utilization.

- Separate definition will cause fragmented terminal chipset, and will causes high terminal deployment cost.

Observation: This "non-unified" design has exposed a series of drawbacks in the implementation process of 5G.

Prpposal3: Single 6GC compatible to both 6G IoT device and eMBB devices and the unified design is necessary for both devices in first releases.

<benefit of unified design>

- The technologies of 6G IOT (such as PSM, eDRX, small data transmission, bigger coverage, and narrowband transmission) can feed back into the eMBB device in perspective scenarios e.g., the eMBB device is in low-power mode. When the eMBB utilize the 6G IoT features, the service and IP continuity should be guaranteed to minimize the impact to the ongoing services.

- Single RAT and Single Core to save the network deployment/operation costs and maximum resource utilization, in case the operator is willing to serve both 6G IoT device and eMBB devices.

- Single terminal chipset to save terminal deployment costs.

- One standardization to easily for maintenance.

- Better coexistence and forward compatibility to migrate to future systems.

1.2. Technical Impact

To achieve a unified design adaptive to both 6G IOT and eMBB devices, the following aspects needs to be considered:

- Maximum common design, e.g., the following:

- Support Common NAS protocol

- Unified DRX

- Unified Service Request

- Unified UP& PDU Session

- Small data transmission.

- Allow 6G IOT specific enhancement:

- Power saving mechanism.

- Coverage enhancement.

- Allow eNBB to utilize the technologies of 6G IOT in specific scenarios:

- The traditional IoT features, such as PSM, eDRX, small data transmission, bigger coverage, and narrowband transmission can feed back into the eMBB devices in perspective scenarios, e.g., the eMBB device is in low-power mode; or the eMBB device is in idle but small packets (e.g. heart beat packets) transmission still happens.

- Keep the service/IP continuity when eNBB devices to utilize the technologies of 6G IOT feature in specific scenarios

**** First Change ****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#8: Support to the unified design for both devices in first releases, including the following aspects:

-  Allow Single 6GC and 6G RAN compatible to both 6G IoT devices and eMBB devices

-  Maximum common design (e.g., common NAS, unified DRX, unified Service Request, etc.)

-  Allow 6G IOT specific technologies enhancement.

-  Allow eNBB to utilize the technologies of 6G IOT in specific scenarios.

**** Second Change ****

5.X. Key Issue #X: Key Issue Title

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The definition of 6G IoT device is assumed to be devices with the following characteristics:

-  Bandwidth around 5MHz.

-  Single Tx & Rx.

-  Up to 10 Mbps, but still transmit small data frequently.

-  Massive number.

Editor's Note: The final capability and definition of 6G IoT device will be cooperated with RAN WGs.

The study for 6G IoT includes the following aspects:

- #1: Study how to support a unified design adaptive to both 6G IoT and eMBB devices, including the  following aspects:

-   How to maximum common design for both eMBB and 6G IOT (e.g., common NAS, unified DRX, unified Service Request, etc.)

-   How to enable the eMBB devices to utilize the 6G IOT features in some perspective scenario and how to keep the service/IP continuity when eNBB devices to utilize the technologies of 6G IOT feature in specific scenarios

-   How to support small data transmission.

-   How to support Power saving mechanism.

-   How to reflect the coverage enhancement defined by RAN WGs.

**** End of Changes ****