SA WG2 Meeting #170	S2-2506644

25 - 29 August 2025, Goteborg, Sweden

Source:	China Mobile

Title:	KI#1: New Sol: Solution for UP-based NB-IoT GEO IMS Voice

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#1.

# 1	Discussion

This solution address Key Issue #1: "support IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC."

For non-NB-IoT UEs, during attach procedure, the UE sets up a PDN connection for internet service with a default bearer. If the UE supports IMS service, it then establishes another PDN connection for IMS service with a default bearer after attach. At this point, the UE has two DRBs - default bearer for Internet service and the IMS default bearer carries IMS signaling. When the UE initiates call, it creates a dedicated bearer for IMS voice. This process requires 3 DRBs and dedicated bearer setup, which has limitations when the UE access via NB-IoT.

The solution focuses on releasing the internet service PDN connection before call setup, and establishing a dedicated bearer for IMS voice under NB-IoT(GEO) access during call setup procedure, to ensure QoS for the UE's IMS voice call.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x | x |  |  |  |

* * * Next Change * * *

- X	Solution #X: <Solution for UP-based NB-IoT GEO IMS Voice>

6.X.1	High level principles

This solution resolves Key Issue #1 Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC,  the following principles are included in this solution:

-	This solution is based on UP+IP.

-	The default bearer is used to transmit IMS signaling, while the dedicated bearer is used to transmit IMS voice.

-	The Attach and IMS PDN connection establishment procedures defined in TS 23.401 can be reused.

-	The UE initiates the release of the Internet default bearer before call origination or upon receiving a call.

-	The QoS parameters for the IMS voice dedicated bearer need to be defined to accommodate the NB-IoT NTN 	via GEO satellite scenario.

6.X.2	Description

#### This proposal addresses the limitations for UEs accessing via NB-IoT(GEO) in supporting IMS voice services. Currently, for UEs accessing via E-UTRAN, 3GPP specifications TS 23.401 and TS 23.228 define the standard procedures for establishing IMS voice calls. However, when a UE accesses via NB-IoT, the following two limitations exist:

- Lack of Dedicated Bearer Support: Per TS 23.401 clause 4.7.2.1, Dedicated Bearers are not supported over NB-IoT. The PDN Gateway utilizes the RAT Type information to ensure that no Dedicated Bearers are activated when the UE is accessing via NB-IoT.

- Data Radio Bearer (DRB) Quantity Limitation: According to TS 36.331 clause 4.7.2.1, A NB-IoT UE only supports 0, 1 or 2 DRBs,.

These limitations make it difficult to provide voice services over NB-IoT using the current IMS voice bearer setup. To solve the problems of DRB limits and missing Dedicated Bearers for UEs using NB-IoT (GEO) satellites, this solution improves the existing TS 23.401 and TS 23.228 procedures. This solution aims to support IMS GEO Voice Service without making big changes to current standard UP procedures.

#### 6.X.3.1	Attach and IMS PDN connection establishment

The following figure depicts the attach and PDN connection establishment procedure for IMS session over the UP-based NB-IoT (GEO) deployment.

Figure 6.X.3.1-1: Attach and IMS PDN connection establishment Procedure

1-2. The UE accesses the EPC network via GEO satellite and initiates an EPC attach request, with the message carrying RAT Type="NB-IoT(GEO)".

3. The MME performs the subscriber authentication procedure with the HSS and notifies the HSS of "GEO IMS Voice homogeneous support"..

4.  The MME initiates a default PDN connection for the user's Internet data services. MME sends a Create Session Request message to the SGW/PGW, requesting the establishment of the default bearer. The message carries RAT Type="NB-IoT(GEO)", the Internet APN, and the IP PDN Type.

5-9. Same as the procedure specified in clause 5.3.2.1 in TS 23.401. Completes the establishment of the default bearer for Internet service.

10. Before initiating IMS registration, the UE needs to establish an IMS PDN connection. The UE sends a PDN Connectivity Request to the MME, indicating the requested PDN type is IP and the APN is IMS.

11-16. Same as the procedure specified in TS 23.228. Completes the establishment of the default bearer for IMS Signalling.

#### 6.X.3.2	UE requested PDN disconnection

The following figure depicts the UE requested PDN disconnection procedure for IMS session over the UP-based NB-IoT (GEO) deployment.

Figure 6.X.3.2-1: UE requested PDN disconnection

- When the UE accesses via NB-IoT (GEO) and is about to initiate a call, or when the UE receives a call request (INVITE), the UE triggers the Internet PDN disconnection procedure.

- The UE sends a PDN Disconnection Request to the MME, with LBI=9 included, indicating that the default bearer of the Internet PDN connection is to be released.

2-7. Same as the procedure specified in clause 5.10.3 in TS 23.401, completing the deletion of the default bearer for Internet service.

#### 6.X.3.3	MO call

The following figure depicts the mobile originating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.X.3.3-1: MO call Procedure

1. After registration and Internet PDN disconnection procedure, the UE initiates IMS call. The UE sends INVITE message to the P-CSCF via the IMS Signalling default Bearer, carrying the Access Type="NB-IoT(GEO)".

2. The called network returns a 183 Session Progress message to the P-CSCF. The message carries SDP which include media IP/port information.

3. The P-CSCF receive the SDP answer to obtain media stream parameters (e.g., IP/port/protocol). It then sends an AAR to the PCRF. The AAR carries Access Type="NB-IoT(GEO)", requesting media stream resources for the GEO voice call. The AVP carries the destination IP, destination port, and protocol for the IMS voice media stream.

4. The PCRF, based on the UE Access Type="NB-IoT(GEO)", generates PCC rules. It sends an RAR (Re-Auth-Request) to the SGW/PGW. The RAR carries the destination IP, destination port, protocol, and QoS information (QCI=X) for the IMS voice media stream.

5. The SGW/PGW generates GTP-U TEID and initiates the procedure to create a dedicated EPS bearer by sending Create Bearer Request to the MME. The CBR contains Bearer Context information of the dedicated EPS bearer to be created. Bear TFT, SGW GTP-U TEID and Bearer QoS(which includes QCI ,UL/DL MBR and  UL/DL GBR for this dedicated EPS bearer are included in this message.

6. Upon receiving the CBR, the MME allocates the EPS bearer ID and requests the UE to activate the dedicated EPS bearer by sending Activate dedicated EPS bearer context request towards the UE. The message is delivered to the UE being contained in the E-RAB Setup request and the RRC Downlink Direct Transfer over S1AP and RRC interfaces respectively.

7. The MME sends an Update Bearer Response to the S-GW, indicates the establishment of the IMS voice dedicated bearer is completed.

8. The P-CSCF forwards 183 message to the UE.

#### 6.X.3.4	MT call

The following figure depicts the mobile terminating procedure for IMS session establishment over the UP-based NB-IoT (GEO) deployment.

Figure 6.X.3.4-1: MT call Procedure

- 1. After registration, the called UE receives an INVITE message sent by the calling network. The message carries the SDP offer.

- 2. The called UE sends a 183 Session Progress message to the P-CSCF via the IMS Signalling default Bearer. The message carries Access Type="NB-IoT(GEO)" and SDP which include media IP/port information.

- 3. The P-CSCF receive the SDP answer to obtain media stream parameters (e.g., IP/port/protocol). It then sends an AAR to the PCRF. The AAR carries Access Type="NB-IoT(GEO)", requesting media stream resources for the GEO voice call. The AVP (Attribute-Value Pair) carries the destination IP, destination port, and protocol for the IMS voice media stream.

- 4-7. Same as Step 4-7 in Clause 6.X.3.3 MO call. The establishment of the IMS voice dedicated bearer on the called side is completed.

- Editor’s note: Internet PDN re-establishment procedure after IMS voice call release is FFS.

- X.4	Impacts to Services, Entities and Interfaces

UE:

-	Support the establishment of IMS voice dedicated bearers when accessing via NB-IoT (GEO).

-	Support initiating the release of the default bearer for Internet service before call origination or upon receiving a call request when accessing via NB-IoT (GEO).

EPC:

-	Support the establishment of IMS voice dedicated bearers when the UE accesses via NB-IoT (GEO).

* * * End of change * * *