SA WG2 Meeting #170	S2-2506645

25 - 29 August 2025, Goteborg, Sweden

Source:	China Mobile

Title:	KI#2 : New Sol: IMS signaling performance optimization to support voice over GEO

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#2.

# 1	Discussion

To achieve SA1’s IMS voice call setup time KPI over GEO satellites, the end-to-end signaling process needs adjustments for satellite link limitations. Main issues are unavoidable delays and limited bandwidth, which increase call setup time.

The solution focuses on optimize signaling flows, adapting protocol parameters,compressing message and satellite-ground transcoding procedure to reduce end-to-end latency.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * *  *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  | x |  |  |

* * * Next Change * *  *

6.X	Solution #X: <Solution for IMS signaling performance optimization to support voice over GEO>

6.X.1	High level principles

This solution resolves Key Issue #2 IMS enhancement for GEO NB-IoT NTN access,  the following principles are included in this solution:

-	SIP signaling needs to be simplified.

-	The P-CSCF functions act as B2BUA, performing simplification of original SIP messages and subsequent restoration of the simplified messages.

-	The SIP timer values for NB-IoT (GEO) access need to be defined.

-	The IMS-AGW supports transcoding between satellite codecs and terrestrial codecs.

6.X.2	Description

To meet SA1 performance requirements for IMS voice call setup time using GEO satellite, the IMS signaling messages exchanged between UE and IMS network via GEO satellite need to be optimized.

The IMS signaling flow always requires high reliability. It’s  challenge to support IMS signaling exchange between UE and IMS network in the scenarios of voice services via GEO satellite. The transmission data rate of NB-IoT via GEO satellite is very restricted, e.g., 1-3 kbps as described in TR 22.887 [6], which causes long transmission delay for the IMS signaling message delivery. The propagation delay via satellite is associated with these orbit ranges. The UE to ground max propagation delay is estimated to be 272 ms, as specified in TS 22.261 [7]. In addition, the re-transmission of IMS singling via GEO satellite is more frequently than terrestrial access due the radio resource condition.

Considering the above, the message size and number of exchanged signaling messages between UE and IMS network need to be reduced to meet SA1 requirements on IMS call setup time. At the same time, due to the constraints of long transmission delays under GEO satellite acc ess, adjustments must be made to the existing SIP timer settings. In GEO satellite access scenarios, the delay mainly comes from the SIP request and response signaling exchanged between the UE and P-CSCF over the Gm interface. To reduce transmission delay, the size of SIP messages needs to be simplified. At the same time, to reduce one-way propagation delay, the IMS call flow can be optimized. The following solutions focus on optimizing the SIP URI, SIP header, SIP body, and SIP timers.

6.X.3	Procedures

6.X.3.1	 Call Procedure Optimization

To reduce call setup delay for users accessing via GEO satellites, the SIP call establishment procedure can be optimized. Given that the primary delay originates from signaling transmission between the UE and the core network, SIP message exchanges over the Gm interface can be reduce by delete unnecessary messages.

6.X.3.1.1 Mobile origination procedure

The following figure depicts the MO procedure in the optimized IMS call flow for NB-IoT access over GEO satellites.

Figure 6.X.3.1.1: Mobile origination procedure

- The UE sends simplified INVITE request to the P-CSCF through NB-IoT(GEO) access.

- Upon receiving the simplified INVITE request, the P-CSCF authenticates the originating UE's identity and converts the simplified INVITE request into a standard INVITE request and forwards it to the I/S-CSCF along the path determined by the originating UE’s latest registration process.

- The I/S-CSCF sends a standard 100 Trying response to the P-CSCF. Before sending the message, the I/S-CSCF performs the following operations: stores the Contact header from the response message and sends the response based on the Via header content.

- The P-CSCF sends a simplified 100 Trying response to the originating UE. Before transmission, the P-CSCF stores the Contact, From, and To information.

- The I/S-CSCF sends a standard 183 Session Progress response to the P-CSCF.

- The P-CSCF sends an AAR to the PCRF to trigger the IMS voice Dedicated bearer establishment for the originating side.

- The P-CSCF forwards a simplified 183 Session Progress response to the originating UE.

- UE sends a simplified PRACK message to the P-CSCF.

- P-CSCF converts and sends a standard PRACK message to the I/S-CSCF.

- I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- P-CSCF forwards a simplified 200 OK response to the originating UE.

- The P-CSCF initiates a standard UPDATE message to the I/S-CSCF for the called side.

- The I/S-CSCF sends a 200 OK response to the P-CSCF.

- When the called UE rings, I/S-CSCF sends a standard 180 Ringing response to the P-CSCF. If the called supports Polyphonic ringtones service, the I/S-CSCF removes the P-Early-Media header from the 180 Ringing response and skips subsequent procedures.

- P-CSCF sends a simplified 180 Ringing response to the originating UE.

- P-CSCF sends a standard PRACK message to the I/S-CSCF.

- I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- After the called UE answers, the I/S-CSCF sends a standard 200 OK response to the P-CSCF.

- P-CSCF forwards a simplified 200 OK response to the originating UE.

- UE acknowledges the 200 OK response with a simplified ACK message.

- P-CSCF sends a standard ACK message to the I/S-CSCF.

6.X.3.1.2  Mobile termination procedure

The following figure depicts the MT procedure in the optimized IMS call flow for NB-IoT access over GEO satellites.

Figure 6.X.3.1.2: Mobile termination procedure

- Upon receiving the INVITE request, if the called party supports CRBT, the I/S-CSCF removes the P-Early-Media header field from the INVITE request before forwarding it to the P-CSCF.

- The P-CSCF, after receiving the INVITE request, checks whether the SIP message is intended for a satellite user. If not, it converts the INVITE into a simplified INVITE message and forwards it to the called UE.

- The called UE sends a simplified 100 Trying response to the P-CSCF.

- The P-CSCF converts and forwards a standard 100 Trying response to the I/S-CSCF.

- The called UE sends a simplified 183 Session Progress response to the P-CSCF.

- The P-CSCF sends an AAR to the PCRF to trigger the IMS voice Dedicated bearer establishment for the terminating side.

- The P-CSCF forwards a standard 183 Session Progress response to the I/S-CSCF.

- The I/S-CSCF sends a standard PRACK message to the P-CSCF.

- The P-CSCF converts and forwards a simplified PRACK message to the called UE.

- The called UE sends a simplified 200 OK response to the P-CSCF.

- The P-CSCF converts and forwards a standard 200 OK response to the I/S-CSCF.

- The I/S-CSCF sends a standard UPDATE message to the P-CSCF.

- The P-CSCF sends a standard 200 OK response to the I/S-CSCF.

- The called UE sends a simplified 180 Ringing response to the P-CSCF, which caches this simplified response.

- The P-CSCF converts the cached simplified 180 Ringing into a standard 180 Ringing response and forwards it to the I/S-CSCF.

- The I/S-CSCF sends a standard PRACK message to the P-CSCF.

- The P-CSCF sends a standard 200 OK response to the I/S-CSCF.

- The called party answers, and the called UE sends a simplified 200 OK response to the P-CSCF.

- The P-CSCF converts and forwards a standard 200 OK response to the I/S-CSCF.

- The I/S-CSCF sends a standard ACK message to the P-CSCF.

- The P-CSCF converts and forwards a simplified ACK message to the called UE.

6.X.3.2	 Call Procedure Optimization

- 6.X.3.2.1	 SIP URI simplification

The SIP URI appears multiple times in SIP messages (e.g., in the request line, Via, From/To, Contact headers, etc.), leading to redundant transmission and parsing overhead. The table below shows the mandatory and common fields where SIP URIs appear：

Table 6.X.3.2.1-1: SIP URI in SIP Headers

| Header | Example | Presence |
| --- | --- | --- |
| Request-Line | sip:460080123456789@ims.mnc008.mcc460.3gppnetwork.org | M |
| From | From: <sip:+8613512345678@ims.mnc008.mcc460.3gppnetwork.org>; tag=12345678 | M |
| To | To: <sip: +8613912345678@ims.mnc008.mcc460.3gppnetwork.org>;tag=12345678 | M |
| Contact | Contact: "Mr. Watson"<sip: +8613512345678@ims.mnc008.mcc460.3gppnetwork.org>; q=0.1; expires=60 | M |
| Via | Via: SIP/2.0/UDP +8613512345678@ims.mnc008.mcc460.3gppnetwork.org:5060;rport;branch=z9hG4bK123456 | M |
| Route | Route: <sip:server10.ims.mnc008.mcc460.3gppnetwork.org;lr> | O |

SIP URIs may also appear in the SIP SDP message body (e.g., in the <o> and <c> fields). Therefore, simplifying SIP URIs should be prioritized. Typically, a SIP URI is approximately 45–55 bytes in length. Through optimization, this can be reduced to 30–40 bytes, achieving a compression rate of around 30%. Below is an example of a simplified SIP URI:

Table 6.X.3.2.1-2: Example of SIP URI Simplification

| SIP URI | SIP URI after Simplification | Reduction Size |
| --- | --- | --- |
| 460080123456789@ims.mnc008.mcc460.3gppnetwork.org | 460080123456789@i.008.460.3g.org | 17 B |

6.X.3.2.2	 SIP header simplification

In terms of simplifying SIP header fields, an efficient compression strategy can be achieved by streamlining the message headers. Specifically, based on the requirements of RFC 3261, only the mandatory header fields (such as Via, From, To, Call-ID, CSeq, Contact, and Max-Forwards) should be retained in SIP messages, as they are mandatory for message routing, transaction integrity, and session control. Non-essential headers (e.g., User-Agent, Allow, Supported) can be dynamically removed depending on the actual scenario to reduce redundant data.

Furthermore, for the retained essential headers, RFC 3261 permits the use of abbreviated forms to further compress the message size. This optimization can reduce the size of a single header field by 1 to 14 bytes. A simplified example is provided in the following table:

Table 6.x.3.2-1: Simplified SIP headers by referencing RFC 3261

| Full Name | Short Form | Reduction |
| --- | --- | --- |
| Call-id | i | 6B |
| Contact | m | 6B |
| Content-length | l | 14B |
| Content-type | c | 11B |
| From | f | 3B |
| Session-Expires | x | 14B |
| Supported | k | 8B |
| To | t | 1B |
| Via | v | 2B |

When UE sends a simplified SIP message to the P-CSCF in the IMS network, the P-CSCF shall perform standardized message reconstruction: First, abbreviated header fields (e.g., "v" restored to "Via") shall be converted to their original format to ensure backward compatibility with downstream IMS network functions. Additionally, mandatory header fields (e.g., Record-Route, Path, P-Asserted-Identity) shall be inserted in accordance with operator-specific policy requirements to enable proper signaling routing, user authentication, and service continuity.

6.X.3.2.3	 SIP message body simplification

The message body in SIP messages shall use the SDP protocol. The SDP protocol supports the description of various session parameters, including media type, media format, media address, etc. For voice communication services, the message body shall retain only the mandatory SDP fields—v, o, s, c, t, m, and a—while delete unnecessary fields such as e, k, and p to reduce message body size. The specific content of each retained line shall remain unchanged. Below is an example of a simplified SDP message body:

v=0 o=Alice 45698 121678 IN IP4 1.1.1.1 s=- c=IN IP4 10.10.10.40 t=0 0 m=audio 10052 RTP/AVP 8 a=rtpmap:8 PCMA/8000

- Figure 6.x.3.2.3-1: Example of simplified SDP

6.X.3.3	 SIP Timer Optimization

In GEO satellite access scenarios, if the default SIP timers (e.g., T1/T2 retransmission timers) designed for terrestrial networks are retained, the propagation delay (~270 ms one-way, ≥540 ms round-trip) will cause timer expiration. This results in unnecessary signaling retransmissions, increasing network congestion and call setup failure rates. To avoid this issue, the timer thresholds should be extended to align with GEO latency characteristics, effectively prevent retransmissions and ensuring reliable SIP protocol operation and session establishment in high-delay environments.

Table 6.X.3.3-1: SIP Timer Optimization Requirements

| Timer | Standard Value | Definition | Suggest Value | Optimization Information |
| --- | --- | --- | --- | --- |
| T1 | 0.5s | Round-trip message delay | 9s | The time required to send a SIP message: 8 × 0.7 KB (average SIP message size) / 1 kbps (satellite link rate) + 2.6 s = 8.2 s, recommended to adjust to 9 s. |
| T2 | 4s | Maximum retransmission interval for non-INVITE request messages and INVITE response messages | 18s | The SIP response message transmission time is approximately 9 s. It is recommended to set T2 ≥ 2 × SIP response transmission time (18 s), ensuring at least 1 retransmission. |
| T4 | 5s | Maximum lifetime of a SIP message in the network | 20s | The time to stop retransmissions must be greater than the T2 timer duration. |
| Timer A | T1 | Retransmission interval for INVITE messages when using UDP transport | 9s | Same as the T1 timer setting. |
| Timer B | 64T1 | INVITE transaction timeout duration | 72s | For the maximum total retransmission duration timer, the intervals are 9 s, 18 s, 36 s, 72 s, set to 72 s for a total of 4 retransmissions. Previously, 0.5 s, 1 s, 2 s, 4 s, 8 s, 16 s, 32 s allowed 7 retransmissions. |
| Timer C | 180s | Proxy server timeout duration for processing INVITE transactions | - | The maximum duration of an INVITE transaction, after process simplification, is less than 180 s. No adjustment is needed. |
| Timer D | 64T1 | Time to wait for retransmitted response messages after receiving a response | 72s | Same as the Timer B setting. |
| Timer E | T1 | Retransmission interval for non-INVITE request messages when using UDP transport | 9s | Same as the T1 timer setting. |
| Timer F | 64T1 | Non-INVITE transaction timeout duration | 72s | Same as the Timer B setting. |
| Timer G | T1 | Retransmission interval for INVITE response messages | 9s | Same as the T1 timer setting. |
| Timer H | 64T1 | Time to wait for an ACK after sending a final response | 72s | Same as the Timer B setting. |
| Timer I | T4 | Time to continue waiting for retransmitted ACKs after receiving an ACK | 20s | Same as the T4 timer setting. |
| Timer J | 64T1 | Time to continue waiting for retransmitted non-INVITE requests after sending a final response to a non-INVITE request | 72s | Same as the Timer B setting. |
| Timer K | T4 | Time to continue waiting for retransmitted response messages after receiving a response | 20s | Same as the T4 timer setting. |

#### 6.X.3.4	 IMS Transcoding Optimization

The terminal accessing high orbit satellite uses specific low bit rate codec of 1-2kbps. The existing ground network codec bit rate is greater than or equal to 4.25kbps. For the interconnection between the satellite network and the ground network, it is necessary to perform transcoding procedure within the IMS. The specific low bit rate codec bit flow that passes through IMS needs to be transcoded to the ground network codec to achieve successful media negotiation. After the call is setup, the IMS completes the satellite-ground transcoding of the media. In the situation of  satellite network internal call, satellite-ground-satellite multiple transcoding brings extra time delay. To avoid it, IMS is required to perform route detection of called side to determine whether the called UE is accessing satellite network and whether to perform transcoding.

6.X.4	Impacts to Services, Entities and Interfaces

UE:

-	Supports sending simplified SIP messages

-	Supports determining when to send simplified SIP messages based on the access type

P-CSCF:

-	Supports sending simplified SIP messages

-	Supports restoring simplified SIP messages to their original standard format

-	Supports determining when to send simplified SIP messages based on the UE's access type

IMS-AGW:

-	Supports the transcoding between satellite codec(ULBC) and Terrestrial codec(e.g. AMR, EVS, etc.)

* * * End of Changes * * *