SA WG2 Meeting #170	S2-2506647

25 - 29 August 2025, Goteborg, Sweden

Source:	China Mobile

Title:	KI#4: New Sol: Location Verification of UE via NB-IoT GEO Access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#4.

# 1	Discussion

Emergency calls and some regulatory services rely on the UE's location information, with different services requiring varying levels of location accuracy. In scenarios where the UE accesses via NB-IoT NTN, some traditional positioning methods (e.g., OTDOA, AoA) will become ineffective. Additionally, while the GNSS location information reported by the UE may be precise but untrusted.

Therefore, this solution proposes a method to obtain precise and trusted UE location information.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  |  |  | x |

* * * Next Change * * *

6.X	Solution #X: <Support of Location Verification of UE via NB-IoT GEO Access>

6.X.1	High level principles

This solution resolves Key Issue #4 Location service for IMS emergency call and regulatory services over NB-IoT NTN,   the following high level principles are included in this solution:

-	The MME can obtain and store coarse location information by SMC procedure.

-	The UE reports GNSS location information during emergency calls.

-	The reported GNSS information from the UE (if untrusted) is verified based on pre-stored coarse location.

6.X.2	Description

As specidied in TS 23.167. The Location information is needed for 2 main reasons in emergency services. The initial purpose of the location information is to enable the IMS network to determine which PSAP serves the area where the UE is currently located, so that the IMS network can route the emergency session to the correct PSAP. The second purpose is for the PSAP to get more accurate or updated location information for the terminal during or after the emergency session where required by local regulation.

In the case of satellite access for NB-IoT, the MME can only get the coarse location from UE as specified in TS 23.401[5]:

4.13.10	UE Coarse Location information for NB-IoT satellite access

As described in clause 4.13.4, in the case of NB-IoT, the MME may request the UE to report its Coarse Location Information for location verification purposes. In addition, as described in TS 36.300 [5] and TS 36.413 [36], the eNB may request, in a S1-AP Initial UE message, the MME to provide the Coarse Location Information to eNB. In that case, the MME provides, if available, the Coarse Location Information to the eNB as described in TS 36.413 [36].

As Specified in TS 36.413, the accuracy of coarse location is approximately 2 km, which can satisfy the purpose of PSAP selection. However, this does not meet certain regulatory service requirements (e.g., the accurate location information mandated by Public Safety Answering Points, PSAPs).

UEs equipped with GNSS modules can report GNSS information to the IMS network, but this location data is considered untrusted by the IMS.

To solve this, this solution proposes a method of verify GNSS information against coarse location , to obtain  accurate and trusted location information.

6.X.3	Procedures

6.X.3.1		Obtaining location information for emergency calls

Figure 6.x.3.1-1: Location retrieval for IMS emergency call over NB-IoT via GEO

- The UE initiate emergency IP-CAN bearer establishment over NB-IoT NTN access, the UE reports its coarse location information to the MME during the NAS Security Mode Command procedure.

- The UE initiates an emergency call session request over the emergency call PDN via NB-IoT NTN access. The INVITE message carries UE’s geographical location information (e.g. GNSS)

- If the location information provided in step 3 is trusted and sufficient to determine the correct PSAP, the procedure continues from step 8 onwards. If the location information is untrusted or insufficient. The IMS core sends the Location Verification request to the LRF, the request shall include information identifying the IP-CAN and the UE and shall include any geographical location information provided by the UE in step 2.

- The LRF sends the Location Verification request to the MME, the request shall include information identifying the IP-CAN and the UE and UE’s geographical location.

#### 5. Upon receiving the request, if  MME does not have the coarse location info, the MME retrieves the coarse location from UE aspecified in 23.401[5] and stored. The MME generates the requested QoS based on operator policy and the received request type. If the previously obtained coarse location meets the requested QoS, the procedure skips directly to step 6. If the coarse location does not meet the requested QoS requirements, the MME interacts with the E-SMLC, sending the coarse location and the UE's Geographical location to the E-SMLC for location information verification. This verifies whether the GNSS information is within the E-SMLC's scope and returns the verification result.

#### 6. The MME sends a Location Verification Response to the LRF. If the coarse location in step 5 met the requested QoS, the message carries the Coarse Location. If the location verification procedure in step 5 was performed, the message carries the GNSS information and verification result if verification was successful, or the Coarse Location and verification result if verification failed.

#### 7. The LRF sends a Location Verification Response to the IMS Core. The location information carried in this message is determined by operator policy and configuration, indicating whether to return the Coarse Location or the UE's Geographical location. (i.e., The operator policy configures whether the E-CSCF makes routing decisions based on the trusted but precision-unsatisfied Coarse Location or the untrusted but precision-satisfied GNSS.)

#### 8. The IMS core selects the PSAP based on the location information obtained from the LRF.

#### 9. The IMS Core forwards the INVITE message to the PSAP. The message carries the location information obtained from the LRF.

Editor’s note: The method of carrying geographic location information in the INVITE (e.g., via SDP, header, etc.) and whether to define size restrictions to the reported geographic location information IE (e.g. GNSS) is FFS.

6.1.3	Impacts to Services, Entities and Interfaces

UE:

-	Supports GNSS function.

MME:

-	Supports obtaining and storing the UE's coarse location via SMC.

-	Supports determining whether the coarse location meets the requested QoS.

LRF:

-	Supports initiating the location verification procedure upon receiving the "Retrieve Location-routing information" request from the IMS Core.

SMLC:

-	Supports verifying the geographical location based on the Coarse Location.

* * * End of change * * *