SA WG2 Meeting #170	S2-2506648

25 - 29 August 2025, Goteborg, Sweden

Source:	China Mobile

Title:	KI#3: New Sol: Location Retrieval for GEO IMS Emergency Calls

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4-ARC / Rel-20

Abstract of the contribution: The contribution proposes solutions for KI#3.

# 1	Discussion

For emergency calls over GEO NB-IoT NTN access, it is also necessary to consider meeting SA1’s IMS voice call setup time KPI over GEO satellites. The main problems come from ensuring emergency call QoS guarantees under GEO access and the accuracy of UE location.

The solution focuses on Emergency PDN Session Establishment and UE detectable/non-detectable IMS emergency session procedures during GEO NB-IoT NTN access.

# 2	Proposal

It is proposed to include the following changes in TR 23.700-19 V0.2.0.

* * * First Change * * *

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| x |  |  | x |  |

* * * Next Change * * *

6.X	Solution #X: <IMS Emergency Service for NB-IoT NTN Access>

6.X.1	High level principles

This solution resolves Key Issue #3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC,  the following  high level principles are included in this solution:

-	Remove the restriction on NB-IoT NTN access for emergency call services.

-	For emergency call services, the selection of P-GW and PSAP should based on the UE's geographic location(e.g. coarse location, GNSS) or pre-configured method.

-	The QoS parameters for the Emergency IMS voice call under GEO access need to be defined

- X.2	Description

This solution resolves Key Issue #3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC.

As specified in TS 23.167, emergency call services are categorized into UE Detectable Emergency Session and Non-UE Detectable Emergency Session based on UE capabilities.

For UE Detectable Emergency Session, an Emergency PDN Connection Establishment must be performed before initiating the emergency call setup procedure. However, as specified in TS 23.401 4.3.12.1, emergency bearer services are not supported when the UE operates in NB-IoT access. Therefore, the MME must support broadcasting Emergency Service indications, enabling an NB-IoT GEO access UE to initiate an Emergency PDN Connection Request upon detecting an emergency.

For Non-UE Detectable Emergency Session, the approach defined in TS 23.167 is reused. Upon receiving the call request from UE, the IMS network shall identify the call as an emergency call based on the dialed number and notify the UE via a response. Then UE establishes an emergency PDN connection and sends the emergency call request.

Regarding PGW selection for emergency PDN sessions, as specified in TS 23.401 4.3.12.4 and 4.3.8.1:When selecting a PGW for IMS emergency services, the standard PGW selection mechanism (applied to the Emergency APN) is used, or the MME directly selects the PGW from its Emergency Configuration Data.However, in GEO satellite access, due to the large beam coverage, a single cell may span multiple TAIs. This makes traditional TAI+ECGI-based PGW selection ineffective. Therefore, UE’s precise location information should be considered for PGW selection (and potentially PSAP selection as well).

As specified in clause 4.3.12.6 in TS 23.401, Dynamic PCC is used for UEs establishing emergency service and the PCRF provides the PDN GW with the QoS parameter. According to SA1’s IMS Voice Call KPIs for over NB-IoT NTN GEO access,  corresponding QoS parameters for the IMS emergency voice dedicated bearer need to be defined.

6.X.3	Procedures

- 6.x.3.2 UE detectable IMS emergency session

Figure 6.x.3.2-1: UE Detectable IMS Emergency session

#### 0. For UEs accessing via NB-IoT(GEO), the MME shall support indicating emergency bearer service capability through the Emergency Service Support indicator, and an NB-IoT-NTN cell may broadcast emergency service support in system information (when accessing via NB-IoT(GEO), the UE detects this indication in Access Stratum system broadcasts).

- UE initiates an emergency call session request over the emergency call PDN via NB-IoT NTN access. The INVITE message carries the emergency service URN in the Request-URI header field and includes the UE's geographical location information (e.g., Coarse location, GNSS).

2. According to the emergency service URN and UE’s geographical location information,  P-CSCF triggers interaction with EPC to verify the UE’s geographical location information.

3. The P-CSCF identifies the call as an emergency call based on the called number and sends an AAR to the PCRF to request the establishment of a dedicated bearer for the emergency call. The message carries the Service-URN AVP indicating an emergency call and includes the UE's geographical location information (e.g., GNSS).

4. The PCRF performs emergency call service validation and generates QoS rules for GEO IMS emergency calls based on the user access type (RAT Type="NB-IoT(GEO)") and the emergency indication(These QoS rules include an ARP priority higher than that of GEO IMS normal calls). PCRF sends the RAR to P-GW, includes the GEO IMS emergency calls QoS rules.

5. The P-GW returns an RAA to the PCRF after the establishment of a dedicated bearer for GEO IMS emergency voice calls.

6. The PCRF returns  AAA to the P-CSCF, includes the user location information (UE geographical location) and verification result provided by the PCRF.

7. The P-CSCF forwards the INVITE to the E-CSCF, including the verified UE geographical location information.

8. The E-CSCF queries local routing tables using the UE’s geographical location information to determine the address of the emergency call center and routes the emergency call to the appropriate PSAP.

9–11. Subsequent emergency call setup procedures continue, resulting in the successful establishment of the emergency call between the UE and the PSAP.

Editor’s note:  The method of UE location verification can refer to the solutions to KI#4, which is FFS.

#### 6.x.3.3	non-UE detectable IMS emergency session

1. The UE initiates a normal call session request message via NB-IoT NTN access.

2. The P-CSCF in the IMS identifies the session as an emergency call based on the UE's called number and replies to the UE with a response (e.g. SIP 380) indicating an emergency call.

3. Upon receiving the 380 response indicating an emergency call, UE resend INVITE request to initiates an emergency call session, the subsequent procedures are same to UE detectable IMS emergency session .

### 6.1.3	Impacts to Services, Entities and Interfaces

UE:

-	Supports GNSS function.

-	Supports reporting its precise geographical location information (e.g. coarse loaction, GNSS) during IMS emergency calls over GEO satellite access.

P-CSCF:

-	Supports to verifying the UE's reported geographical location information with EPC.

MME：

-	Supports broadcasting Emergency Service (under IoT-NTN access).

PCRF:

-	Supports the provision of PCC Rules for Emergency Calls When UE Access via NB-IoT (GEO)

* * * End of change * * *