SA WG2 Meeting #170	S2-2506677

25 – 29 Aug, 2025, Goteborg , SE

Source:	NEC

Title:	[WT#3, AI in 6G] Key Issue on AI in 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Work Task (WT) and Key Issues (KIs) on AI in 6G.

# 1. Justifications

Architectural Motivation:

As recognized in TR 22.870, Artificial Intelligence (AI) is envisioned as a foundational enabler for 6G systems, addressing not only intelligent application requirements but also broader environmental, social, and economic challenges. Within this vision, two complementary directions are considered for AI integration in 6G:

- AI for 6G System: Leveraging AI capabilities, such as AI Agents, to enhance system and device functionalities in supporting 3GPP services.

- 6G System for AI: Building system competencies that facilitate a range of AI-driven applications, through exposure and enablement of 6G capabilities.

This proposal focuses on the first concept, i.e., "AI for 6G System." In this context, an AI Agent is defined as an autonomous, intelligent entity capable of interacting with its environment, acquiring contextual information, performing reasoning, learning, making decisions, and executing tasks (either individually or collaboratively with other AI Agents) to fulfill specified objectives. AI Agents may be realized natively within the 6G architecture, serving roles such as interpreting user intent, orchestrating network services, optimizing resource allocation, and ensuring quality of service. They may also function as vertical applications, leveraging network capabilities for tasks including perception, analysis, and complex interaction.

Note: The "6G System for AI" direction is addressed in WT#5 and WT#6 regarding support for AI-driven service enablement and exposure.

Gap Analysis:

In current 5G architectures (Releases 15–20), AI functionalities are primarily centralized, instantiated through entities such as NWDAF (defined in TS 23.288) and MDAF (defined in TS28.104)to provide network analytics, with knowledge exposure mediated via components like the NEF (defined in TS.23.501). While these approaches enable analytics-driven optimization and external exposure, they lack native support for distributed intelligent entities—specifically, AI Agents capable of autonomous decision-making, contextual adaptation, and intentional collaboration across domains. These limitations restrict the system's abilities in real-time user intent interpretation, cross-domain orchestration, and dynamic adaptation.

Technical Impact:

Addressing these gaps through the proposed Work Task/Key Issue would have the following impacts:

- System Architecture: Introduction of new reference points, logical/functional models, and possibly new network functions to support AI Agent lifecycle, interaction, and trust management.

- Function Interactions: Definition of mechanisms for AI Agent discovery, communication, and orchestration within/between network domains and external applications.

The scope aligns with WT#3, focusing on augmenting system capabilities for environmental interaction, contextual data acquisition, autonomous reasoning, learning, decision-making, and goal-directed execution.

**** First Change ****

# Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

**** Second Change ****

# 5.X. Key Issue #X: Key Issue on AI in 6G

  - Study on weather and how to support AI Agent as an automated intelligent entity capable of e.g interacting with its environment, acquiring contextual information, reasoning, self-learning, decision-making, executing tasks (autonomously or in collaboration with other Al Agents) to achieve a specific goal.

**** End of Changes ****