SA WG2 Meeting #170	S2-2506680

25 – 29 Aug, 2025, Goteborg , SE

Source:	NEC, SoftBank

Title:	[WT#6, Computing in 6G] Key Issue on Computing in 6G.

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Work Task (WT) and Key Issues (KIs) on computing in 6G.

# 1. Justifications

Architectural Motivation:

3GPP Technical Report TR 22.870 describes multiple use cases for 6G networks that require the native integration of compute services. In this context, compute resources refer to the underlying physical or virtualized capabilities—such as CPU, memory, and storage—distributed throughout the network, including at the edge, in the core, or within the cloud. Compute services are the managed, consumable capabilities built on top of these resources, such as workload execution or application hosting, and are made available to consumers via standardized network APIs.

Advanced use cases (e.g., XR split rendering) for compute service are illustrative: the network primarily delivers traffic to rendering servers instead of directly exposing and managing the rendering compute capacity as an intrinsic network function.

The document highlights significant architectural challenges associated with the orchestration and management of distributed compute resources across various network domains. Unlike previous generations where computing was not an intrinsic part of the system architecture, 6G elevates computing to a foundational network capability, positioning the network as a distributed computing platform rather than only as a communications infrastructure. In this evolution, compute resources—spanning UEs, device-edge (e.g., Customer Premises Equipment or CPE), network edge, central cloud, and non-terrestrial network (NTN) elements—are abstracted and managed in a unified manner to support the delivery of advanced compute services directly through the network. Such pervasive integration of computation is expected to unlock new business models, enhance user experiences, and enable automation and intelligence at unprecedented scale.

TR 22.870 introduces the Service Hosting Environment (SHE), which provides standardized, managed compute services by orchestrating underlying compute resources distributed across the network. SHE enables the exposure and consumption of these services through interoperable interfaces.”

Gap Analysis:

In 5G architectures (Releases 15–18), foundational support for network-integrated computing emerged, exemplified by initiatives like Edge Computing (EC). EC moved the network beyond being a simple pipe, enabling low-latency and optimized connectivity to applications placed at the edge. 5G frameworks concentrated on traffic steering, connectivity, and providing external application functions (AFs) the means to operate efficiently near the network edge.

However, several limitations persist:

- Computing as an external service: In 5G, computing services remain external to the core network—applications are deployed on top of or adjacent to the network, with computation orchestrated at the application layer rather than by the network itself.

- Limited Exposure and Orchestration: 5G defines mechanisms for basic exposure and traffic management for Edge Computing, but lacks comprehensive frameworks for coordinated, network-managed compute orchestration across UE, edge, core, and cloud.

- Fragmentation: The lack of a unified computation framework inhibits cross-layer automation, dynamic resource allocation, and seamless end-to-end service creation, especially when integrating novel locations such as NTNs or deeply embedded device-edge compute.

Technical Impact:

This Work Task will have significant impact in the following areas:

- Overall System Architecture:
The study will inform how the 6G architecture evolves to treat computing as a core, managed network resource. This includes establishing new reference points, extending the functional model to support integrated compute across UE, edge, core, and cloud, and defining mechanisms for resource exposure and lifecycle management within the SHE paradigm.

- Interaction Between Network Functions/Services:
The work will identify necessary workflows, protocols, and interfaces that enable different network entities (e.g., core functions, edge nodes, UEs) to interact and coordinate on compute services. This involves control and data exchange for the dynamic allocation, scaling, and migration of computing workloads,

**** First Change ****

# Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g. coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE: Application layer mechanism and exposure framework may require coordination with SA6.

**** Second Change ****

# 5.X. Key Issue #X: Key Issue on Computing in 6G

- Overall System Architecture (e.g., reference points, functional models):

  - Study on functional models (e.g., compute functions that abstract and manage compute resources in a unified manner, Service Hosting Environment) to support the exposure and delivery of network-integrated compute services

  - Study on required reference points and interfaces for integrating computing functions with other network functions, ensuring interoperability, security, and scalability.

- Interaction between Network Functions/Services:

  - Study on mechanisms for exposing consumable compute services (e.g., workload execution, application hosting, compute resource) via network APIs, leveraging the capabilities of computing functions and underlying compute resources.

  - Study on procedures for discovery and selection of available compute functions and compute services over standardized interfaces.

  - Study policies and mechanisms for controlling compute resource and service access, allocation, and usage

**** End of Changes ****