3GPP TSG-WG SA2 Meeting #170	S2-2506681

Goteborg, Sweden, 25th August – 29th August, 2025

Source:	China Telecom

Title:	23.700-30: New Solution of DO-A procedure for DO-A Capable AIoT Devices

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: This is a new solution proposal related to WT#2 in the R20 Ambient IoT SID SP-250834.

1. Introduction/Discussion

Based on the SID objective (SP-250834), the following work task shall be studied:

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

This pCR provides a new solution for corresponding to this WT2 about an DO-A Capable AIoT Device originated DO-A procedure to send data to the AIoTF or AF(via AIoTF).

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new texts)* * * *

6 Solutions

6.X	DO-A procedure for DO-A Capable AIoT Devices

6.X.1	Description

This solution is for Key Issue #X, corresponding to WT2. This solution is to support for an DO-A Capable AIoT Device autonomously originated DO-A procedure to send data to the AIOTF, and support for routing the received data by AIOTF to AF.

The main points are as following:

- The DO-A Capable AIoT Devices support autonomously originat DO-A procedure to send data to the AIOTF.

- The DO-A Capable AIoT Device has pre-configurated network resource/assistance information(e.g. frequence, Qos, Resource Expiration Date,etc) for autonomously originating DO-A procedure; or support to receive the network resource information/assistance information (e.g. frequence, Qos, Resource Expiration Date,etc) from the network for autonomously originating DO-A procedure, using by AIoT write commands or new AIoT service, or network system information etc.

- When the DO-A Capable AIoT Device will autonomously originats DO-A procedure to send data, is based on the DO-A policy information. The DO-A policy information may be received from network or AF via the network, which send by AF using AIoT write commands or some new AIoT service. Or the DO-A policy information can be pre-configurated in the AIoT device or UICC if have.

- The DO-A policy information may includes:

  - Target information: identify where and which AF the DOA data sends/routes to by AIoT device, for example AF identify, etc；

  - Service policy: describe when and how the AIot device autonomously originated to send DOA datas, such as: 9:00 am every day, when the timer expired, the upper layer initials, etc.

- The DO-A Capable AIoT Device supports to autonomously originat procedure to send data to the AIOTF, and support for routing the received data by AIOTF. The procedure may includes: security data, target information and the data information. The secruity data is used for the network authentication AIoT device; and the target information is used to identify where and which AF the DOA data sends/routes to, for example AF identify, etc; the data information is the real data which the AIoT device wants to send for AF.

6.X.2	Procedures

The following figure presents a DO-A procedure for DO-A Capable AIoT Devices for Topology 1 and Topology 2.

Figure 6.x.2-1: DO-A procedure for DO-A Capable AIoT Devices

0-a. DO-A Capable AIoT Devices may be pre-configured with network resource/assistance information(e.g. frequence, Qos, Resource Expiration Date,etc) for autonomously originating DO-A procedure.

0-b. DO-A Capable AIoT Devices may be pre-configured with DO-A policy information for autonomously originating DO-A procedure.

NOTE 1:	How information is configured into AIoT device is out of scope of 3GPP.

- AF sends like DO-A service request to NEF via AIoT write command or new AIoT Service, it may includes following parameters: AIoT devices ID, AF ID, DO-A policy information, and so on.

- DO-A policy information may includes:

    - Target information: identify where and which AF the DOA data sends/routes to by AIoT device, for example AF identify, etc；

    - Service policy: describe when and how the AIot device autonomously originated to send DOA datas, such as: 9:00 am every day, when the timer expired, the upper layer initials, etc.

- AIoTF/NEF receives the DO-A service request from AF, and will check the authorization of AF and the authorization and DO-A Capable of AIoT devices. If it is allowed, the network function(e.g. AIoTF or RAN/UE reader) will send the network resource information/assistance information(e.g. frequence, Qos, Resource Expiration Date,etc) for DO-A Capable of AIoT devices autonomously originating DO-A procedure. DO-A Capable of AIoT devices receives these network resource information/assistance information.

- AIoTF/NEF sends the DO-A policy information from AF to DO-A Capable of AIoT devices to DO-A Capable of AIoT devices. DO-A Capable of AIoT devices receives these DO-A policy information.

- DO-A Capable of AIoT devices store these receiving network resource and/or DO-A policy information for DO-A procedure from network.

- When DO-A policy information is applied, for example: the timers from the DO-A policy information expired, the DO-A Capable of AIoT device performs the DO-A procedure.

- The DO-A Capable of AIoT device sends DO-A data to network/AF via RAN/UE Reader, using the storing or pre-configured network resource.

- RAN/UE Reader routes the DO-A Data from AIoT device to AF via AIoTF.

6.X.3	Impacts on services, entities and interfaces

NEF:

- Supports AF to provide DO-A policy informaiton;

AIoTF:

- Supports to receive DO-A data from DO-A capcable AIoT Device and route for AF;

- Supports to provide network resource/assistance information(e.g. frequence, Resource Expiration Date, etc) for AIoT Device DO-A procedure;

RAN Reader or UE Reader:

- Supports to receive DO-A data from DO-A capcable AIoT Device and route for AIoTF;

- Supports to provide network resource/assistance information(e.g. frequence, Resource Expiration Date,etc) for AIoT Device DO-A procedure；

AIoT Device：

- Pre-configured or stores network resource/assistance information(e.g. frequence, Resource Expiration Date, etc) and/or DO-A policy information;

- Performing DO-A procedure once it is needed to send DO-A data.

# * * * * End of changes * * * *