SA WG2 Meeting #170	S2-2506687

25 – 29 Aug, 2025, Goteborg , SE

Source:	Samsung

Title:	Solution for KI#2 to Support Asset Tracking

Document for:	Agreement

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC/Rel-20

Abstract of the contribution: This contribution proposes a solution to support asset tracking regarding Key Issues for WT#2.

1. Discussion

The existing description of WT#2 as in the agreed SID titled as “Study on Architecture support of Ambient power-enabled Internet of Things - Phase 2” [SP-250834] is described as follows:

WT#2: Study the support of DO-A Capable AIoT Devices, including:

-	Support of the AIoT Device informing the network of its presence autonomously (e.g., an AIoT Device initiated registration-like procedure).

-	Support for an autonomous AIoT Device originated procedure to send data to the AIOTF, and support for routing the received data by AIOTF.

-	Naiotf and Nnef interface enhancements to provide the data received from an AIoT Device to the AF.

NOTE 4:	topology 2 aspect of WT#2 has dependency on WT#1.

Ambient IoT Service Use cases in TS 22.269 includes Asset Tracking. With asset tracking, the main purpose is to determine the location of goods. Ambient IoT devices attached to these goods report an identifier associated with the good. This can then be combined with location information.

It is assumed that KI#2 is key issue for WT#2.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.700-30.

* * * Start of Change (all new text) * * * *

6.X	Solution #X: Support of Asset Tracking

6.X.1	Description

This solution addresses KI#2. The solution includes Service Reqeust from AF to 5GS, Parameter Configuration for the Service, Data Transfer for the Service.

The solution is based on the following assumptions:

-	The Asset Tracking Service utilizes Periodic DO-A Asset Tracking Service from the Device including AF, AIOTF, and AIoT Device. It uses Inventory Request type with a Period.

-	The Asset Tracking Service utilizes Periodic DO-A Data Transfer (Device, AIoT Reader, AIOTF)

-	Asset Tracking Service Paramters (e.g., Tracking Period, StartTime, Event-Triggered) can be pre-configured on AIoT Devices or configured by AF.

-	AIoT Reader maintains (Device ID and Correlation ID) context for every periodic behavior or event-triggered behavor.

6.X.2	Procedures

The procedure is shown in Figure 6.X.2-1. It includes Service Reqeust from AF to 5GS, Parameter Configuration for the Service, Data Transfer for Asset Tracking Service.

Figure 6.X.2-1: Asset Tracking Procedure

1.	AF sends AIoT Inventory Request to NEF. It includes Device(s) Info, Loc(ext.) which is 3GPP-external location information, AF Transaction ID, Period, and optimially Tracking Parameters which include [Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off].

2.	NEF sends AIoT Inventory Request to AIOTF. It includes Device(s) Info, Loc(Int.) which is internal location info like RAN ID or Cell ID mapped to Loc(ext), Transaction ID which is mapped to AF Transaction ID, Period, and optionally Tracking Parameters which include [Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off].

3.	AIOTF generates Inventory Request based on Device(s) Information included in step 1 in cooperation with ADM.

4.	AIOTF sends AIoT Inventory Response to NEF. It includes Status (Success/Failure) and Transaction ID.

5.	NEF sends AIoT Inventory Response to AF. It includes Status (Success/Failure) and AF Transaction ID.

6.	Steps 7 to 10 of the Procedure for Inventory in clause 6.2.2 of TS 23.369.

7.	AIOTF retrieves Device Profile Data Including DO-A Capability. If the device does not support DO-A Capability, step  15 to step 19 or step 20 to setp 24 are skipped.

8a~12a, 8b~12b, 8c~12c.	Parameter set with Write Command can be done before steps 8~12.

8.	AIOTF sends AIoT Inventory Request to AIoT Reader. This includes Device ID (temporary), (Reader ID), Correlation ID, Period, and Tracking Parameters ([Tracking Period Offset, period,] [StartTime Offset, Now/time/Off] [Event-Triggered Offset, On/Off]). AIoT Reader stores Correlation ID with AIoT Device ID as context data.

9.	AIoT Reader sends AIoT Inventory Request to AIoT Device. This includes Device ID (temporary), (Reader ID), Correlation ID, Period, and Tracking Parameters ([Tracking Period Offset, period], [StartTime Offset, Now/time/Off], [Event-Triggered Offset, On/Off]).

10.	AIoT Device set parameters, including Tracking Period (a specific Period), StartTime (Now, a specific time, or Off) and Event-Triggered (On or Off). If StartTime is set to Now, the device sends Inventory Report immediately and every Period. If StartTime is set to a specific time, then the device sends Inventory Report at that time and every Period thereafter. If StartTime is set to Off, the devce sends Inventory Report once, then stops periodic reporting. If Event-Triggered is set to On, the device sends Inventory Report when it detects a change in Reader ID. If Event-Triggered is set to Off, the device stops sending  Inventory Report when the device detects a change in Reader ID. Depending on the size of the report, the radio resource allocation procedure can be different.

11.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary), Reader ID.

12.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

13.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

14.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

15.	Every Period, the device sends Inventory Report. Depending on the size of the report, the radio resource allocation procedure may vary.

16.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary) and Reader ID.

17.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

18.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

19.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

20.	On Event (When the device detects the Reader ID has changed), the device sends Inventory Report. Depending on the size of the report, the radio resource allocation procedure may vary.

21.	AIoT Device sends AIoT Inventory Response to AIoT Reader. This includes Device ID (temporary) and Reader ID.

22.	AIoT Reader sends AIoT Inventory Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

23.	AIOTF sends AIoT Inventory Notify to NEF. This includes Device ID (permanent), Reader ID, and Transaction ID. AIOTF retrieves Transaction ID with Correlation ID from the stored context.

24.	NEF sends AIoT Inventory Notify to AF. This includes Device ID (permanent), Reader ID, and AF Transaction ID. NEF retrieves AF Transaction ID with Transaction ID from the stored context.

The optional flows of 8a~12a, 8b~12b, 8c~12c are shown in Figure 6.X.2-2. Parameter set with Write Command can be used before steps 8~12.

Figure 6.X.2-2: Optional Steps for Asset Tracking

8a.	AIOTF sends AIoT Command Request to AIoT Reader. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, Tracking Period Offset, and Period. AIoT Reader stores Correlation ID with AIoT Device ID as context data.

9a.	AIoT Reader sends AIoT Command Request to AIoT Device. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, Tracking Period Offset, and Period.

10a.	AIoT Device sets Tracking Period to Period (a specific period).

11a.	AIoT Device sends AIoT Command Response to AIoT Reader. This includes Device ID (temporary) and Sensing Result.

12a.	AIoT Reader sends AIoT Command Response to AIOTF. This includes Device ID (temporary), Sensing Result, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

8b.	AIOTF sends AIoT Command Request to AIoT Reader. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, StartTime Offset, and Now/time/Off. AIoT Reader stores Correlation ID with AIoT Device ID as context data.

9b.	AIoT Reader sends AIoT Command Request to AIoT Device. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, StartTime Offset, and Now/time/Off.

10b.	AIoT Device sets StartTime to Now/time/Off. If StartTime is set to Now, the device sends Inventory Report immediately and every Period. If StartTime is set to a specific time, the device sends Inventory Report at that time and every Period thereafter. If StartTime is set to Off, the device sends Inventory Report once, and turns off the periodic reporting.

11b.	AIoT Device sends AIoT Command Response to AIoT Reader. This includes Device ID (temporary) and Reader ID.

12b.	AIoT Reader sends AIoT Command Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

8c.	AIOTF sends AIoT Command Request to AIoT Reader. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, Event-Triggered Offset, and On/Off. AIoT Reader stores Correlation ID with AIoT Device ID as context data.

9c.	AIoT Reader sends AIoT Command Request to AIoT Device. This includes Device ID (temporary), (Reader ID), Write, Correlation ID, Event-Triggered Offset, and On/Off.

10c.	AIoT Device sets Event-Triggered to On or Off. If Event-Triggered is set to On, the device sends Inventory Report when it detects a change in Reader ID. If Event-Triggered is set to Off, the device stops sending  Inventory Report when the device detects a change in Reader ID.

11c.	AIoT Device sends AIoT Command Response to AIoT Reader. This includes Device ID (temporary) and Reader ID

12c.	AIoT Reader sends AIoT Command Response to AIOTF. This includes Device ID (temporary), Reader ID, and Correlation ID. AIoT Reader retrieves Correlation ID with AIoT Device ID from the stored context.

6.X.3	Impacts on services, entities and interfaces

Impacts on existing entities:

AIOTF:

-	Receives Inventory Request with a Period from AF through the NEF.

-	Performs Inventory procedure and then then sends Inventory request with Period and Tracking Parameters.

-	Performs Inventory procedure and then then sends Write Command request with Tracking Parameters.

-	Retrieves Device Profile Data Including DO-A Capability. If the device does not support DO-A Capability, AIOTF skips DO-A related steps.

-	Receives AIoT Inventory Report from Device(s) through AIoT Reader(s) and deliveres it to the the AF

-	Sends AIoT Inventory Notify with Reaer ID as 3GPP-Internal Location information, which is converted into 3GPP-External Location information at NEF, to AF.

ADM:

-	Supports the AIoT Device profile including DO-A Capability.

AIoT Reader (RAN Reader / UE Reader):

-	Supports the AIoT TempID handling including Device ID and Corrleation ID mapping, and maintains the mapping context.

-	Supports DO-A procedure for AIoT Inventory Report from AIoT Device.

AIoT device:

-	Supports DO-A procedure for AIoT Inventory Report from AIoT Device.

-	Sensing AIoT Inventory Report to AIoT Reader periodically via the DO-A procedure.

-	Sets Tracking parameters. It sets Sensing Period to Period (a specific period). It sets StartTime to Now, a specific time, or Off. If StartTime is set to Now, the device sends Inventory Report immediately and every Period. If StartTime is set to a specific time, the device sends Inventory Report at that time and every Period thereafter. If StartTime is to Off, the device sends Inventory Report once, and turns off the periodic reporting. It sets Event-Triggered to On or Off. If Event-Triggered is set to On, the device sends Inventory Report when it detects a change in Reader ID. If Event-Triggered is set to Off, the device stops sending  Inventory Report when the device detects a change in Reader ID.

-	Performs different the radio resource allocation procedure depending on the size of the report.

* * * End of Change * * * *