3GPP TSG-WG SA2 Meeting #170	S2-2506694

Goteborg, SE, 25th – 29th Aug, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Interim conclusion proposal

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: To provide a basis for future discussion, interim conclusions have been incorporated into Clause 7.

1. Introduction/Discussion

SA2 initiated the study for sensing in Rel-20 in TS 23.700-14, focuses on the 5GA sensing framework, particularly on architectural design aspects such as service authorization and revocation, discovery and selection of sensing entities/functions, sensing data collection and result exposure, and configuration parameter provisioning.

On the other hand, Rel-20 specification narrows its scope to gNB-based mono-static sensing for UAV target use cases, as outlined in RP-251861 and SP-250809 during TSG #108 as follows:

In SP-250809: The study item aims at investigating the function enhancement, end to end service operations and procedures to support Integrated Sensing and Communication with the considerations of a sub-set of use cases (e.g. commercial, automotive, public safety and emergency services) based on the SA1 requirements. In this study, the scope is gNB-based sensing, aligned with Rel-20 5G-A RAN ISAC study scope.

In RP-251861: Evaluate the performance of gNB-based mono-static sensing (i.e., single TRP with co-located sensing transmitter and receiver) for UAV use case [RAN1] - Identify and study metrics, measurements, and relevant measurement quantization for UAV use case - As baseline, existing DL NR waveform and DL NR reference signals are to be used for evaluations. - For other waveform and reference signals, companies are to share relevant information - No UE impacts - Deployment scenario and assumptions for channel model calibration for UAV sensing targets in the Rel-19 ISAC channel model SI [FS_Sensing_NR] are used as starting point for evaluation assumptions. - FR1 frequency range is prioritized. Study the procedures, signaling between RAN and CN to support ISAC [RAN3] Study network architecture for gNB-based mono-static sensing for UAV sensing target use cases [RAN3] - Applicability to gNB bistatic sensing may be considered as part of this network architecture without additional architecture impacts. - No inter-gNB coordination will be studied. - Coordination with SA2 as necessary.

Currently, six Key Issues have been agreed upon and remain relatively stable. As outlined in the TR, the gNB-based monostatic sensing mode represents a relatively straightforward scenario for two key reasons:

-	No UE involvement: The procedure eliminates the need for user consent, UE selection, or interaction, as it does not involve User Equipment (UE).

-	Stationary focus: Mobility considerations are excluded in this initial release, with the scope limited to stationary scenarios for gNB-based monostatic sensing.

Additionally, numerous commonalities exist among the proposed solutions. Therefore, it is timely to reach an interim conclusion at this stage.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-14.

# * * * * First change * * * *

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

The reference architecture in figure 7.1.1-1 is proposed to be adopted for KI#1.

Editor's Note:	The interface between RAN and Sensing Function requires further study.

The Sensing Function is composed by Sensing Control Function which is involved to support Sensing Service and responsible for providing the related configuration information, and Sensing Processing Function, which is responsible for receiving the 3GPP Sensing Data and for performing the data processing to provide the Sensing Result.

Figure 7.1.1-1: Reference Architecture

# * * * * Second change * * * *

7.1.2	Agreed Principles for KI#2

The following principles are agreed for KI#2:

-	The network should authorise/revoke the AF as Sensing service consumer as defined in TS 23.501 [3].

-	The network should authorise/revoke Target Sensing Service Area based on the Sensing Service request authorisation information.

-	The authorisation/revocation is irrelevant to gNB as Sensing Entities.

# * * * * Third change * * * *

7.1.3	Agreed Principles for KI#3

The following principles are agreed for KI#3:

-	The network should select the Sensing Control Function based on the Target Sensing Service Area, and the supported Sensing Service Area of the Sensing Control Function.

-	The Sensing Control Function should select the gNB(s) based on the Target Sensing Service Area, and the supported Sensing Service Area of the gNB(s).

# * * * * Fourth change * * * *

7.1.4	Agreed Principles for KI#4

The following principles are agreed for KI#4:

-	The gNB sends the collected 3GPP sensing data to the Sensing Processing Function for sensing result generation.

NOTE:	The detailed information provided by the gNB will be determined by RAN WG3 and specified during the normative phase.

# * * * * Fifth change * * * *

7.1.5	Agreed Principles for KI#5

The following principles are agreed for KI#5:

-	The Sensing Control Function provides the Sensing results containing the object(s) characteristics and the associating geographical coordinates within the Target Sensing Service Area to the Sensing Service Consumer.

-	Sensing results provided by the Sensing Control Function can be one time or periodic.

# * * * * Sixth change * * * *

7.1.6	Agreed Principles for KI#6

The following principles are agreed for KI#6:

-	The configuration parameters are provided by the Sensing Control Function to the selected gNB(s) for generating 3GPP Sensing Data.

NOTE:	The detailed information provided by the Sensing Control Function to the gNB(s) will be coordinated by RAN WG3 and specified during the normative phase.

# * * * * Seventh change * * * *

8	Conclusions

Editor's note:			This clause will capture conclusions for the study.

Where there is consensus, interim agreements (e.g. solution principles descriptions) should be documented in the TR as soon as possible during a study.

These can be documented in the TR as "7.1.Y Agreed Principles for KI#Y" in the "Interim Agreements" clause. If the interim agreement has impacts on another clause in the TR and if there is consensus, that TR clause can be updated.

By consensus interim agreements can become part of the final conclusions of the study.

The Overall Evaluation clause previously used in TR skeletons should not be used.

There should be a Topics for further consideration clause per Key Issue. It is recommended that this is used e.g. to capture common issues that need to be resolved for multiple solutions.

The procedure described in figure 8.1-1 is adopted as interim agreed conclusion.

Editor's note: the detailed description of each step will be further determined as the decision process will progress.

Figure 8.1-1: Procedure of the sensing general framework

- Sensing entities may register its capability to SF

Editor's note: This step is proposed in solution #4 and #8. The solution #6 proposes to register to the GW.

1.a	The Sensing Service Consumer (i.e. AF) requests the NEF to obtain the Sensing result from the network via Sensing Service Request.

Editor's note: The Sensing GW is proposed in solutions #1 and #6, while other solutions propose the NEF.

1.b	The Sensing Service Consumer (i.e. AF) internal may request to obtain the Sensing result from the Sensing Function via Sensing Service Request.

Editor's note: The Sensing Service consumer is an AF in solution #4, #5, #6, #7,#8, #9,#10 , #12 and #13. In solution #2 is the UE which perform directly step 1b. In solution #1 the sensing consumer is generic.

NOTE 1:	Detailed information of sensing service requirement, sensing service type and corresponding information is described based on agreed principle and solution to Key Issue #2 and/or Key Issue #5.

2.	Upon receiving request from AF, the NEF authorizes the AF for usage of the Sensing Service.

Editor's note: This step is proposed by solutions#5, #6, #8, #9. the Sol#4 proposes the interaction with UDR. The sol #6, #1 proposes the usage of Sensing GW instead of the NEF, while the NEF is proposed by solutions #4, #5, #7, #8, #9, #10, #12

NOTE 2:	Detailed description of authorisation performs by NEF based on agreed principle and solution to Key Issue #2.

3.	The NEF may discover and select the candidate Sensing Function(s).

Editor's note: This step is proposed by solutions #4, #5, #13, the other solution does not mentioned it.

NOTE 3:	Details regarding NEF selection of Sensing Function is addressed by solutions to Key Issue #3.

4.	The NEF sends the request to the Sensing Function, where the request message contains information received in step 1a.

5.	The Sensing Function may perform further authorisation for the sensing request based on the parameters provided by the AF.

Editor's note: the step 5 is proposed in solutions #5, #10, #13, other solution did not mention it.

NOTE 3:	Details regarding Sensing Function authorising the AF's request is addressed by solutions to Key Issue #2.

6.	If the Sensing Function performs authorisation, it may indicate whether the request has been authorised or rejected via Nsf_SensingServiceExposure_Subscribe Response.

Editor's note: : This step 6 is proposed by solutions  #5. #8, #9, #12 , the other solution does not mentioned it.

7.	The NEF notifies the AF whether the subscription has been authorised or rejected via Sensing Service Response.

NOTE 4:	the Sensing Function or NEF may respond in step 6 and 7 indicating the result of sensing service subscription (e.g. for the failure case).

8.	The Sensing Function performs the discovery and selection for the Sensing Entity, based on e.g. the sensing service requirement, and sensing capability supported by the Sensing Entity.

Editor's note: This step 8 is proposed by solutions #1, #4, #6, #7#10, Solution #5 proposes that SF select the AMF and the AMF select the Sensing Entity.….

NOTE 6:	The details regarding the discovery and selection of the Sensing Entity are addressed by the solution to Key Issue #3.

9.	The Sensing Function determines the information for the subsequent Sensing measurement and sends the sensing service request message including the configuration parameters to the Sensing Entity.

Editor's note: the step 9 is proposed in solutions #1, #6, #4, #5, #7, #10, Whether the exchange is performed via N2 or a new dedicate interface is FFS.

NOTE 7:	Details regarding Sensing Function sending the configuration parameters, including whether those configuration parameters are directly sent to the Sensing Entities or not, is addressed by solution to Key Issue 1 and Key Issue 6.

10.	Upon receiving the sensing service request message from the Sensing Function, the Sensing Entity may trigger the establishment of the path for 3GPP sensing data reporting.

Editor's note: whether the 10 is needed to set up the connection to SF is FFS.

11.	The Sensing Entity performs the Sensing measurement and obtains 3GPP sensing data. Add something related to RAN to trigger LS to RAN regarding the information???

Editor's note: the step 11 is proposed in solutions #1,#4, #5, #6, #7, #10 (basically all solutions)

12.	The 3GPP sensing data is transmitted to the Sensing Function.

Editor's note: the step 12 is proposed in solutions #1, #4, #5, #6, #7, #10 (basically all solutions).

NOTE 8:	Details regarding the 3GPP sensing data and the reporting is addressed by the solution to Key Issue 4.

13.	After receiving the 3GPP sensing data, the Sensing Function determines the Sensing result.

14.	The Sensing Function sends the Sensing result to the NEF via Nsf_SensingServiceExposure_Notify.

15.	The NEF provides the Sensing result to the AF via Sensing Service Notify.

NOTE 9:	The steps 11-15 may be performed one-time or a periodically or based on event trigger depending on the sensing service request from AF.

NOTE 10:	Details regarding exposing the Sensing Result are addressed by the solution to Key Issue #5.

NOTE 11:	In addition to support the Sensing Result exposure via control plane, whether the Sensing Result can also be exposed via data connection for sensing operation is addressed by the solution to Key Issue #5.

NOTE 12:	Details regarding the procedure between the Sensing Function and the Sensing Entity from step 9 to 12 will be addressed by the associated solutions.

# * * * * End of changes * * * *