SA WG2 Meeting #170	S2-2506700

25 – 29 August, 2025, Goteborg, SE

Title:	KIs #4, #5: New Solution on Sensing Data Processing

Source:	Apple

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution on Processing of Sensing Data for FS_Sensing_ARC.

# 1.		Introduction

This contribution proposes a new solution on Processing of Sensing Data for FS_Sensing_ARC (KIs #4, #5).

# 2.	Text Proposal

It is proposed to agree the following solution to be part of TR 23.700-14.

************ 1st change *************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #x |  |  |  | X | X |  |

************ 2nd change (all new text) *************

## 6.x	Solution #X: Sensing Data Processing

This solution addresses KIs #4 and #5.

### 6.x.0	High-level solution principles

The following high-level solution principles apply to this solution:

-	A solution supporting multiple sensing data consumer entities (e.g. Sensing Processing Function)

-	The solution supports sensing processing outside the 5GS by leveraging 5GS functions and services

-	The solution supports multiple sensing processing and consumers outside and inside of the 5GS by leveraging 5GS functions and services (i.e. processing, consumption of sensing data is partly done in the 5GS and partly done outside of 5GS)

### 6.x.1	Description

It is anticipated that while sensing is ongoing, one or multiple sensing entities will generate sensing data that requires processing to obtain a final sensing result. This processing may require multiple and independent processing steps (e.g. a user, sensing subscriber, has to include proprietary fusion algorithms or external AIML models for sensing processing). To ensure that processing is fast, reliable, flexible and scalable, it is proposed that multiple Sensing Processing Functions (SePF) are supported. This might require supporting the chaining of multiple SePFs to get a final Sensing Result (or multiple Sensing Results). Furthermore, this proposal allows the selection of a single SePF or multiple SePFs. In case multiple SePFs are used, this solution provides a procedure which allows to deploy a Sensing Processing chain, which is scalable and flexible.

The term processing is used as a general term which includes different kinds of compute and inference processes such as fusion, AIML and rule-based processing. It is also possible that a SePF may be specialized only in one type of computing algorithms, e.g. AI/ML. How the processing is implemented is out of scope of this solution.

In addition, it is also proposed in this solution that a SePF is not necessarily located only in the 3GPP Network. It can be required to use a SePF which is located outside of the 3GPP Network, i.e. in the DN (e.g. Cloud, Internet)).

The proposed procedures enable, for instance, an MNO to offer 5G services that meet customer requirements and provide flexibility for processing in the 5G network and, if necessary, at the customer’s location (e.g., edge, cloud). Customers of an MNO still benefit from the robust 5G network (e.g. guaranteed latency, slicing, QoS) without relying on the MNO’s processing algorithms. Furthermore, this solution fosters an increased privacy. Data processing can be configured and used under 3rd party customer’s control and responsibility.

### 6.x.2	Procedures

#### 6.x.2.1	Procedure for Sensing Data Processing in the Core Network

Figure 6.x.2.1-1: Procedure for Sensing Data Processing in the Core Network

The following is a description of the procedure for processing in the 5GC only:

0.	The procedures of registering network functions and services, evaluating the Sensing Service Request (SSR) by the SeMF and discovering network functions and services required to fulfill the SSR are performed here. The SSR itself is not shown in Figure 6.x.2.1-1. The SSR was triggered before by the Sensing Service Consumer (SSC).

1.	[SensingEntityConfigurationReq]: The SeMF sends a configuration request to a gNB. As an example, the message signature may include an identifier and description of sensing parameters. The identifier is required to identify a SSR during the whole procedure of a sensing session. The description of parameters includes more detailed information, e.g. parameters which are specific to the gNB which needs to be configured for sensing.

NOTE 1:	Interactions between the gNB and any 5GC NF (e.g. SeMF, NEF) are assumed to go via AMF, which is not pictured for simplicity.

2.	[SensingEntityConfigurationReq]: The gNB responds with a message indicating if the configuration was successful or not.

3.	[SensingProcessingReq]: The SeMF sends a processing request to the SePF. The request includes the identifier and additional parameters description configured similar as described in step 1. Additional parameters can be information like which gNBs shall be used, which SSC(s) shall receive the result, or other SePF(s) involved in sensing processing.

4.	[SensingDataReq]: The SePF sends a data request message to the gNB. The SensingDataReq configures the gNB with information on how and what kind of sensing data shall be provided to the requesting SePF. This request includes the SSR identifier and may include other parameters, e.g. whether the data request expects a periodic response (e.g. every 30ms) or a one-off response is expected. As mentioned previously, the data request message may be sent to a Sensing Storage entity to obtain Sensing Data which was stored there before.

NOTE 2:	After this message the Sensing Entity starts the measurements.

5.	[SensingDataRsp]: The gNB (or the Sensing Storage entity, if applicable) responds with sensing data. Depending on the configuration this event can occur periodically or only once. After SePF #1 has received data from the gNB (or Sensing Storage entity), it processes the data.

6.	After processing the data, the SePF #1 needs to know before step 7 and step 8 are triggered which the next processing entity is. To figure that out, the following options are possible:

-	a static pre-configuration at SePF with a list of the next processing entities. In addition, which entity to choose from in this list might be depending e.g. on the sensing result.

-	the SePF may trigger a service discovery on the service bus to identify the next processing entity. Parameters to discover the next entity via a service discovery request are provided by the parameters in step 3.

-	the parameter-set in the request from step 3 may include a list of next possible entities. The SePF has to follow that list.

NOTE 3: 	As an example, the top of the list may be the next target processing entity to visit. After identifying the next entity, the SePF may remove the top from the list and with that the next entry in the list becomes the new top entry. Removing the entity form the top of the list modifies the parameter-set. This changed parameter-set has then to be provided by the actual SePF to the next entity as described in step 8.

7.	[SensingProcessingReq]: SePF #1 sends a SensingProcessingReq message to the SePF #2 to notify that new data for processing will arrive. This step is triggered when not all required SePFs were notified by the SeMF in Step 3.

8.	[SensingResult]: After processing the data, the result is sent via a sensing result message SePF #2. It is required to add all required parameters described in Step 3, 4, 6 to this message as well to ensure that the SePF #2 knows from which entity to request new or additional data (if required), and to which entity to send the result (see optional step 7). This allows that multiple SePFs can be chained in a controlled way. The sensing result message might be an unsolicited message since SePF #2 was notified before by a SensingProcessingReq message that it has to expect data. SePF #2 confirms the result calculation and may provide it to SePF #1, depending on implementation.

9.	The sensing results are delivered to the SSC. Either SePF may provide them, depending on implementation.

10.	[SensingProcessingRsp]: After the sensing result is delivered to the SSC, SePF#1 will send a response message back to the SeMF (or to the requester of the SensingProcessingReq message as in the optional step 7) to signal the success or failure of the request. The response includes a response code as well to indicate the success/failure of the operation.

NOTE 4:	The reason to send the sensing response message back so late in the procedure is avoid the asynchronous usage of the same SSR multiple times, thereby limiting the complexity. If parallelism of the same SSR is required, then the sensing processing response message has to be sent back to the SeMF after Step 3 or to the SePF for the optional Step 7. All later steps will then be executed asynchronously to the original sensing processing request.

#### 6.x.2.2	Procedure for Sensing Data Processing outside the 5GS

Figure 6.x.2.2-1: Procedure for Sensing Data Processing outside the 5GS

The procedure in Figure 6.x.2.2-1 extends the call flow described in clause 6.x.2.1 by introducing the possibility to expose sensing data for external processing at the DN via the Network Exposure Function (NEF). The main differences with Figure 6.x.2.1-1 are that the SePF is not be an entity of the 5G System (e.g. a 3rd party SePF), and therefore interactions between the 5GC (incl. SeMF) and SePF occur via NEF, which exposes interfaces and sensing data.

NOTE 1:	Interactions between the gNB and any 5GC NF (e.g. SeMF, NEF) are assumed to go via AMF, which is not pictured for simplicity.

The step-by-step procedure is as follows:

0.	same as step 0 in Figure 6.x.2.1-1.

1.	same as step 1 in Figure 6.x.2.1-1.

2.	same as step 2 in Figure 6.x.2.1-1.

3.	[SensingProcessingReq]

-	Option a: This option entails an external 3rd Party SePF taking care of the processing of sensing measurements (sensing data), with exposure from the 5GC happening via NEF (for security, authentication and authorization reasons). Other than the involvement of NEF, the description of step 3 in Figure 6.x.2.1-1 is valid here too.

-	Option b: This option follows the procedure as described for step 3 in Figure 6.x.2.1-1.

4.	[SensingDataReq]: This step is nearly the same as step 4 in Figure 6.x.2.1-1. The differences are:

-	Option a: The SePF is a 3rd party SePF. For that reason, the 3rd party SePF must invoke an NEF service (e.g. NefSensingDataReq) which then forwards the message to the gNB, if all security, authorization checks are successful.

-	Option b: This option follows the procedure as described for step 4 in Figure 6.x.2.1-1.

NOTE 2:	After this message gNB starts the measurements.

At this stage, there are two alternatives, depending on whether the 3rd party SePF is involved in the sensing data processing or not.

[Alternative 1]

5a.	[SensingDataRsp]: The gNB (or a Sensing Storage entity) responds with sensing data. Depending on the configuration this event can occur periodically or only once. Here, the SensingDataRsp must be forwarded by the NEF to the 3rd party SePF. After the 3rd party SePF has received data from the gNB (or Sensing Storage), it processes the data.

What happens after or during the processing at the 3rd SePF is beyond the scope of 3GPP SA2. In case internal processing at the 5GC is required after the 3rd party processing, this step is followed by the optional step 6a.

6a.	[SensingProcessingReq, Optional]: This step can be understood as reverse to step 3a above. In Step 3a the SensingProcessingReq is going from the 5GC to an external entity.  In this step 6a the message is coming from the 3rd party SePF to the NEF and then forwarded to the 5GC SePF. This allows an external 3rd party SePF to use the processing capacities of the 5GC. Step 6a is equivalent to the optional step 7 in Figure 6.x.2.1-1.

7a.	[SensingResult, Optional]: Step 8 in Figure 6.x.2.1-1 is the equivalent for this optional step 7a. If a 3rd party SePF requires additional processing from the 5GC, the sensing results from the processing must be made available to the next SePF. In this case the next SePF is in the 5GC. For that reason, the NEF again must handle the message and forward the message to its target in the 5GC (i.e. SePF).

[Alternative 2]

5b.	[SensingDataRsp]: This option follows the procedure just as defined for step 5 in Figure 6.x.2.1-1, followed by processing.

NOTE 3:	After the processing at the SePF completes in both Alternatives 1 and 2, the call flow proceeds with steps 9 and 10 as described in Figure 6.x.2.1-1.

### 6.x.3	Impacts on services, entities and interfaces

SePF:

-	Supports the operation of multiple SePF instances.

-	Supports deployments both within and outside the 5GS.

SeMF:

-	Supports the management of multiple SePFs, within and outside the 5GS.

gNB:

-	Supports exposure of sensing data to the AF.

************ End of changes **************