3GPP TSG-SA2 Meeting #170	S2-2506709

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025		(revision of S2-2505143)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0739 | rev | 1 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedures for Positioning Information Collection for NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CEWiT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_AIML_air-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on SA2 approved LS Reply (S2-2505943) to RAN 3 "Reply LS on AI/ML Positioning Case 3a" in SA2#169 meeting, SA2 has concluded that solution 1 of "Positioning information collection for UEs under positioning" is feasible and captures as follows: “For solution 1 it may be desirable to document related LMF procedures in TS 23.273. There are SA2 impacts to support the solutution but SA2 could address those impacts as aligned with RAN in Rel-19” This CR captures the solution 1 and proposes the procedures for positioning information collection for NG-RAN for AI/ML Positioning Case 3a in TS 23.273. This covers the case for positioning information transfer by LMF for an ongoing UE positioning procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new clause 6.X |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If not approved, the positioning information collection for NG-RAN cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.X(new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Removed 6.X.2 clause |  |  |  |  |  |  |  |  |

# * * * * First Change* * * *

## 6.X	Procedures for Positioning Information Collection for NG-RAN

### 6.X.1	Positioning Information Transfer by LMF for ongoing UE positioning procedure

NG-RAN, as part of data collection for its AI/ML assisted positioning, may require positioning information from LMF. This procedure is used for transferring positioning information to NG-RAN for a UE currently undergoing positioning. The procedure is initiated only when the NG-RAN requests for positioning information collection for the UE being positioned.

NOTE:	The procedure is applicable only for Uplink-based UE Positioning method.

Figure 6.X.1-1: Positioning information transfer for ongoing UE positioning procedure

Pre-condition: Uplink based Positioning procedure is ongoing for a UE in LMF.

1.	As part of an ongoing UE positioning, the LMF sends NRPPa measurement request to multiple NG-RAN nodes to collect UE measurement data.

2.	Upon receiving measurement request from LMF, NG-RAN node(s) may decide to initiate data collection for the UE.

3.	NG-RAN node(s) may include data collection indication to LMF in NRPPa measurement response.

4.	The LMF checks whether the UE, with its corresponding SUPI, provides the user consent for data collection with UDM using Nudm_SDM_Get including subscription data type "User consent". If user consent is not granted, then the remaining steps are skipped, and a failure indication with appropriate cause value is to be sent to NG-RAN.

5.	Once LMF has the positioning information, it sends a Positioning Data Collection Report to the NG-RAN. The message contains information related to UE location and relevant correlation information. The correlation information enables the NG-RAN to map the information related to the UE’s location to the corresponding UE’s positioning measurement data stored within it.

If LMF is unable to provide the positioning information, a failure indication is to be sent to NG-RAN.

NOTE: The failure indication with appropriate cause value to NG-RAN and the contents of the Positioning Data Collection Report are to be decided by RAN WGs.

# * * * * End of the Change* * * *