3GPP TSG-SA2 Meeting #170	S2-2506714

Goteborg, Sweden, 25 – 29 August, 2025

Source:	Samsung

Title:	KI #3: New Sol for Support of IMS voice call over NB-IoT NTN using I1 protocol

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes an IMS call setup procedure reusing I1 protocol over NB-IoT NTN via GEO satellite connecting to EPC.

Discussion

The proposed solution is to reduce IMS SIP signalling overhead for call setup, to use I1 protocol regardless of ICS and CS related entity e.g. ICS-AS, MSC etc.

Instead of introducing ICS AS, P-CSCF can be updated to serve I1 protocol as well as SIP protocol. And such used protocols can be distinguishable via their dedicated UDP ports respectively.

Proposal

It is proposed to add this solution for KI#1 in the TR 23.700-19.

Start of 1st Change

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| XX |  |  | x |  |

Start of 2nd Change (all new text）

6.xx	Solution #xx: Support of IMS voice call over NB-IoT NTN using I1 protocol

6.x.0	High-level solution Principles

This solution addresses KI#3

To enable IMS-based voice services over NB-IoT NTN access through GEO satellite connected to the EPC, the following high-level architectural principles are applied.

-	use I1 protocol in TS 24.294 [xx] for IMS session setup/release over between UE and P-CSCF.

-	P-CSCF switches I1 protocol to/from SIP protocol and SDP offer/answer is missed between UE and P-CSCF since voice call is assumed and its dedicated audio codec(i.e. Ultra Low Bitrate Speech Codec) is pre-configured via NB-IoT NTN thourgh GEO satellite, where transcoding to the audio codec is expected in IMS-AGW.

6.x.1	Description

6.x.1.1	General

6.x.1.3	Use of I1 protocol for IMS voice call Session Setup

It is proposed to use I1 protocol (as defined in TS 24.294 [14]) for IMS voice call setup over NB-IoT NTN through GEO satellite to reduce the signalling overhead. The I1 protocol is designed lightweight as it was used in CS domain for IMS Centralized Services, and originally transaction between CS UE and SCC AS(specified in TS 23.292[15] was considered, but in our situation we are adjusting this I1 protocol regardless of CS or ICS services, and so instead  we applies it for transaction between UE and P-CSCF over NB-IoT NTN through GEO satellite.

Additionally we are assuming that UDP is used as transport layer protocol to carry I1 messages between UE and P-CSCF and in order to differentiate from SIP message transaction, some UDP port numbers are dedicated and used for I1 message transaction.

When a voice call is initiated or received by a user with an IMS subscription and NB-IoT NTN (GEO) access, the session setup via I1 message transaction in place of SIP message transaction b/w UE and P-CSCF with the following:

-	P-CSCF implements an interworking function to receive session signalling over the I1 protocol and translate between I1 and SIP protocol. SDP offer/answer between UE and P-CSCF is not delivered explicitly since voice call is assumed and its dedicated audio codec is used NB-IoT NTN thourgh GEO satellite so that their conten ts can be expected by pre-configured in UE&P-CSCF.

-	IMS-AGW should support transcoding toward UE to use the Ultra Low Bitrate Speech Codec, which is designed for the GEO situation and under study in SA4.

6.x.2	Procedures

6.x.2.1	MO Call Procedures

Figure 6.x.2.1-1 provides an example flow for MO call procedures, where similar procedures are applied to MT call situation also.

Figure 6.x.2.1-1: Example flow for MO call procedures

- PDU Session for IMS use is established, where the I1-capable P-CSCF address and its UDP port dedicated to I1 protocol is shared via PCO from PGW to UE, as UDP port dedicated to SIP protocol can be assigned.

- UE performs IMS registration procedure, which can be done via SIP protocol(or I1-like protocol).

Editor's note:	In case I1-like protocol is used  for IMS registration, it’s FFS how to define new I1-like messages.

- For voice call setup, UE generates I1 invite message without SDP offer, and sends the I1 invite message to P-CSCF with the I1-dedicated UDP port number.

- P-CSCF restores SIP INVITE and its SDP offer based on its access type NB-IoT(GEO) and its pre-configured SDP value. And P-CSCF may request IMS-AGW to keep the dedicated voice codec and allocate the resources for network side over  NB-IoT(GEO).

- P-CSCF sends the restored SIP INVITE message to IMS core and UE2.

- P-CSCF may get response from IMS core/UE2 on session progress by SIP 183 with SDP offer.

- P-CSCF may request IMS-AGW to keep the dedicated voice codec over  NB-IoT(GEO) (i.e. request transcoding to the dedicated voice codec over  NB-IoT(GEO)) and P-CSCF converts the SIP 183 with I1 progress with 183 and miss the SDP offer, whether the I1 progress message sent to UE at step 8.

- Based on the pre configured SDP answer value for voice call over NB-IoT(GEO), dedicated EPS bearer for voice data is established or modified, where this step can be occurred after step 1 before step 6.

- I1 progress message sent to UE, which UE performs behaviour like receiving SIP 183 message.

- the remaining IMS SIP/I1 message transactions for voice call setup are continued.

6.4.3	Impacts on Services, Entities and Interfaces

UE:

-	Support of I1- protocol as well as SIP protocol.

P-CSCF:

-	Support of switching I1 protocol message over NB-IoT NTN(GEO) to/from SIP protocol.

-    in case that access type is NB-IoT NTN(GEO), for the voice call setup procedure the media is defined in SDP as its dedicated audio codec as the Ultra Low Bitrate Speech Codec over NB-IoT NTN(GEO) path.

End of Change