3GPP TSG SA WG2 Meeting #170	S2-2506727

25 – 29 Aug, 2025, Goteborg, SE

Source:	NTT DOCOMO

Title:	[WT#3, all topics] Study how to support and enable use of AI in 6G

Document for:	Agreement

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper based on the technical justifications proposes scope and key issues of the “WT#3: Study how to support and enable use of AI in 6G” of the 6G SID. The paper includes both the “Network for AI” and “AI for Network” aspects.

# 1. Justifications

The current 5G Core specifications possess AI capabilities such as the analytics provided by NWDAF and offer some supports for AI/ML-based applications. However, these capabilities fall short of addressing the complexity and comprehensive AI requirements anticipated for future networks.

From the AI for Network perspective, the main principle of 5GC design was to dedicate NWDAF to the entire set of AI/ML functionalities, including data collection, model training, and providing predictions/analytics. It introduces several fundamental limitations:

- There is no ML model handling framework in 5GC; it is entirely left to NWDAF implementation. Accordingly, interoperability among vendors is a challenging issue and the operators do not have full observability of ML model operations. The 5G approach prevents operators from providing customized ML models based on their own network deployments and it is not straight forward for operators to directly monitor the performance of ML models or to understand their behaviour despite the direct impact that ML model outputs can have on overall network performance.

- This centralized design causes performance bottleneck in the network. AI/ML operations are inherently data- and compute-intensive. NWDAF must collect a large amount of data, train multiple ML models, and then perform several simultaneous inferences in response to requests from other NFs. This results in a heavy load on NWDAF and introduces delays that may be unacceptable for near real-time operations.

The second fundamental principle in the 5GC AI for Network design is the concept of Analytics, which are provided by the NWDAF. By definition, the analytics are predictions (and statistics) of certain system parameters. Consumer NFs may subscribe to these predictions and optionally take them into account in their decision-making processes. This design also imposes a number of limitations.

- There is a lack of intelligence in 5G core network functions. The NWDAF, as the ML model training entity, has no visibility into the impact of its analytics on network performance. It is up to the implementation of the consumer NF to decide whether and how to use the analytics. Consequently, NWDAF cannot improve ML model performance based on observation of system performance impacted by the ML model inferences.

- Supported AI/ML techniques are quite limited. The AI/ML techniques are evolving rapidly, with new methods constantly being introduced. However, the current 5GC specification for Analytics is primarily based on traditional supervised learning. While recent 5GC releases support federated learning and unsupervised machine learning (mainly for anomaly detection), there are still many other ML techniques, such as reinforcement learning, transfer learning, online learning, large language models (LLMs), etc., that are not supported in 5GC specifications, despite their demonstrated potential to improve network performance and user experience in academic research.

The third category of limitations in the current design of 5GC AI for Network is the lack of consideration for deployment requirements.

- Another missing aspect in the 5GC AI for Network design is sustainability. The deployment and operation of AI/ML models need a significant amount of data collection and are inherently computationally intensive. Currently, no energy or sustainability metrics have been considered in the current AI/ML design within 5GC.

- End-to-end AI/ML based solutions deployment in the operator’s network is quite complex. The network operators often need to carry out considerable integration and validation work to ensure that AI/ML solutions from different vendors function well together. This process increases the time, cost, and complexity of deployment, which can slow down the broader adoption of AI/ML technologies in the network. Troubleshooting and fixing problems in a multi-vendor, AI-powered environment is harder when the underlying AI/ML logic is not transparent or standardized.

From the Network for AI perspective, the 5GC is not ready to fully support the emerging AI/ML-based solutions.

- 5G core architecture is communication-centric and lacks any defined compute services for AI/ML. There is no standard support for deploying AI inference tasks within core network elements or at the network edge.

- The 5G core does not yet provide specialized data handling for AI traffic while some AI services (such as split cloud/device inference for video analytics) may require extremely high throughput and ultra-low latency simultaneously.

- The 5G core relies on the Network Exposure Function (NEF) and standardized APIs to share network information or accept requests from external applications. However, these exposure capabilities are limited in scope and strictly controlled, which hinders advanced AI/ML applications that crave rich network data.

- Advanced AI/ML applications like intelligent agents often benefit from a distributed deployment (e.g. part of the model on the device, part at the edge/cloud) and from cooperation between multiple agents. The 5G core architecture, however, does not explicitly support multi-node agent orchestration as an inherent feature. This lack of distributed AI awareness means higher latency and complexity when trying to deploy intelligent agents that seamlessly span the device, edge, and cloud, as the network cannot actively facilitate their operation under the current architecture.

Based on these identified limitations in the 5GC (from AI for network and Network for AI perspectives), it is proposed to study how to design the 6G core to address the gaps. The scope of the study and the associated key issues are discussed in the following.

**** First Change ****

# Annex A.X. WT#3 Scope

The scope of the WT includes the following aspects:

- Designing AI/ML models handling framework that enables provisioning, configuring/customizing, training, deployment and monitoring of ML models in the multi-vendor 6GC deployment scenarios

- Designing architectural framework to enable AI-native NFs (via decision making by ML model, evaluating the decisions and online ML model (re)training) in 6GC

- Developing solutions to address reliability and sustainability of AI/ML based solutions

- Designing architectural framework to facilitate deployment of AI/ML based applications (e.g., AI Agents) on top of the 6GC

**** Next Change ****

# 5.X. Key Issue #X:  ML Model Handling Framework in 6GC

This KI will study the following aspect of the ML Model handling framework in 6GC:

- Whether and how to support customized ML model provisioning/deployment in 6GC NF functions

- Whether and how to support ML model evaluation and operation visibility in 6GC NF functions

- Whether and how to support ML model (re)training management in 6GC NF functions

- Whether and how to support interoperability for model provisioning, training and monitoring in the multi-vendor deployment scenarios

**** Next Change ****

# 5.X. Key Issue #X:  AI-Native NFs in 6GC

This KI will study the following aspects of AI-native NFs in 6GC:

- Whether and how to support decision making by ML models in 6GC network functions

- Whether and how to support evaluating the impact of decision by ML models in 6GC NFs

- Whether and how to support model (re)training based on the evaluation of network performance

**** Next Change ****

# 5.X. Key Issue #X:  Reliability and Sustainability of AI/ML in 6GC

This KI will study the following aspects of reliability and sustainability of AI in 6GC:

- Whether and how to assess reliability and sustainability of AI/ML in 6GC

- Whether and how to improve reliability of AI/ML in 6GC

- Whether and how to improve sustainability of AI/ML in 6GC

**** Next Change ****

# 5.X. Key Issue #X:  Supporting AI/ML based Applications on 6GC

This KI will study the following aspects of supporting AI/ML based applications (e.g. AI Agent) on top of 6GC:

- Identify AI/ML based application requirements (e.g., communication, computation, data) to be deployed on top of 6GC considering the use cases and requirements documented by SA1.

- Identify architectural enablers to address the AI/ML based application on top of 6GC

- Whether and how to design architecture to support deployment of AI/ML based application on top of 6GC

- Whether and how to address privacy, security and charging aspects of services provided to AI/ML based application on top of 6GC

- Whether and how to address user consent and application layer traffic identification.

**** End of Changes ****