SA WG2 Meeting #170	S2-2506739

25 – 29 Aug, 2025, Goteborg , SE

Source:	MediaTek Inc.

Title:	[WT#1.3, all topics] Study proposal for non-3GPP access

Document for:	Agreement

Agenda Item:	20.6.1.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the analysis and the study proposal for non-3GPP access.

1. Justifications

Architectural Motivation:

N3IWF/ TNGF have been specified for non-3GPP access in 5GS since R15.  They are already a well fit to the system in many aspects.

There is a clear functional boundary in non-3GPP access architecture in 5GS. Entities enabling non-3GPP access (e.g., N3IWF) play a similar role as NG-RAN in 3GPP access. The remaining core network functions (e.g., AMF) work as they do in 3GPP access. Similar principles as 3GPP access followed on authentication and security management.

In particular, the NAS signalling used to communicate between the UE and the core network is the same protocol utilized in both 3GPP and non-3GPP access. This characteristic allows the UE to communicate more easily with core network functions without the need to define a new protocol.

From protocol aspect, we can see N3IWF/TNFG bring several advantages over non-3GPP access design in EPS (e.g. based on ePDG). A few cases are exemplified as below:

|  | N3IWF/TNGF | ePDG | Preference |
| --- | --- | --- | --- |
| Connection management | UE context will be retained in the AMF even if the UE enters idle. | UE context will be lost when the IPsec is lost. | N3IWF/TNGF is preferred. Retaining UE context in the AMF allows the possibility of temporarily disabling Wi-Fi, which is crucial for power saving. |
| Registration procedure | Yes. | Combined with PDN connectivity establishment procedure | N3IWF/TNGF is preferred. Exchange sufficient information before initiating other procedure. |
| Additional PDN/PDU | Using established IPsec SA to send NAS (SM) signaling | Needs to start from IKE_SA_INIT | N3IWF/TNGF is preferred. N3IWF/TNGF consist fewer steps than ePDG solution. |
| PDN/PDU type | All types defined in 3GPP are allowed. | Only IP type allowed | N3IWF/TNGF is preferred. |

Gap Analysis:

Despite several advantages of Non-3GPP access design in 5GS, we still see some enhancement can be done to reduce complexity. As an example, 3GPP defines different handling in N3IWF for a UE registering to the same PLMN versus to different PLMNs. The procedure and the context handling for two accesses in 5GS, when the UE is registered in the same PLMN, are too tightly coupled. In other words, the UE and the network need to consider the state on the other access.

-	It is unclear for a UE whether the information that is only applicable to 3GPP access but received over non-3GPP access should be ignored or retained.

-	This design may degrade performance, because the UE must wait for registration procedure complete over the other access if the UE wants to register to the same PLMN.

Technical Impact:

-	A harmonized approach for non-3GPP access (e.g., trusted, untrusted, wireline)

-	Using NAS signalling similar to non-3GPP access solutions in 5GS to communicate with the core network as the starting point.

-	Minimize dependency between 3GPP access and non-3GPP access

-	NAS parameters and context (e.g., GUTI, security context, MM context) need to be maintained independently, regardless of whether the UE is registered to the same PLMN or not.

-	The UE and the network are able to perform procedure (e.g., registration procedure) over both accesses simultaneously and independently.

**** First Change ****

Annex A.X. WT#1.3 Scope

Extracted below from SP-250806:

1.3. Study whether and how to support and/or enhance different non-3GPP access (e.g. Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

Note the below update is just intended as information in the TR and no change is expected in the WT wording in the SID.

This WT study whether and how to support and/or enhance different non-3GPP access (e.g., Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

-	Whether and how to harmonize control signalling for non-3GPP access with 3GPP access?

-	Whether and how to decouple UE context and procedures of different access?

-	Whether and how to support non-3GPP access interworking with legacy system.

NOTE:	This study assumes non-3GPP access in 5GS as the starting point.

**** Second Change ****

5.X. Key Issue #X: Support of non-3GPP access in 6G

Study whether and how to support and/or enhance different non-3GPP access (e.g., Wi-Fi, wireline) in 6G and support multi-access data connections between 3GPP access and non-3GPP access.

-	Whether and how to harmonize control signalling for non-3GPP access with 3GPP access?

-	Whether and how to decouple UE context and procedures of different access?

-	Whether and how to support non-3GPP access interworking with legacy system.

NOTE:	This study assumes non-3GPP access in 5GS as the starting point.

**** End of Changes ****