SA WG2 Meeting #170	S2-2506752

25 – 29 Aug, 2025, Goteborg , SE

Source:	CATT

Title:	[WT#1.1] Discussion and proposal on key issue for work task#1.1

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the motivation for work task#1.1, analyses the gap and proposes the scope and key issue accordingly.

1. Justifications

1.1	Architectural Motivation

In 5G, the AMF is the termination point of NAS message and supports multiple functions, for example, registration management, authentication and authorization, connection management, SMF selection and session management messages routing, LMF selection and location service messages routing. The combination of different functions in AMF leads to the following issues:

-	Issue#1: Inflexible support of new service

With the rapid development of AI agent services, computing, sensing and other beyond connectivity services, the iteration speed of future services is accelerating. In 5G network, the AMF is the termination of NAS, this means the AMF and NAS should be enhanced when introducing new services. This significantly increases the time-to-market for new services and the maintenance costs. So the 5G network cannot meet the operator’s needs for rapid deployment of new services.

-	Issue#2: Inefficient routing for 5GC NF deployed in local network

In general, the AMF is deployed in public network. For network function(s) (e.g. LMF) deployed in local network which could be a PNI-NPN, the signals transferred between UE and network function need to be routed via the public network and then back to the local network, so the routing is inefficient. With routing the signals and the data (e.g. positioning measurements) into the public network, it struggles to fulfil industry customers’ requirements for privacy protection and on-premises data processing.

1.2	Gap Analysis

1.2.1	Analysis on issue#1

Based on description in clause 8.2.2.1, the NAS-MM supports the following capabilities:

-	NAS procedures that terminate at the AMF. This includes:

-	Handles Registration Management and Connection Management state machines and procedures with the UE, including NAS transport; the AMF supports following capabilities:

-	Provide a secure NAS signalling connection (integrity protection, ciphering) between the UE and the AMF, including for the transport of payload;

-	Provide access control if it applies;

-	It is possible to transmit the other type of NAS message (e.g. NAS SM) together with an RM/CM NAS message by supporting NAS transport of different types of payload or messages that do not terminate at the AMF, i.e. NAS-SM, SMS, UE Policy and LCS between the UE and the AMF.

And the NAS transport for SM, SMS, UE policy and LCS is shown in Figure 1.

Figure 1: NAS transport for SM, SMS, UE Policy and LCS

Based on the existing mechanisms above, when new service is deployed in operators’ network, besides introducing new NF in core network, the NAS-MM and AMF also need to be enhanced, as follows:

-	A new type of payload or message need to be introduced to support the control plane message transfer between UE and the NF for the new service. So the NAS-MM shall identify and handle the new type.

-	The AMF needs to be enhanced to select the suitable NF for the new service and support the control plane message routing between the UE and the NF for the new service.

The above enhancements increase the time-to-market for new services and the maintenance costs. So the 5G network cannot meet the operator’s needs for rapid deployment of new services.

In 6G, to meet the operator’s needs for rapid deployment of new services, it is proposed to study the following issue for work task#1.1:

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

1.2.2	Analysis on issue#2

The issue#2 has been studied in Rel-18 FS_eLCS_Ph3 SID. In order to fulfil industry customers’ requirements for privacy protection and on-premises data processing, the LMF and GMLC is deployed in the local network as shown in Figure 2. And a local AMF is introduced in the local network to transfer positioning message between UE and LMF.

Figure 2: PNI-NPN architecture to support location service with signalling optimisation

However, based on the conclusion, the local AMF can only transfer very limited positioning message, i.e. non-UE associated NRPPa messages between UE and LMF. The majority positioning messages included in the positioning procedures still need to be transferred via the serving AMF in the public network. So in general, this kind of solution does not solve the issue totally.

The source of this issue is that the messages between UE and control plane NF are always transferred via serving AMF which always deployed in the public network. So it is proposed to study the following issue for work task#1.1:

-	Whether and how to develop general mechanisms for UE-Core Network interaction to support different deployment requirements.

1.3	Technical Impact

The work task#1.1 influence:

-	Overall system architecture.

-	Interaction between UE and control plane NF.

**** First Change (all text new) ****

Annex A.X. WT#1.1 Scope

The scope of WT#1.1 is as follows:

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

**** Second Change (all text new) ****

5.X. Key Issue #X: Support of 6G non-access stratum functionality

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

6G supports connectivity services and beyond connectivity services. The connectivity services are responsible to provide connectivity to transfer user data. The beyond connectivity services are not associated to user data, e.g. AI service, sensing service.

To support non-access stratum functionality for different services, solutions for this key issue will study:

-	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

-	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

-	Whether and how to develop generic mechanisms for UE-Core Network interaction to support different deployment requirements.

**** End of Changes ****