SA WG2 Meeting #170	S2-2506753

25 – 29 Aug, 2025, Goteborg, SE

Source:	CATT

Title:	[WT#1.4] Discussion and proposal on key issue for location service in work task#1.4

Document for:	Agreement

Agenda Item:	20.6.1.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the motivation for location service in work task#1.4, analyses the gap and proposes the scope and key issue accordingly.

- Introduction

There are multiple essential/regulatory services included in work task#1.4 as follows:

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

The different services support different architecture, functionalities and procedures, so the services need to be discussed separately. This pCR focuses on the location service.

Based on the description in clause 5.2.3.2 in TR 22.870, the 6G system is assumed to support most of the existing 5GS service requirements, including the location service. So this pCR discusses the gap and proposes the scope and key issue for location service.

- Justifications

2.1	Analysis on 5G location service

In 5G, the location service has been supported since Rel-15. Based on location service architecture, the GMLC, LMF and AMF are mandatory NFs to support location service. This clause analyses the drawbacks of 5G location services.

2.1.1	Positioning message routing

AMF is always involved to transfer the LPP message(s) and NRPPa message(s). The LPP message(s) is handled by UE and LMF and the NRPPa message(s) is handled by RAN and LMF. The illustration of routing of LPP and NRPPa messages is shown in Figure 1.

Figure 1: illustration of routing of LPP and NRPPa messages

The role of AMF is to transparently transfer the message between UE/RAN and LMF, i.e. the AMF does not handle the message. So the AMF involvement leads to inefficient message routing and introduces extra latency, i.e. message encapsulation and de-capsulation delay in AMF.

In Rel-18, to avoid AMF involvement for LPP message routing, the user plane positioning is supported to transfer the LPP message via the user plane. However the mechanism requires the UE always keeps the user plane positioning dedicated PDU session active which leads to high power consumption. Furthermore the mechanism only applies to LPP, does not apply to NRPPa.

Observation#1: The AMF involvement for LPP and NRPPa messages routing leads to inefficient message routing and introduces extra latency.

Observation#2: The user plane positioning can avoid AMF involvement for LPP routing. However this solution requires UE always keeps the dedicated PDU Session active which leads to high power consumption.

Observation#3: The user plane positioning does not apply to NRPPa message, so the message is always routed via AMF.

Observation#4: The observation#1 is caused by the location service architecture design in 5G.

Based on the observations above, it is proposed to study the following issues for location service in 6G:

Proposal#1: The architecture to support location service.

Proposal#2: How to achieve an efficient positioning message routing mechanism.

2.1.2	Location service support in local network

In Rel-18, it has been studied how to fulfil the industry customers’ requirements for privacy protection and on-premises data processing. To fulfil the requirements, the LMF is deployed in local network and local AMF is introduced to transfer the positioning message between UE and LMF. The architecture is shown in Figure 2.

Figure 2: The architecture to support location service in local network

Based on Figure 2, the routing of positioning messages are as follows:

-	LPP message: the message is routed between local network and public network. For example, when UE sends LPP message to LMF, the message is routed from UE to serving AMF in public network, and then back to the LMF in local network.

-	UE associated NRPPa message: the message is routed between location network and public network. For example, when RAN sends the UE associated NRPPa message, the message is routed from RAN to serving AMF in public network, and then back to the LMF in local network.

-	Non-UE associated NRPPa message: the message is routing inside the local network. For example, when RAN sends the non-UE associated NRPPa message, the message is routed from RAN to the LMF in the local network.

So in the local network deployment, only the routing of non-UE associated NPRRa message is optimized. In general, this kind of message is used by LMF to send GNSS assistance data to be broadcasted via radio interface to RAN, so this message does not include the UE’s positioning data, e.g. measurement data, UE location. However, the LPP message and UE associated NRPPa message including the UE positioning data still need to be routed via the public network. So the mechanism cannot fulfil the industry customers’ requirements for privacy protection and on-premises data processing.

Observation#5: The industry customers’ requirements for privacy protection and on-premises data processing are not fulfilled based on existing mechanism.

Based on the observation above, it is proposed to study the following issue for location service in 6G:

Proposal#3: how to support the location service in NPN and local network, to fulfil industry customers’ requirements for privacy protection and on-premises data processing.

2.1.3	Routing identifiers

To support UE positioning, several messages are specified:

-	LPP message which is handled by UE and LMF.

-	Supplementary service (SS) message which is handled by UE and LMF.

-	NRPPa message which is handled by RAN and LMF.

To route the messages above, different routing identifiers are specified as shown in Figure 3

Figure 3: Routing identifiers used to route different positioning messages

As shown in Figure 3, to route different positioning messages, different routing identifiers are used. The summary of the routing identifiers is described in Table 1.

Table 1: Routing identifier summary

| Routing ID | Function | UE/RAN impacts |
| --- | --- | --- |
| LCS Correlation ID | - AMF uses the Correlation ID to route LPP messages between UE and LMF. - AMF uses the correlation ID to route SS message between UE and LMF. | UE needs to identify the LCS Correlation ID and provides the parameter to AMF. |
| LMF ID | AMF uses the LMF ID to route the NRPPa messages between RAN and LMF | RAN needs to identify the LMF ID and provides the parameter to AMF. |
| Deferred routing ID | AMF uses the deferred routing ID to route the SS messages between UE and LMF | UE needs to identify the deferred routing ID and provides the parameter to AMF. |

Based on the description above, the route different messages, different routing identifier is used. This design increases the complexity of implementation. Furthermore, UE needs to identify different routing identifiers to route different messages, this also increases UE complexity.

Observation#6: To route different positioning messages, different routing identifiers are specified. This increases UE complexity and implementation complexity.

Based on the observation above, it is proposed to study the following issue for location service in 6G:

Proposal#4: How to achieve an efficient positioning message routing mechanism.

2.1.4	Deferred location request

With a deferred location request, an LCS client or AF sends a location request to a PLMN for a target UE (or group of target UEs) and expects to receive a response containing the indication of event occurrence and location information if requested for the target UE (or group of target UEs) at some future time (or times), which may be associated with specific events associated with the target UE (or group of target UEs).

To support the deferred location request, UE is responsible to determine whether the event happens or not. If yes, the UE further notifies the LMF that the event happens. In this case, the LMF is responsible to notify the GMLC that the event happens and along with UE location if requested by LCS Client or AF. So the existing procedure has lots of impacts to UE. Furthermore, the procedure is also very huge and complexed. All these factors increase the difficulty of commercial application.

Observation#7: The deferred location request procedure is complexed and has lots of impacts to UE.

Based on the observation above, it is proposed to study the following issue for location service in 6G:

Proposal#5: how to support the deferred location reporting in a simplified manner compared to 5G.

2.2	Analysis on 6G location use cases and requirements

2.2.1	Introduction

Based on TR 22.870, the use cases related to location service can be divided into three categories:

-	Category#1: the use cases involving AI and location e.g. use cases in clause 6.3, clause 6.6 and clause 6.13.

-	Category#2: the use cases involving sensing and location, e.g. all the use cases in clause 7 with KPI including accuracy requirement of location.

-	Category#3: GNSS free positioning use case in clause 8.4.

This clause analyses the gap for location service to support the use cases above.

2.2.2	Gap analysis on category#1

For the use cases belong to category#1, in general, the location is used as input parameter for AI inference. The network function supporting AI can invoke the location service to obtain the UE location. So the location service shall support service exposure to the network function supporting AI.

Observation#8: There are multiple AI related use cases in clause 6 in TR 22.870 requires UE location. To support the requirement, the location service shall support service exposure to the network function.

Based on the observation above, it is proposed to study the following issue for location service in 6G:

Proposal#6: how to support location service exposure, e.g. to 6GC NF, AF.

2.2.3	Gap analysis on category#2

Based on the use cases in clause 7 in TR 22.870, the KPI for each use case includes accuracy requirement on location. So location of sensing target is an important and mandatory sensing result. However the sensing target is an object or environment. And the target of the location service is the UE registered in network. So the positioning of such sensing target is not in scope of location service.

Observation#9: the location of sensing target is an important and mandatory sensing result. However the sensing target is an object or environment which is not covered by location service, because the target of location service is UE registered in network.

2.2.4	Gap analysis on category#3

Based on the use case in clause 8.4 in TR 22.870, the requirement of the use case is as follows:

[PR 8.4.6-1]	The 6G system with satellite access shall be able to provide the UE with positioning service with 3GPP technologies, independently of non-3GPP positioning technologies (e.g. GNSS) with the following KPIs:

Table 8.4.6-1: Performance requirements for satellite-based positioning services

| Scenario | Accuracy (95 % confidence level) |  | Positioning service availability | Positioning service latency | Environment of use | UE speed | UE type |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Horizontal Accuracy | Vertical Accuracy |  |  |  |  |  |
| Airplane en-route | [50] m | [50] m | [99 %] | [1] s | Outdoor | Up to [1500] km/h | Airplane mounted |
| Airplane landing | [10] m | [10] m | [99 %] | [1] s | Outdoor | Up to [350] km/h | Airplane mounted |
| UAV positioning | [1] m | [1] m | [99 %] | [1] s | Outdoor | Up to [160] km/h | UAV mounted |
| NOTE: Multiple satellites can be used to support 3GPP positioning technologies. |  |  |  |  |  |  |  |

The function required by the requirements above is that for UE using satellite access, RAT dependent positioning method(s) is used to determine UE location, instead of GNSS assisted positioning method.

Actually in 5G, the function above has been studied and specified to support UE location verification which requires the location accuracy of 5 – 10km. The precondition of the study is that there is only one LEO taking part in the positioning measurement. Considering the SA1 requirement of location accuracy is higher than 50m, so the existing mechanism cannot fulfil the requirement. Furthermore, the positioning latency of existing mechanism is about 20s which also cannot fulfil the SA1 requirement of 1s above.

Observation#10: for UE accesses satellite access, to support UE location verification in 5G, the RAT dependent positioning method is used to obtain UE location. The location accuracy is 5 – 10 km and the positioning latency is around 20s. The existing mechanism cannot fulfil the SA1 requirements.

Based on the observation above, it is proposed to study the following issue for location service in 6G:

Proposal#7: for UE accessing over satellite, how to support location service, independently of non-3GPP positioning technologies (e.g. GNSS).

2.3	Technical Impact

The location service in work task#1.4 influences:

-	Location service architecture.

-	Network functions and procedures to support location service.

**** First Change (all text new) ****

Annex A.X. WT#1.1 Scope

The scope of WT#1.4 is as follows:

1.4.	Study whether and how to support and/or enhance the essential/regulatory services (i.e. voice, Messaging, location services, Emergency services, MPS, Mission Critical services, PWS) in 6G.

**** Second Change (all text new) ****

5.X. Key Issue #X: Support of 6G location service

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

To support location service in 6G, solutions for this key issue will study:

-	The architecture to support location service.

-	The network functions and procedures to support location service.

-	How to support location service exposure, e.g. to 6GC NF, AF.

-	How to achieve an efficient positioning message routing mechanism.

-	How to support the location service in NPN and local network to protect privacy of location information and measurement data.

-	How to support the deferred location reporting in a simplified manner compared to 5G.

-	For UE accessing over satellite, how to support location service, independently of non-3GPP positioning technologies (e.g. GNSS).

**** End of Changes ****