3GPP TSG SA WG2 Meeting #170			S2-2506756

Goteborg, Sweden, 25th - 29th Aug, 2025	(revision of S2-250xxxx)

# Source:	CEWiT

# Title:	New WID on an optimized method for AF to acquire UE IP address

# Document for:	Information

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: 	WID on an optimized method for AF to acquire UE IP address

Acronym:	TEI20_AF_ACQ_UEIP

Unique identifier:		TBD

Potential target Release:	Rel-20

# 1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X | X | X |  | X |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| X | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

# 3	Justification

For analytics generation and ML Model training, NWDAF may request AF to collect data from UE Application(s) by providing UE ID (SUPI or GPSI) as specified in clause 6.2.8.1 of TS 23.288. To collect UE data, the AF has to translate UE ID to UE’s DN-facing IP address that is the IP address as seen at N6 interface. Currently, AF obtains the DN-facing UE IP address, corresponding to the UE ID (SUPI or GPSI) provided by the NWDAF, using two procedures:

- Procedure-1: UE private IP address is obtained using the procedure defined in clause 6.2.8.2.4.2 of TS 23.288.

- Procedure-2: UE public IP address is obtained using the procedure defined in clause 6.2.8.2.4.5 of TS  23.288.

If NAT is not enabled, the AF uses Procedure-1 to obtain DN-facing UE IP address. If NAT is enabled, then both the procedures must be invoked sequentially.

In the existing method to fetch the DN-facing UE IP address, following gaps are identified:

- The AF should know if NAT is enabled for the data flow of its interest, to choose appropriate procedure. Currently, there is no clear mechanism for AF to know the NAT status.

- The AF needs to interact with multiple NFs like UDM, SMF and UPF in sequence.

- The method is not optimized for group of UEs.

- The method only supports ongoing PDU session and not future PDU session(s).

The scope of this WID is to address the above gaps. An effective approach to address these gaps may be for 5G Core to offer a single service which the AF could utilize to get the DN-facing UE IP address irrespective of NAT status.

# 4	Objective

The following aspects will be specified:

WT 1:	Enhance the services offered by 5G Core to AF to retrieve UE IP address as seen at N6 interface.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| TS 23.288 | Describe an optimized procedure to support UE IP address retrieval by AF irrespective of NAT status. | SA#110 (Dec 2025) |  |
| TS 23.502 | Enhance the services of 5G core NFs, for AF to obtain the DN-facing UE IP address | SA#110 (Dec 2025) |  |

# 6	Work item Rapporteur(s)

Anusuya B, CEWiT (anu@ cewit.org.in)

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

CEWiT