SA WG2 Meeting #170	S2-2506775

25 – 29 Aug, 2025, Goteborg , SE

Source:	ZTE

Title:	[WT#1.1, NAS] Discussion and proposal for NAS

Document for:	Discussion

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the detailed scope for WT#1, specifically for sub working task 1.1 in SA2 6G SID. Based on the discussion, a detailed key issue is proposed.

1. Justifications

1.1	Architectural Motivation/ Gap Analysis

In existing 5G System, AMF is responsible for many functionalities e.g. connection management, mobility management, registration management, termination of RAN CP interface N2, termination of NAS N1 including NAS ciphering and integrity protection and so on. All these functionalities managed by the single termination point for core network make AMF complex which reduces the robustness due to signalling load impact and may become the bottleneck for network performance. The AMF/NAS signalling should be simplified in 6G, especially for some scenarios, e.g. FWA.

In addition, since all NAS signalling are routed through AMF, AMF has to be upgraded whenever a new feature is introduced. Also when the AMF is deployed in centralized location while the SMF is deployed close to the RAN, all SM NAS signalling has to go via AMF. This may cause privacy issue for localized services. In order to improve the system efficiency, and also resolve the privacy issue, it is desired that the SM NAS signalling can be routed without relying on the centralized AMF.

In the SA2 6G SID, the following WT is approved.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services

1.2	Technical Impact

The following aspects can be considered in this regard:

1) The functionalities managed by AMF can be decomposed into dedicated and independent NFs.

For example, functionalities related to connection (i.e. connection, N2 connection) can be managed by one dedicated NF e.g. CMF (connection management function) and functionalities not related to connection (e.g. authentication, subscription control) can be managed by another dedicated NF e.g. RMF (registration management function). The CMF can route the NAS signalling towards other NFs e.g. SMF. The CMF can be deployed close to the RAN, which can improve the NAS signalling transmission efficiency. The RMF can be deployed at centralized location so it doesn’t have to be re-allocated frequently in mobility scenario.

The following is one example of the AMF decomposition. How to split the CMF and AMF needs to be further discussed.

2) Distributed NAS between UE and multiple NFs and possibly NAS over UP for non-essential/add-on service (beyond connectivity services).

For basic NAS functionalities (for connectivity service), e.g. MM, SM and possibly policy, in order to ensure signalling efficiency and user experience, direct NAS connection via control plane between UE and AMF/SMF/PCF may be more suitable than via user plane.

For non-essential/add-on services (beyond connectivity services), e.g. LCS, AI service and computing service, data exchange between UE and core network can be routed via user plane or control plan. In this case, any introduction of new services will not force to upgrade the other service, especially on the UE side. This will also benefit the new service deployment.

2. WT Scope

No update in the Working task description

No need for sub-WT.

3. Potential KI(s) Scope

It is proposed to study the following key issues:

KI#X: Study how to support NAS signaling between UE and network for connectivity services. This will include the following aspects:

- Study whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities

- Study whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

KI#Y: Study how to support NAS signaling between UE and network for beyond connectivity services. This will include the following aspects:

- Study whether and how to enable the introduction of a new beyond connectivity service without impacting the connectivity service and other beyond connectivity service.

- Whether and how to develop generic mechanisms for UE-Core Network interaction to support beyond connectivity services, e.g. connection establishment, service authentication/authorization, service exposure between UE and Core network, etc.