SA WG2 Meeting #170	S2-2506776

25 – 29 Aug, 2025, Goteborg, SE

Source:	ZTE

Title:	[WT#1.2, Localized network access] Discussion and proposal for Localized network access

Document for:	Discussion

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the detailed scope for WT#1, specifically for sub working task 1.2 for support and enhancement of localized service access in SA2 6G SID. Based on the discussion, a detailed key issue is proposed.

1. Justifications

1.1	Architectural Motivation/ Gap Analysis

[TS23.501] A Localized Service is a service, which is provided at specific/limited area and/or can be bounded in time. The service can be realized via applications (e.g. live or on-demand audio/video stream, electric game, IMS, etc.), or connectivity (e.g. UE to UE, UE to Data Network, etc.).

A Localized Service provider is an application provider or network operator who make their services localized and that are offered to the end user via a network. A network providing Localized Services can be an SNPN or a PNI-NPN

The approved SA2 6G SID includes the following work task:

WT#1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

In SA1, the following use cases and potential new requirements on localized service access are captured in the TR 22.870 v030.

| 5.6.2 | Fast network provisioning to improve resilience | [PR 5.6.2.6-1] Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g. in response to an urgent event (e.g. disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. NOTE 1: The level of local control can be based on operator policies and agreements with 3rd party. For example, the authorization and policy control of users to access the provisioned services are not affected by the failure of the operator’s PLMN network. NOTE 2: The enabled functionalities can be based on operator policies and agreements with 3rd party. For example, data connectivity service and voice service are prioritized when an urgent event happens in a residential community; small data transfer service is prioritized when an urgent event happens in an IoT based farmland. NOTE 3: Some situations can target the required network services to be provisioned within hours to serve certain users whose QoE is impacted by an urgent event. NOTE 4: Local control refers to the capability of part of the operator’s PLMN network to operate autonomously and independently, e.g. management of local subscription, local traffic, without interaction with the operator’s PLMN. [PR 5.6.2.6-2] Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator. NOTE 5: This applies to scenarios where a service is not available in the local network services that have been provisioned on-demand but is available from the PLMN of the same operator. |
| --- | --- | --- |
| 5.9.2 | Use case on 6G local area networks | [PR 5.9.2.6-1] Subject to operator’s policy and agreement with 3rd party, the 6G network shall support a mechanism to start and stop offering certain network service(s) in a local area network adapting to the demand of e.g. the users, 3rd party or the network operator. |
| 11.9 | Use case on 6G localized network for vertical | [PR 11.9.6-1] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize UEs owned by a trusted authorized 3rd party (e.g. vertical industry) to access services provided by a localized network (deployed by the PLMN operator for the authorized 3rd party). [PR 11.9.6-2] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. |
| 11.10 | Use case on in-vehicle local communication | Potential new requirements to enable local connectivity within a vehicle are listed as follows: [PR 11.10.6-1] Subject to regulatory requirements and the operators’ policy, 6G system shall be able to support (partial-)autonomous local networks for the communication between UEs and devices. NOTE: (Partial-)autonomous means that the management of local network can be done locally even in case of loss of connectivity to the mobile network. [PR 11.10.6-2] Subject to regulatory requirements and operator policy, the 6G network shall allow the operator to provide the required control information including configurations to the local network that enables it to provide necessary configurations for the local communication and to operate also out of coverage of the mobile network. [PR 11.10.6-3] Subject to regulatory requirements and the operators’ policy, the 6G system shall provide the possibility of local authorization for a new device or UE joining the local network. |

1.2	Technical Impact

Optimization on the localized service access

In 5G, a network providing Localized Services can be an SNPN or a PNI-NPN, or Edge computing. The following issues need to be studied in 6G.

- Support for access to PLMN services via Stand-alone Non-Public Network and access to Stand-alone Non Public Network services via PLMN have dependency on non 3GPP access mechanisms, e.g. N3IWF. However the N3IWF has not been widely deployed in real markets. In 6G there is need to study other mechanism to access SNPN via PLMN, or access PLMN via SNPN, or access SNPN via SNPN.

- The localized service is a service provided at specific/limited area and/or can be bounded in time. This may have characteristics which don’t need full 5G features. Therefore there is need to study the characteristic for localized service and study how to define a minimal set of existing or new features which are essential for localized service. For example, whether and how to simplify the network when the network start or stop the localized service based on demand, e.g. without impact on the UE.

- The localized service may also be accessed via Edge Computing. When the operator deploys localized service via Edge Computing, there is need to minimize the impact on network functions for existing services, e.g. PCF/A-SMF/A-UPF. In 6G there is need to study how to improve the system that allow the operator to provision localized service on demand without any impact on existing services.

Support network resilience via localized service access

Network resilience is one of important requirements for 6G system.. The following aspects need to be studied in 6G.

- During an urgent event (e.g. disaster, emergency and DDoS event), when the backhaul is not available/limited, how the network can provide localized service for both public safety user and commercial user.

- In 5G, disaster roaming is support between PLMNs. In 6G it is also desirable to study how to support disaster roaming between PLMN and SNPN, or between SNPN and SNPN, to support network resilience.

Support discovery of network for localized service

In 6G the localized service may include both connectivity service and beyond connectivity service, e.g. computing service, sensing service, data service, AI service. The operator may need to collect all localized service information support for each network. This information can be used to discover and select a proper network support for the localized service, e.g. when the UE access the network, or when the PLMN needs to select SNPN for beyond connectivity service.

2. WT Scope

No update in the Working task description

No need for sub-WT.

3. Potential KI(s) Scope

It is proposed to study the following key issues:

KI#X: Study any optimization to support localized service access in 6G, this includes the following aspects:

- How to access SNPN via PLMN, or access PLMN via SNPN, or access SNPN via SNPN, without dependency on N3IWF.

- Whether and how to simplify the network when the network start or stop the localized service based on demand, e.g. without impact on the UE

- How to improve the system that allow the operator to provision localized service on demand without any impact on existing services

KI#Y: Study how to enhance the network resilience via localized service access. This includes the following aspects:

- How to support localized service for commercial user when backhaul is not available/limited.

- How to support disaster roaming between PLMN and SNPN, or between SNPN and SNPN.

KI#Z: Study how to discover network for localized service. This includes the following aspects:

- How to manage the localized service information supported by the network.

- How to use the information above to discover and select the network for proper localized service.