SA WG2 Meeting #170	S2-2506777

25 – 29 Aug, 2025, Goteborg , SE

Source:	ZTE, Orange, OPPO

Title:	[WT#1.2, Network Slicing] Discussion and proposal for Network Slicing

Document for:	Discussion

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper discusses the detailed scope for WT#1, specifically for sub working task 1.2 for support and enhancement of network slicing in SA2 6G SID. Based on the discussion, a detailed key issue is proposed.

1. Justifications

1.1	Architectural Motivation/ Gap Analysis

Network slicing is a core feature of 5G system which enables multiple virtualized and isolated logical networks to be built on a shared physical network, meeting differentiated SLA requirements. As firstly introduced in R15 and gradually enhanced with add-one new services in later releases R16/R17/R18/R19, 5G network slicing has become unnecessarily complex and hence costly to deploy from both UE side and NW side.

One dedicated network slice refers to dedicated resources (either logical or physical) allocated at NW side (including both RAN and CN), there are actually no dedicated resources allocated to dedicated network slice at UE side. However, 5G network slicing is designed to be end-to-end, i.e. UE support of network slicing related capabilities is pre-condition for each network slice feature to work. Since it requires complex update at UE side to support network slice related features, and has tight dependency on the HLOS, the value of network slice feature in 5G network is not maximized.

In 6G, UE impacts to support network slicing can be minimized so that operators can quickly and efficiently deploy network slice related features to meet different requirements for verticals to maximize the value of network slicing.

1.2	Technical Impact

Minimize UE impacts to support network slicing.

- On the MM part, the UE needs to support Configured NSSAI, Requested S-NSSAI, Allowed NSSAI, Rejected NSSAI with cause value, Pending NSSAI, partially allowed NSSAI, partially rejected NSSAI, NSSRG, validity timer, validity area, etc. It is too complicated for the UE to support these new parameters. Therefore there is need to study whether these parameters are really needed in both the UE side and network side to support network slicing.

- On the SM part, the UE needs to support URSP rules to associate the PDU session used for the applications traffic with the S-NSSAI. It is recognized that URSP rules has been deployed in some markets. However there are several restrictions to support the URSP rules in all 6G device, e.g. UE capability, HLOS capability, non-standardized OSAppId, etc. Therefore there is need to investigate the network slice selection solution without relying on the URSP rule in the UE side.

- In Rel-19, TEI19_SliceSel (Network Controlled Network Slice Selection) introduced a mechanism to allow Application Function to trigger network slice replacement. Due to restriction of no UE impact, this mechanism still has dependency on the URSP rule. There is need to study whether network slice replacement can be triggered by the application function, without relying on the URSP rule in the UE side.

Simplify network slicing related functionalities at NW side.

For example, in existing 5G system, with gradual improvement over several releases, separate NFs dedicated to different network slicing related functionalities are introduced (e.g. NSSF, NSSAAF and NSACF). For simplification and future-proofing, one single NF dedicated to network slicing management can be defined from DAY 1 of 6G system. In this case, the complex interface interaction between different NFs and complex coupled procedures and possibly introduction of new NFs for new network slicing related functionalities in future can be avoided.

2. WT Scope

No update in the Working task description

No need for sub-WT.

3. Potential KI(s) Scope

It is proposed to study the following key issues:

KI#X: Support network slicing in 6G

Study whether and how to support network slicing with minimizing the complexity on functionalities and procedures at both UE side and NW side. The following aspects should be considered:

- Whether to support network slicing during UE registration.

- Whether and how to support network slicing in the SM signalling, including how the network slice can be selected by the network if the S-NSSAI is not provided by the UE.

- Whether and how to support application triggered network slice replacement without relying on URSP rule.

- Whether and how to simplify the network slice overall architecture, e.g. merging the network slicing related NFs in network side.