3GPP TSG-WG SA2 Meeting #170	S2-2506790

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of -)

Source:	Huawei, HiSilicon

Title:	KI#1,Update Solution#18: Resolving the ENs for UE data collection

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract: For training data collection from the UE to the UE model training entity server, this paper proposed to update solution #18 to resolve some Editor’s notes.

# 1. Introduction/Discussion

This paper focus on the following editor’s notes.

Editor's note:	Whether the DCF is a new NF or the existing one (e.g., NWDAF) is FFS.

NWDAF is designed to manage and provide data in the core network. In the case that 5G core supports UE data collecting over UP and exposure to AF, a dedicated NWDAF is supposed to take the role. We clarify the registration and discovery for the dedicated NWDAF.

Editor's note:	Whether to include initiation, termination, or modification operations in the 'UE Data Transfer request message, or to define separate messages for each operation, is FFS.

'UE Data Transfer' request can be used to request UE to initiate, modify and terminal data collection. We clarify that UP information in 'UE Date Transfer' request is not mandatory since in some case (e.g. termination case), UP information is unnecessary to be transferred to UE.

Editor's note:	Whether UP information includes additional content is FFS.

Similar to how LMT and UE establish User Plane connection in TS 23.273, the IP address or FQDN of the DCF (NWDAF in this solution) from the UP information is enough for UE to build the UP connection.

Editor's note:	Whether data collection parameters include additional content is FFS.

We modify the data collection parameters to better conform to current data subscription request mechanism (e.g. EventExposure services). Also, to avoid confusion, we make a distinction between data collection parameters from NWDAF to UE and data exposure parameters from the AF to NWDAF. Data exposure parameters are provided by the AF, i.e. UE model training entity server, to NWDAF to define the required data. Data exposure parameters may include some parameters which will not be included in data collection parameters to UE, e.g. Target of Data Collection, Filter info for UE selection and aggregation level for NWDAF to handle UE data.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-04.

# * * * * First change * * * *

## 6.18	Solution #18: General Framework for UE Data Collection over UP

### 6.18.1	High level principles

High level principles of the solution are:

-	Data Collection Function (a dedicated NWDAF in this solution) controls and manages standardized UE data collection for UE-side model training. It performs the following functions:

-	Registers to NRF with 'UE Data Exposure Indication' which indicates the NWDAF supports providing 'UE Data Exposure' service to the AF, i.e. the UE model training entity server.

-  Provides the UE with 'UE Data Transfer' request via NAS signalling. The 'UE Data Transfer' request contains UP information and data collection parameters.

-	Collects standardized UE data over the user plane (UP).

-	Receives UE data subscription request from the AF, i.e. UE model training entity server. The UE data subscription request contains data exposure parameters.

-  Verifies and transfers the collected data to the AF, i.e. UE model training entity server.

-	The URSP is used to configure how to select the route for standardized UE data collection, such as determining the associated DNN, S-NSSAI and/or PDU Session type to be used for 'UE Data Collection' traffic.

-	The UE registers with the 'UE Data Collection' capability, allowing the AMF to identify UEs with this capability satisfying filter info (e.g. in a specified Area of Interest (AoI)).

-	The UE establishes a PDU Session to the specified DNN and S-NSSAI that can be used for standardized UE data collection

-	The destination address of the 'UE Data Collection' traffic is the NWDAF address.

-	The NRF is used for the discovery of the NWDAF that provides 'UE Data Exposure' service.

### 6.18.2	Description

This solution is for Key Issue#1 for UE data collection over UP.

The following terminologies are used specifically in the context of this solution.

-	A dedicated NWDAF (e.g. Data-Exposure-NWDAF, DE-NWDAF) is the NF that controls and manages UE data collection from UE for UE-side model training, and exposure UE data to the AF, i.e. the UE model training entity server.

-	UE model training entity server is a trusted AF or untrusted AF that performs UE-side model training using 'UE Data Collection' data.

-	'UE Data Exposure' service is a 5G core network service provided by NWDAF to the AF, i.e. the UE model training entity server.

-  'UE Data Exposure Indication' indicates that the NWDAF supports providing 'UE Data Exposure' service to the AF, i.e. the UE model training entity server. The dedicated NWDAF shall register to NRF with this indication.

-	'UE Data Collection' data refers to data collected by the UE and provided to the AF, i.e. the UE model training entity server. This data is received by the NWDAF and may be verified and aggregated before being delivered to the AF, i.e. the UE model training entity server.

-	'UE Data Transfer' messages are exchanged between the UE and the NWDAF to manage data collection over the User Plane. These include the 'UE Data Transfer' request and 'UE Data Transfer' response messages.

-	The 'UE Data Transfer' request is a message sent by the NWDAF to the UE to initiate, terminate or modify data transfer for UE-side model training. It may include UP information and data collection parameters.

-	The 'UE Data Transfer' response is a message sent by the UE to the NWDAF to acknowledge the corresponding request.

-	The Correlation ID is a unique identifier used for routing 'UE Data Transfer' messages across the UE, AMF and NWDAF. It is used by the AMF to route these messages appropriately. The Correlation ID consists of a NWDAF ID and a connection ID, allowing support for multiple NWDAFs. The correlation ID is allocated by the NWDAF.

-	'UE Data Collection' traffic refers to one or more data packets transmitted from the UE, which are intended to be used for UE-side model training.

-	'UE Data Collection' capability indicates that the UE is capable of collecting and transmitting 'UE Data Collection' traffic. It may be included in the registration request message. 'UE Data Collection' capability may be specified for each Data context ID.

-	UP information is used by the UE to generate and transmit 'UE Data Collection' traffic packets. It includes the IP address or FQDN of the NWDAF.

-	Data collection parameters define the data to be collected and transmitted as 'UE Data Collection' traffic by the UE. The data collection parameters include Data context ID(s), number of data samples and/or time window and optional Sub case info. The Data context ID identifies the use case for which the required data is used for, e.g. Beam management, CSI prediction, Positioning. The Sub case info identifies the specified sub-case for a specific Data Context ID, e.g. when Data context ID is Positioning, the Sub case info can be AI-assisted/direct positioning. Data context ID and optional Sub case info are used to determine what data should be collected.

-	Data exposure parameters define the UE data to be exposed to the AF. The data exposure parameters include Data context ID(s), optional Sub case Info, Target of Data Collection (a list of UEs, or Group IDs, any UE, or TAC(s)), Filter info (e.g. AoI(s)), number of data samples and/or time window.

This solution refers to the user plane connection establishment described in clause 6.18.1 of TS 23.273 [6] and in clause 6.2.1 of TS 24.572 [7].

The following behaviours are described in this Solution.

-	The NWDAF controls and manages UE data collection for UE-side model training.

-	The NWDAF registers to NRF with 'UE Data Exposure Indication' which indicates the NWDAF supports providing 'UE Data Exposure' service to the AF, i.e. the UE model training entity server.

-  The NWDAF is provisioned by the AF with data exposure parameters. The data exposure parameters include Data context ID(s), optional Sub case Info, Target of Data Collection (a list of UEs, or Group IDs, any UE, or TAC(s)), Filter info (e.g. AoI(s)), number of data samples and/or time window.

-	The NWDAF may resolve the list of UEs by querying the AMF to obtain UEs with the 'UE Data Collection' capability satisfying the filter info in data exposure parameters (e.g. located in the specified AoIs).

-	The NWDAF checks user consent, with the assistance of the UDM, for the provision of 'UE data collection' data to UE model training entity server.

-	The NWDAF provides the UE with UP information and data collection parameters over NAS signalling. The UP information includes the NWDAF's address or FQDN for receiving 'UE Data Collection' traffic and the data collection parameters include Data context ID(s), optional Sub case info, number of data samples and/or time window.

-	The UE registers with the 'UE Data Collection' capability and the AMF manages UEs that support this capability.

-	The PCF provides the UE with URSP rules that include route selection descriptors for 'UE Data Collection' traffic towards the NWDAF.

-	The UE establishes a PDU Session that can be used for standardized UE data collection.

-	The destination address of the UE Data Collection packet corresponds to the NWDAF address as provided in the UP information.

-	The UPF forwards the UE Data Collection packet from the UE to the NWDAF.

-	The UDM stores the user's consent regarding UE data collection for UE-side model training.

-	The NWDAF checks the integrity of data from the UE and verifies it and sends a notification to the AF, i.e. the UE model training entity server. If required, the NWDAF may aggregate one or more 'UE Data Collection' packets received from the UE for efficient delivery of notification to the AF.

Editor's note:	How the DCF verifies the received data from the UE is FFS.

The 'UE Data Collection' traffic can be differentiated from the regular UE traffic by the UPF:

-	If the URSP specifies a dedicated DNN or S-NSSAI for 'UE Data Collection' traffic, the traffic is carried in a specific PDU Session designated for 'UE Data Collection'.

-	If the URSP does not specify a dedicated DNN or S-NSSAI, the 'UE Data Collection' traffic can be detected by the UPF using a Packet Detection Rule (PDR) that matches the destination address corresponding to the DCF.

### 6.18.3	Procedures

#### 6.18.3.1	Initiation of UE data collection transfer

Figure 6.18.3.1-1: Initiation of UE Data Collection over the User Plane

1.	The NWDAF registers 'UE Data Exposure Indication' with the NRF as described in clause 4.17.1 of TS 23.502 [3].

2a.	The UE follows the registration procedure defined in clause 4.2.2.2 of TS 23.502 [3], indicating support for the 'UE Data Collection' capability. 'UE Data Collection' capability may be specified for each Data context ID.

2b.	The PCF provides the UE with URSP rules that include route selection descriptors, such as DNN, S-NSSAI and/or PDU session type, for 'UE Data Collection' traffic as described in clause 4.2.4.3 of TS 23.502 [3].

3.	Trusted AF, i.e. a UE model training entity server, discovers the NWDAF that provides 'UE Data Exposure' service as described in clause 4.17.4 of TS 23.502 [3]. For an untrusted AF's request for 'UE Data Exposure' service, the request is authorized by the NEF, which then discovers the NWDAF.

4.	The AF or NEF requests subscription to the 'UE Data Exposure' service, which may include input parameters such as Data context ID(s), optional Sub case Info, Target of Data Collection (a list of UEs, or Group IDs, any UE, or TAC(s)), Filter info (e.g. AoI(s)), number of data samples and/or time window. If NWDAF doesn’t have the required UE data, NWDAF may trigger UE data collection procedure as following steps show. Otherwise, step 5-16 can be skipped, and NWDAF can provide the required UE data immediately.

5.	Optional, if no UE list is provided in step 4, NWDAF determines the UE list for data collection with assistance of the UDM and AMF. The NWDAF may retrieve a list of UEs satisfying the filter info received in step 4 (e.g. retrieve a list of UE located in the specified AoI(s), if one or more AoIs are provided in step 4, retrieve a list of UE corresponding to the TAC(s) from UDM). If the AoIs span multiple AMFs, the NWDAF may retrieve the UE lists with 'UE Data Collection' capability from each corresponding AMF. The NWDAF may provide event ID of "UEs in/out area of interest" with corresponding filters, such as "UE Data Collection Capability", "PEI " or "TAC", "UE CM State", "number of UEs", etc. The AMF may also check other requirements for selected UE, e.g. ensuring that the selected UEs do not have an established PDU session for emergency service.

6.	The NWDAF may check user consent by retrieving subscription data from the UDM.

7.	The NWDAF may subscribe to notifications about changes in user consent for consented UE.

8.	The NWDAF may decide the number of UEs based on the data collection requirements, e.g. the number of samples, Data context ID(s), time window. If multiple AMFs are involved, the NWDAF may decide the number of UEs per AMF based on, e.g. the proportion of AoIs served by the AMF.

The NWDAF sends a 'UE Data Transfer' request with initiation to the UE along with a correlation ID, which is allocated by the NWDAF. The request may include UP information and data collection parameters. The UP information may contain IP address or FQDN of the NWDAF to be used by the UE for data transfer over User Plane. Data collection parameters may include Data context ID(s), optional Sub case Info, number of data samples and/or time window.

If AMF determines that the required UE is not suitable for UE data collection, e.g. the UE has an established PDU session for emergency services, AMF rejects the request and indicates that the UE is unavailable for data collection.

The NWDAF may send a non-release indication to AMF that indicates that AMF shall not release the certain UE during the data collection. The NWDAF decides whether to include the non-release indication based on e.g. the type of data to be collected, data collection parameters, the number of selected UEs.

Editor's note:	Whether and in which cases data collection from UEs in connected state is required would need to be checked with RAN.

9.	The AMF may trigger a Service Request procedure for a UE in CM-IDLE state.

Editor's note:	Whether and how AMF trigger service request when UE is in CM-IDLE for UE data collection is FFS.

Editor's note:	Whether data collection from UEs in idle state is required would need to be checked with RAN

10.	The AMF sends the 'UE Data Transfer' request and the correlation ID via DL NAS Transport message. If the 'UE Data Transfer' request includes a non-release indication, the AMF decides not to initiate the release of the signalling connection until the completion of UE data collection.

11.	The UE may establish a PDU Session if no UP connection is available for 'UE Data Collection'. The UE may use the URSP rules to determine the parameters for PDU Session establishment. The SMF configures Forwarding Action Rules (FARs) for the PDU Session.

12.	The UE starts collecting data according to the request from the NWDAF.

13.	The UE may send 'UE Data Transfer' response and the correlation ID via UL NAS Transport message.

14.	The AMF may forward the 'UE Data Transfer' response to the NWDAF.

15.	The UE sends the collected data over the UP connection to the target NWDAF address. The UPF forwards the data traffic from the UE to the NWDAF based on the configured FARs.

16.	The NWDAF checks the integrity of data received from the UE and verifies that it conforms to the parameters, e.g. Data context ID, specified in the request. If required, the NWDAF may aggregate one or more 'UE Data Collection' packets received from UE.

17.	The NWDAF sends a notification to the AF, i.e. the UE model training entity server, containing the collected data. In the case of an untrusted AF, i.e. a UE model training entity server, the NWDAF provides the collected data to the UE model training entity server (i.e. the untrusted AF) through the NEF.

#### 6.18.3.2	Modification and Termination of UE data collection transfer

Figure 6.18.3.2-1: Termination of UE Data Collection over the User Plane

0.	The AF or NEF may request (un)subscription to the 'UE Data Exposure' service.

0a.	The NWDAF may determine that the 'UE data transfer' needs to be modified or terminated with or without the (un)subscription request from the AF or NEF. It may modify the 'UE Data Transfer' request to change the time window if requested by the AF or NEF. Additionally, the NWDAF may trigger termination of the 'UE Data Transfer' from the UE, such as when the number of received data samples exceeds the originally specified amount or when the revocation of the user's consent is notified by the UDM.

1.	The NWDAF sends a 'UE Data Transfer' request with a modification or termination instruction to the UE, along with a correlation ID, which is allocated by the NWDAF. The request may include data collection parameters to be modified, such as Data context ID(s), number of data samples and/or time window.

2.	The AMF forwards the 'UE Data Transfer' request and the correlation ID to the UE via a DL NAS Transport message. If the UE is in CM-IDLE state, the AMF may trigger a Service Request procedure before sending the DL NAS Transport message.

UE terminates data collection and releases or deletes any data which has been previously logged as part of data collection.

3.	Optional, if the UP connection is no longer required such as UE consent is revoked, the UE may release the PDU Session.

4.	The UE sends a 'UE Data Transfer' response and the correlation ID via an UL NAS Transport message.

5.	The AMF forwards the 'UE Data Transfer' response to the NWDAF.

6.	The NWDAF may send a notification to the AF, i.e. the UE model training entity server, indicating a failure, e.g. due to the revocation of user consent.

### 6.18.4	Impacts on services, entities and interfaces

UE:

-	Handles the sending and receiving of a new payload type related to 'UE Data Collection' in NAS signalling.

-	Gathers the data specified in the 'UE Data Transfer' request.

-	Transfers the gathered data to the NWDAF over the UP.

NWDAF:

-	Registers 'UE Data Exposure Indication' to the NRF.

-	Handles the 'UE Data Collection' subscription request from the NEF or the AF, i.e. the UE model training entity server.

-	Obtains a list of UEs with 'UE Data Collection' capability satisfying the filter info.

-	Checks the user consent of the UE.

-	Handles the sending and receiving of 'UE Data Transfer' messages.

-	Allocates a correlation ID for the UP connection used for 'UE Data Collection'.

-	Handles the reception, verification and processing of 'UE Data Collection' traffic.

-	Notifies the NEF or the AF, i.e. the UE model training entity server, with the collected 'UE Data Collection' data.

UE model training entity server:

-	Subscribes to 'UE Data Collection' data from the NEF or the NWDAF for UE-side model training.

NEF:

-	Supports 'UE Data Collection' exposure of 5GC to un trusted AF, i.e. the UE model training entity server).

NRF:

-	Supports NWDAF registration and discovery service for 'UE Data Exposure' service.

AMF:

-	Supports routing of 'UE Data Transfer' messages using correlation ID.

-	Supports forwarding of 'UE Data Transfer' messages between NAS signalling and SBI.

-	Introduces a new payload type used for 'UE Data Transfer' messages in NAS signalling.

-	Exposes the UE list with UE data collection capability with other filter info.

PCF:

-	Provides the UE with URSP rules that include Route Selection Descriptors for 'UE Data Collection' traffic.

UDM:

-	Supports user consent for the provision of 'UE Data Collection' data to UE model training entity server.

# * * * * End of changes * * * *