3GPP TSG-WG SA2 Meeting #170	S2-2506791

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of S2-250xxxx)

# Title:	[DRAFT] Reply LS on the path information in 5G ProSe multi-hop discovery procedures

# Response to:	LS on the path information in 5G ProSe multi-hop discovery procedures from CT1 (C1-253724/S2-2506119)

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Zhang Fu

Tel. Number:

E-mail Address:	zhang DOT fu1 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR0558 for TS 23.304

1. Overall Description:

SA2 would like to thank CT1 for the LS on the path information in 5G ProSe multi-hop discovery procedures (C1-253724/S2-2506119).

SA2 had discussed the matter and would like to provide the following feedback.

Question 1: For both model A and model B discovery, given that in the path to a 5G ProSe multi-hop layer-3 end UE, the 5G ProSe multi-hop layer-3 end UE's first 5G ProSe layer-3 multi-hop UE-to-UE relay inherently stores the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE, why does the path information need to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE?

Answer by SA2: When receiving a Response message, a UE-to-UE Relay cannot determine the source End UE in the message as the User info of End UE may be protected in the Direct Discovery Set. When the UE-to-UE Relay sends the Response message to the Source End UE, the source layer-2 ID of the 5G ProSe multi-hop layer-3 End UE included in the path information is used for sending the response message. SA2 has discussed multiple solutions and concluded to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 End UE in the path information.

Question 2: In the 5G ProSe UE-to-UE relay discovery announcement message (for model A discovery), it specifies the path information includes the "source layer-2 ID of the discoverer ProSe end UE (yellow highlighted texts as below)". However, the term "discoverer ProSe end UE" pertains to model B discovery. Why does the announcing relay UE indicate this source layer-2 ID of the discoverer ProSe end UE in the path information, given that model A does not involve a "discoverer" end UE? " #### 5.8.6.3 Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of non-IP PDU type The 5G ProSe UE-to-UE Relay Discovery Announcement message (Model A) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and contains the following parameters for multi-hop relaying in addition to as described in clause 5.8.4: - For each user info in the Direct Discovery set: … - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE. … … The 5G ProSe UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC and includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4: … - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer UE. The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following parameters for multi-hop relaying in addition to as described in clause 5.8.4: … - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE. The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the target 5G ProSe End UE matching the RSC includes the following the following parameters for multi-hop relaying in addition to as described in clause 5.8.4: … - Path information: an (ordered) list of User Info ID(s) of Multi-hop UE-to-UE Relay(s) that indicates the transmitted path of the message. Path information also includes source layer-2 ID of discoverer Prose End UE. "

Answer by SA2: SA2 has agreed to remove the word “discoverer” from the path information in Model A.

Question 3: Why are the Layer-2 IDs included in the Multi-hop path info IE, together with the list of the User info IDs? " 4a. A 5G ProSe Intermediate UE-to-Network Relay sends the Solicitation message, it additionally includes its own User Info ID in the message. i.e. the message contains the path information which is an (ordered) list of User Info ID and Layer-2 ID of Relays in the path that has relayed the Solicitation message. … … "

Answer by SA2: As described in TS 23.304 clause 5.8.3.1, the path information in 5G ProSe multi-hop UE-to-network relay discovery is “an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s)”. SA2 has agreed to clarify the path information described in the step 4a in clause 6.3.2.5.3 of TS 23.304.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above information into account and align the specifications accordingly.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13th Oct – 17th Oct, 2025		Wuhan, CN

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US