3GPP TSG-WG SA2 Meeting #170	S2-2506793

Goteborg, SE, 25th Aug – 29th Aug, 2025	(revision of -)

Source:	Huawei, HiSilicon

Title:	KI#1, New Solution: CN triggers UE Data Collection and Transfer

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/ Rel-20

Abstract: For training data collection from the UE to the UMTES for UE-side model training, a new solution is proposed to enable the 5G Core to support UE data collection over User Plane.

# 1. Introduction

This solution addresses Key Issue #1: Transfer of data over UP for UE data collection.

The solution includes the following aspects for training data collection from the UE to the UMTES for UE-side model training: 1) UEs transfer the training data to the 5G Core; 2) 5G Core exposes the training data to the UMTES; 3) 5G NFs assist UE selection for data collection; 4) Termination of data collection and exposure due to, e.g., revocation of user consent, or emergency service.

# 2. Text Proposal

It is proposed to capture the following changes to TR 23.700-04.

# * * * * First change * * * *

## 6.0	Mapping of Solutions to Key Issues and Use Cases

Table 6.0-1: Mapping of Solutions to Key Issues and use cases

|  | Key Issues |  | Use Cases (optional) |  |
| --- | --- | --- | --- | --- |
| Solutions | <Key Issue #1> | <Key Issue #2> | <Use Case #1> | <Use Case #2> |
| #1 | X |  |  |  |
| #2 | X |  |  |  |
| #3 | X |  |  |  |
| #4 | X |  |  |  |
| #5 | X |  |  |  |
| #6 | X |  |  |  |
| #7 | X |  |  |  |
| #8 | X |  |  |  |
| #9 | X |  |  |  |
| #10 | X |  |  |  |
| #11 | X |  |  |  |
| #12 | X |  |  |  |
| #13 | X |  |  |  |
| #14 | X |  |  |  |
| #15 | X |  |  |  |
| #16 | X |  |  |  |
| #17 | X |  |  |  |
| #18 | X |  |  |  |
| #19 | X |  |  |  |
| #20 | X |  |  |  |
| #21 | X |  |  |  |
| #22 |  | X | X | X |
| #23 |  | X | X | X |
| #24 |  | X | X |  |
| #25 |  | X |  | X |
| #26 |  | X |  |  |
| #27 |  | X | X | X |
| #28 |  | X | X |  |
| #29 |  | X | X | X |
| #30 |  | X | X |  |
| #31 |  | X | X | X |
| #X | X |  |  |  |

# * * * * Second change (All new text)* * * *

## 6.X	Solution #X: CN triggers UE Data Collection and Transfer

### 6.X.1	High level principles

The high-level principles of the solution are:

- A dedicated NWDAF with UE data exposure capability is responsible for data collection from UE and data exposure to the UMTES (UE Model Training Entity Server) (optionally) via NEF.

- The UE Data Collection NF (DE-NWDAF in this solution) registers to NRF and can be discovered by NEF or UMTES for UE data Transfer.

- In the case when target UE list is not provided by UMTES, the DE-NWDAF and AMF assist UE selection for UE data collection based on data collection requirements and UE information.

- UE establishes User Plane connection with DE-NWDAF for measurement data reporting.

- 5G Core terminates data collection and exposure due to, e.g., revocation of user consent.

- During the active PDU session for emergency service, the data collection is paused if the UE is already collecting data or the network does not select such a UE for data collection

### 6.X.2	Description

This solution includes the following aspects for training data collection from UE to UMTES for UE-side model training: 1) UEs transfer the training data to the 5G Core; 2) 5G Core exposes the training data to the UMTES; 3) 5G NFs assist UE selection for data collection; 4) 5G Core terminates data collection and exposure due to, e.g., revocation of user consent.

For 5G Core assistance in data transfer from UE to UMTES, it is assumed that a dedicated NWDAF with UE data exposure capability is responsible for data collection from UE and data exposure to the UMTES (optionally) via NEF. In this solution, the NWDAF with UE data exposure capability is called Data-Exposure NWDAF (DE-NWDAF).

The data collection and exposure path is shown in Figure 6.X.2-1:

Figure 6.X.2-1: data collection and exposure framework

To support UE data exposure, the DE-NWDAF registers to the NRF with information listed below in its NF profile:

-	UE Data Exposure Indication.

For UE data subscription/request, the UMTES may provide the inputs parameters listed below:

-  Data Context ID(s): identifies the use case for which the required data is used for, e.g., Beam management, CSI prediction, Positioning. Data context ID is used to determine what data should be collected.

- 	[Optional] Sub case Info: identifies the specified sub-case for required data, e.g. AI-assisted/direct positioning.

-  Time Window: indicates the time interval within which the data should be collected.

-  Number of samples: indicates the number of data should be collected.

-  [Optional] Target of Data Collection: indicates the object(s) for which data collection is requested, in the form of UE IDs or Group IDs (e.g. TAC type allocation code).

-  Filter Info: e.g. Area of Interest.

-	[Optional] Data Requirement Information:

-	Urgency level: indicates the urgency level of data request, e.g. "High"/"Low". This parameter may influence UE selection and DE-NWDAF and AMF’s behaviour when data from a UE in idle mode is needed.

-  Indication to report errors in data collection process e.g., due to lack of UE memory for data collection, or low power in the device, etc.

-  In the case of multiple data collection requests submitted at once, which data should be reported first as the UMTES may prefer to receive data for a certain use case before others.

- 	Reporting Policy: indicates when to forward the UE reported data from the network to the UMTES in the case of data collection for a group of UEs, e.g. report data as collected from individual UE / Report whatever data is collected from the UEs after a certain time period / Report after data has been collected from a certain (e.g., minimum) percentage of the UEs.

Editor’s Note: Whether other parameters are required are FFS.

Optionally the DE-NWDAF may be configured to inform the UE whether collected data can be deleted immediately after reporting the data, or the UE should wait for an acknowledgement from the DE-NWDAF that the data has been correctly received e.g., format, contents, etc, before deletion.

### 6.X.3	Procedures

DE-NWDAF

NRF

NEF

UMTES

1. Nnrf_NFManagement_NFRegister (NWDAF instance ID, ..., UE Data Exposure Indication)

2. Nnef_EventExposure_Subscribe (Data Context ID(s), [O]Sub case Info, Time window, [O] Target of Data Collection, Filter Info…)

3. NEF check UMTES Permission

7. NWDAF trigger UE collecting data

4. Nnef_EventExposure_Subscribe Response (Failure Indication, [O] Failure Reason)

5. Nnrf_NFDiscovery_Request (NWDAF, UE Data Exposure Indication, …)

6. Nnwdaf_DataManagement_Subscribe (Data Context ID(s), [O]Sub case Info, Time window,[O] Target of Data Collection,Filter Info…)

8. Nnwdaf_DataManagement_Subscribe Response (UE Data…)

10. Nnwdaf_DataManagement_Notify (Newly Collected UE Data…)

9. Nnef_EventExposure_Subscribe Response (UE Data)

11. Nnef_EventExposure_Notify (Newly Collected UE Data)

6.X.3.1	Procedure of UE Data Exposure from DE-NWDAF to untrusted UMTES

Figure 6.X.3.1-1: Procedure of UE Data Exposure from DE-NWDAF to untrusted UMTES

1. DE-NWDAF registers its NF profile including UE Data Exposure Indication into NRF;

2. UMTES subscribes for UE data from NEF by invoking Nnef_EventExposure_Subscribe service; UMTES includes Data Context ID, optional Sub case Info, Time Window, Filter information and optional Data Quality Requirement in the data request.

3. NEF checks if the UMTES is allowed to get the requested data.

4. Optionally, If UMTES is not permitted, NEF rejects UMTES via Nnef_EventExposure_Subscribe Response with a failure indication and optional failure reason. In this case, Step 5-9 will be skipped.

5. NEF discovers the DE-NWDAF supporting UE Data Exposure from NRF.

6. NEF subscribes for UE Data from DE-NWDAF by invoking Nnwdaf_DataManagement_Subscribe Service; NEF provides Data Context ID, optional Case Info, Time Window, Filter information, Data Requirement received in Step 2 to DE-NWDAF.

7. Optionally, if DE-NWDAF doesn’t have the required UE data, DE-NWDAF may trigger UE data collection procedure as described in clause 6.X.3.2. Otherwise, DE-NWDAF can provide the data immediately.

8-9. DE-NWDAF provides the UE training data to UMTES via NEF.

10-11. Optionally, DE-NWDAF provides the newly collected UE training data to UMTES via NEF.

#### 6.X.3.2	Procedure of UE Data Collection from UE to DE-NWDAF

Figure 6.X.3.2-1: Procedure of UE Data Collection from UE to DE-NWDAF

1. DE-NWDAF determines to collect UE data based either on data request from UMTES, or its internal trigger.

2. Optionally, if no UE list is provided by UMTES, DE-NWDAF and AMF determine the UE list for data collection as described in clause 6.X.3.3.

3. DE-NWDAF checks user consent for UEs. It may subscribe for user consent information from UDM. DE-NWDAF skips those SUPIs that don’t grant user consent for UE Data Collection. And DE-NWDAF may subscribe to user consent updates in UDM for those SUPIs that grant user consent.

NOTE: Details of user consent check for UE data collection for UE-side model training will be discussed with SA WG3.

4. DE-NWDAF invokes Namf_communication_N1N2MessageTransfer service operation to send the user plane information in a NAS container to AMF to indicate UE to utilize user plane for UE data collection. The user plane information includes the user plane address of the DE-NWDAF. DE-NWDAF may optionally send data collection request in another container to AMF, which includes correlation ID and other parameter, such as Data Context ID(s), Time Window, number of data samples, optional Sub case info etc.

The NWDAF may send a non-release indication to AMF that indicates that AMF shall not release the certain UE during the data collection. The NWDAF decides whether to include the non-release indication based on e.g. the type of data to be collected, data collection parameters, the number of selected UEs.

5. When AMF receives the user plane information and optional data collection request from DE-NWDAF in step 4, AMF sends it to UE via a DOWNLINK NAS TRANSPORT message which includes the user plane info in the N1 container. If urgency level in the data collection request is set as High, the AMF may page UE(s) in idle state for data collection and reporting. For low urgency, the AMF may wait for some time before paging unless the UE transitions to connected mode for other reasons.

If the data collection request includes a non-release indication, the AMF decides not to initiate the release of the signalling connection until the completion of UE data collection.

AMF rejects the request and indicates that the UE is unavailable for data collection if the UE has an established PDU session for emergency services.

6. If there is no established applicable PDU session for user plane data transfer, UE establishes PDU Session for data transfer and user plane connection with DE-NWDAF: UE uses the URSP which includes specific PDU session parameters, e.g. a dedicated DNN and S-NSSAI to establish the PDU Session for user plane data transfer.

7. Optionally, if DE-NWDAF doesn’t sent data collection request in step 4, DE-NWDAF sends data collection request to UE over the User Plane connection with correlation ID and other parameter, such as Data Context ID(s), Time Window, optional Sub case info etc.

8. If the data request from DE-NWDAF indicates that data collection for AI-based positioning case is needed, then UE may contact LMF for positioning label data. In such case, 5GC-MO-LR procedure, as defined in clause 6.2 of TS 23.273[x], is triggered.

9. When the requested data available, it transfers collected data to DE-NWDAF over the User Plane connection with the correlation ID.

Editor’s Note: Whether and how the network (e.g. DE-NWDAF) can be aware whether the data collected from the UE is the same from the one measured by the UE is FFS and maybe coordinated with SA3.

Editor’s Note: The detailed information and parameters exchanged between UE and DE-NWDAF over UP connection are FFS.

#### 6.X.3.3	Procedure of UE Selection for Data Collection

The DE-NWDAF determines the UE list for data collection based on data collection requirements and UE information (e.g., user consent, PEI, UE status).

Figure 6.X.3.3-1: Procedure of UE Selection for Data Collection

- DE-NWDAF determines to collect UE data based either on data request from UMTES or its internal trigger. If no UE list is included in the data request from UMTES or UE data collection is triggered internally, DE-NWDAF determines the UE list for data collection

Option 1): DE-NWDAF gathers UE information and selects UE list.

2a. DE-NWDAF subscribes to AMF to retrieve the list of UEs located in an area of interest and the UE Data Collection Capability for each UE using Namf_EventExposure_Subscribe Request (Target of Event Reporting = "any UE", set of Event ID including "UEs in/out area of interest"). When the AoI involves the service areas of multiple AMFs, DE-NWDAF subscribes to each of them.

3a. The AMF sends Namf_EventExposure_Subscribe Response (list of SUPIs in the area of interest, and optionally for each SUPI, UE Data Collection Capability, PEI, TAC, UE CM state…) to DE-NWDAF.

If AMF determines that the required UE is not suitable for UE data collection, e.g. the UE has an established PDU session for emergency services, AMF rejects the request and indicates that the UE is unavailable for data collection.

4a. DE-NWDAF may decide the number of UEs based on the data collection requirements from UMTES as described in clause 6.X.2. If multiple AMFs are involved, DE-NWDAF may decide the number of UEs per AMF based on, e.g. the proportion of AoIs served by the AMF. Then the DE-NWDAF may decide the UE list per AMF based on TAC, Vendor ID, Chipset ID, UE CM-Connected status, user consent, etc.

Option 2): DE-NWDAF sends filters to AMF and gets filtered UE list from AMF.

2b-3b. DE-NWDAF decides the set of event ID including "UEs in/out area of interest", "UE Data Collection Capability", "PEI " or "TAC", "UE CM State", "number of UEs", etc., and the corresponding filters associated with each Event ID. Then NWDAF invokes Namf_EventExposure_Subscribe Request towards AMF.

4b. AMF responses the filtered UE list to DE-NWDAF. If AMF determines that the required UE is not suitable for UE data collection, e.g. the UE has an established PDU session for emergency services, AMF rejects the request and indicates that the UE is unavailable for data collection.

5. DE-NWDAF may decide UE list per AMF based on the AMF response and user consent, etc.

Editor’s note: The details of AMF impact are FFS.

#### 6.X.3.4	Termination of Data Collection

UE may terminate data collection due to emergency service, internal logic, or request from 5GC due to e.g., use consent revocation. The procedure is illustrated in Figure 6.X.3.4-1 for Data Reporting Termination.

UE

AMF

UPF

DE-NWDAF

UDM

NEF

UMTES

1. UE previously involved / is involving in data collection

2. UE revokes user consent for data collection

3. Nudm_SDM_Subscribe Response (updated user consent)

4. Request termination of data collection over CP or UP

5. UE terminates data collection

6. Termination Request

7. Nnwdaf_DataManagement_Subscribe Response (Failure Indication, [O] Failure Reason)

8. Nnef_EventExposure_Subscribe Response (Failure Indication, [O] Failure Reason)

Figure 6.X.3.4-1: Procedure for Data Reporting Termination due to Revocation of User Consent

1. UE is or has already been involved in data collection and reporting e.g., based on a previous request from the UMTES.

2. UE consent for data collection and reporting is revoked at the UDM.

3. UDM informs the DE-NWDAF about the changes of UE consent.

4. DE-NWDAF determines to terminate the data collection, due to e.g. the user consent of the UE is no longer granted for UE data collection. Then DE-NWDAF sends a request to UE to terminate data collection and reporting. The termination request can be sent over CP (i.e. transferred by AMF) or over the UP connection.

5. UE terminates to terminate data collection due to emergency service, internal logic, or request from 5GC in step 4. UE may releases or deletes any data which has been previously logged as part of data collection.

Editor’s note: Whether UE will also release the existing UP connection with DE-NWDAF is FFS.

6. UE sends termination request to DE-NWDAF over CP via a NAS message or over UP. The termination request may be the response to the termination request received in step 4.

7-8. Optionally, if UMTES subscribes collected data of the certain UE, then DE-NWDAF stops providing collected data from this UE to UMTES. DE-NWDAF may terminate UMTES’s subscription with a failure indication and optional failure reason via NEF.

### 6.X.4	Impacts on services, entities and interfaces

UE:

-	Perform and report new measurement.

-  Establish a new application layer connection with DE-NWDAF.

DE-NWDAF:

-	Register UE Data Exposure Capability in NF profile to NRF.

-  Establish a new application layer connection with UE.

-	Collect and optionally store UE data from UE for UE-side model training.

-	Provide UE data to UMTES directly or via NEF.

-	Decide UE list for training data collection.

UMTES:

-	Subscribe for UE data from NEF for UE-sided model training.

NEF:

-	Support UE data exposure from DE-NWDAF to UMTES.

NRF:

-	Support DE-NWDAF registration and discovery procedure for UE data exposure.

AMF:

-	Support User Plane connection establishment between UE and DE-NWDAF: transfer DE-NWDAF user plane information to UE in new NAS container.

-  Support UE selection for training data collection: provide UE information to DE-NWDAF, or provide filtered UE list to DE-NWDAF.

PCF：

-	Provide UE with URSP rules which includes specific PDU session parameters for user plane connection between UE and DE-NWDAF.

# * * * * End of changes * * * *