3GPP TSG-WG SA2 Meeting #170	S2-2506794

Gothenburg, Sweden, 25th – 29th August, 2025	(revision of -)

Source:	Huawei, HiSilicon

Title:	Discussion on Way Forward for UE Data Collection and Transfer

Document for:	Discussion

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/ Rel-20

Abstract: There are still multiple open questions need to be answered before reaching complete conclusions on end-to-end solutions for KI#1. Thus, discussions and suggestions are proposed.

# 1. Introduction

TR 23.700-04 already document 21 solutions for KI#1: Transfer of data over UP for UE data collection. As a summary, to support the End-to-End procedures, the solutions need to address the following important issues: 1) what is the data collection entity (NF) in 5GC receiving the data from UE. 2) How does UE get the UP information to setup UP connection with 5GC. 3) How UE interacts with 5GC during data transfer. 4) How the UE list is determined for data collection. 5) How UMTES (UE model training entity server) interacts with 5GC to get the UE data.

We do see potential convergence on some of the issues, e.g., How does UE get the UP information to setup UP connection with 5GC. However, for other issues, there are still multiple open questions needs to be answered before reaching complete conclusions on end-to-end solutions.

# 2. Discussions

Question 1: If controllability and visibility are fulfilled?

For Option 2-UP, the UE establishes a User Plane connection with a 5GC entity. To support this functionality, we would need a protocol layer (possibly implemented as an app client) for data transfer to be deployed in the UE. The application client is not accessible by the user. Currently, no study has been conducted regarding the ownership and governance of such an application client.

If the transport layer client is not owned or managed by the operator, it raises concerns regarding the ability of the Core Network (CN) function to exercise full control over the client. Specifically, mechanisms are needed to ensure that the client does not transmit data to servers other than the intended CN Network Function (NF).

Furthermore, if the application client within the UE is capable of accessing data from the Access Stratum (AS) layer—e.g., via AT commands—it is necessary to consider how access control can be enforced to prevent other clients from retrieving the same data through similar means. The potential impact on the fulfilment of controllability shall be assessed in coordination with RAN2, SA3, and CT1.

Regarding data visibility, in LS R2‑2411152, RAN2 has the following feedback

RAN2’s answer:

As stated in the LS sent from RAN, visibility of data content signifies that the MNO will be able to be aware of, access, and comprehend the content of the collected/reported data without the need of SLA. Thus, full visibility allows the MNO to verify/match the data specified/configured to be collected and the data that is being reported.

During the ongoing SA2 study, it seems that there still no common understanding how full visibility can be fullfiled. Some solution argue that as long as the data is standardized and collected via the 5GC data collection entity, full visibility can be ensured. However, it is still unclear if the operator should have the ability to verify the match between the data transferred and the data collected? Thus we see the EN, How visibility is fulfilled is FFS, appears in multiple solutions in the TR.

Proposal 1: Send LS to RAN2, SA3 asking feedback on UE client controllability and data visibility. CC the LS to CT1.

Question 2: Is DCAF based solution in the scope of UP Data Collection (Option 2)?

In RP-242389, there exist the following text regarding option 1b and option 2:

1b.	UE collects training data and transfers it to the server for data collection for UE-side model training (inside the MNO) and then optionally from the server for data collection for UE-side model training to the OTT server (outside the MNO).

2.	UE collects training data and transfers it to Core Network. Core Network transfers the training data to the server for data collection for UE-side model training/OTT server.

In the LS, RAN does not clearly mention that option 1b is EVEX or DCAF-based solution which is defined in TS 26.531. However, it is commonly understood that the server for data collection for UE-side model training (inside the MNO) in option 1b is DCAF in EVEX framework. That’s why only one solution, i.e., solution#13, is proposed to use DCAF as the 5GC NF for receiving data from UE.

Since SA2 does not have complete information on why RAN exclude option 1b for the study, we leave the ENs in solution#13:

Editor's note: It is FFS Whether this solution fulfils the architectural requirements to study UP Data Collection (for Option 2) are as documented in RAN LSs RP-243316 and RP-242389.

Editor's note: Whether this solution addresses option 1b or option 2 needs to be checked with RAN before reaching conclusions.

On the other hand, it is true that in TS 23.288, DCAF is used for collecting data from UE via UE applications. Solution#13 has its advantage by leveraging existing DCAF functionality for UE data collection. Thus, it makes sense to ask RAN if solution#13 should be considered as Option 2 UP and be discussed during conclusion.

Proposal 2: Send LS to RAN2 asking if solution#13 should be considered as Option 2 UP

Question 3: What UE status and mode should be considered for UE selection?

When the data collection for a specific use case needs to be conducted, how to select UEs that can do the radio measurement and transfer the data to 5GC is important. Current there are serval factors proposed as UE selection criteria. For example, area, UE capability, Vendor, etc. However, there are other aspects depending on RAN feedback:

- Whether UE data collection requires to keep the UE in connected mode.

While there are different data collection tasks currently where UE(s) measure and collect different kind of       signals. For example, UE(s) may collect CSI-Reference Signal data for the CSI enhancement task. This may require AMF or RAN keeping UE in connected mode for enough time slot. Otherwise, the selected UE(s) may not collect enough data in the requested time as the signal to be measured are configured only for RRC-CONNECTED UEs. That is why we have the following EN in multiple places in the TR:

Editor's note: Whether data collection from UEs in CM-IDLE mode is required would need to be checked with RAN.

- Whether UE is always available for data collection.

The data collection service provided by UE to the network side is considered a non-essential network service. Therefore, some UEs may render the data collection service unreachable under certain conditions(e.g CM-IDLE) based on internal policies. This means that the UE does not allow the NW to execute the Network Triggered Service Request process when UE is in CM-IDLE, and the NW will not send a paging message to the UE for data collection requests.

- Whether to collect/transfer UE data from UE with other services

UE data collection may require high resource (e.g., battery consumption, memory, etc), thus it can impact other more important services in the UE, especially when the UE is having an emergency PDU Session. Whether the UE data collection should be paused/rejected in such case is not clear. Furthermore, if it needs to be paused/rejected, whether new UEs should be selected, or the remaining UEs need to collect more data, so that we can make sure enough data is collected? These issues require RAN WG’s feedback.

Proposal 3: Send LS to RAN2 asking if data collection from UEs in CM-IDLE mode is required. How to make sure that UEs have collected enough data?

Question 4: What is the role of NG-RAN in UE data collection?

In LS R2-2411152, RAN2 provided feedback about NG-RAN involvement regarding UE-data collection controllability. However, the feedback only provided limited information as following:

SA2 can assume that the NG-RAN is involved in providing radio measurement configuration (if needed) for the UE side data collection at least for the beam management use case, as captured in the following agreement in RAN2-127bis:

Data collection initiation and configuration for data collection is under network control.  FFS how the NW determines whether data collection should be initiated (e.g., via UE requests (UE directly or UE server))

During the study, it is proposed gNBs are responsible for UE selection based on procedures running at gNB for the UE as outlined in Solution #9. Furthermore, it is proposed that gNBs may receive data collection requests from the 5GC and subsequently inform the UEs, thereby reducing NAS signaling between the 5GC and the UE, as described in Solution #10.

In light of these proposals, it is considered important to consult the RAN Working Group regarding their plans for future studies on NG-RAN involvement in the data collection procedures.

Proposal 4: Send LS to RAN2 and RAN3 regarding their plans for future studies on NG-RAN involvement in the data collection procedures.

Question 5: What is the role of UE and UMTES (UE Model Training Entity Server) in UE data collection?

In LS S2‑2411191, SA2 requested RAN2 to assess whether full controllability of data collection procedures would impact normal UE operations. RAN2 responded in LS R2‑2411152, indicating that no such analysis or study had been conducted.

During the ongoing SA2 study, it has been proposed that the UE may reject data collection requests from the network based on its own conditions. Additionally, the UMTES may initiate data collection requests towards the 5GC, including parameters such as UE list, sampling rate, and radio measurement frequencies, time window, etc.

However, it remains unclear which aspects of the data collection procedure are intended to be controlled by the UE and UMTES, and which aspects should be governed by the network. Therefore, it is considered important to seek clarification from RAN on what is the role of UE and UMTES in UE data collection.

Proposal 5: Send LS to RAN2 asking which aspects of the data collection procedure are intended to be controlled by the UE and UMTES, and which aspects should be governed by the network.

Question 6: If current User consent can support UE data collection?

In UE-side model training, the training data will be exposed to UMTES, which is considered a third-party server. The current user consent mechanism is designed for scenarios where model training and inferencing are performed within the network domain. Therefore, it is unclear whether the existing consent framework remains applicable when data is shared with external entities such as UMTES.

Proposal 6: Send an LS to SA3 requesting feedback on whether the current user consent mechanism is applicable in cases where training data is exposed to third-party servers.

Question 7: How to avoid competing with solutions that already specified?

According to RAN study, there are mainly three use cases related to UE data collection and transfer: CSI prediction, AI based positioning (case 1, 2a), and beam management. In Release-19, it is newly specified in TS 23.273 that NWDAF can invoke Nlmf_DataExposure service to get data from LMF for LMF-side model training.

Although in we focus on different positioning cases in R-19 and R-20, however from data collection path perspective for AI-based positioning, it is quite similar for LMF-side model and UE-side model, and it is already supported that UE can setup User Plane connection with LMF for positioning parameters/data transmission, which is easily to be reused.

Currently all the solutions for KI#1 propose an NF (e.g. NWDAF, DCCF, DCF) to collect UE data for all three use cases. No solutions are proposed for using LMF to collect UE data for AI-based positioning. There is risk that solution defined in TS 23.273 maybe bypassed, since NWDAF can get the training data for LMF-side model training from the data collection entity concluded in KI#1.

Proposal 7: Propose to continue the study on the development of a unified solution by integrating existing LMF data exposure service and UP connection procedure to UE as defined in TS 23.273.

# 3. Proposals

Proposal 1: Send LS to RAN2, SA3 asking feedback on UE data collection client controllability and data visibility. CC the LS to CT1.

Proposal 2: Send LS to RAN2 asking if solution#13 should be considered as Option 2 UP.

Proposal 3: Send LS to RAN2 asking if data collection from UEs in CM-IDLE mode is required.

Proposal 4: Send LS to RAN2 and RAN3 regarding their plans for future studies on NG-RAN involvement in the data collection procedures.

Proposal 5: Send LS to RAN2 asking which aspects of the data collection procedure are intended to be controlled by the UE and UMTES, and which aspects should be governed by the network.

Proposal 6: Send an LS to SA3 requesting feedback on whether the current user consent mechanism is applicable in cases where training data is exposed to third-party servers.

Proposal 7: Propose to continue the study on the development of a unified solution by integrating existing LMF data exposure service and UP connection procedure to UE as defined in TS 23.273.