SA WG2 Meeting #170	S2-2506797

25 – 29 Aug, 2025, Goteborg, SE

Source:	Tejas Networks Limited

Title:	[WT#1.2] Study Proposal for 6G Fixed Wireless Access

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This document proposes the scope of the study relating to Fixed Wireless Access in 6G.

# 1. Justifications

The 6G SID, SP-250806 proposes to study the following aspects related to 6G FWA in WT#1.2

WT#1.2. Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g., fixed wireless access, localized service access.

Many developing countries do not have last-mile connectivity over copper or fibre. As a result, deployment of FWA for providing broadband access is gaining popularity in many parts of the developing world. Specifically, FWA usage in rural areas is characterized by low user density and high data rate requirements.

Additionally, in urban areas due to the increased right-of-way costs for fiber deployment, telecom operators are considering FWA as a solution to provide high speed broadband connectivity as a complimentary technology for Fiber To The Home (FTTH).  FWA is also being seen as a replacement for proprietary backhaul solutions like MWA (Microwave Access) and/or UBR (Unlicensed Band Radio). Hence, this paper is proposing to study a simplified FWA architecture for 6G.

Architectural Motivation: 5G architecture was designed to support three key IMT-2020 scenarios viz., enhanced mobile broadband (eMBB), ultra-reliable low latency communication (uRLLC) and massive machine type communication (mMTC). FWA is also characterized by Customer Premise Equipment (CPE) that has low mobility or are stationary. The present 5G network architecture is not optimized to support the FWA traffic characteristics and the CPE profile.

Additionally, the following potential requirements identified by SA1 for FWA in TR 22.870 need to be supported by 6G,

[PR 5.7.1.2-1] The 6G system shall provide optimized network capabilities for FWA (e.g., support stationary devices).

[PR 5.7.1.2-3] The 6G system shall enable the means to provide awareness of user service characteristics (e.g., data rate, latency) to support the RAN and CN in making real-time resource allocation for FWA.

Gap Analysis: The existing FWA architecture is based on 5G and 5G-Advanced specifications and reuses the 5GS architecture designed for eMBB, uRLLC and mMTC to provide services to home users. So, there is a need for considering a simplified network architecture for FWA.

Technical Impact: The paper envisages the following technical impacts:

- Overall System Architecture may be impacted due to impacts on RAN and CN architectural modifications. With expected downlink heavy traffic and even higher data rate than eMBB in 6G, there is a need for simplified RAN and CN architectures specially designed for FWA in 6G to meet the ever-increasing demand of home users. This architectural modification study also supports the ongoing effort to get to the lean architecture for the upcoming 6G System (6GS).

- Interaction between network functions/services may be impacted because of a dedicated network architecture for FWA.

- Interworking or migration from 5G to 6G is need to be studied for FWA because there can be CPEs working on 5G as well as 6G systems.

- Simplifying some of the network functions to make it efficient and cost-effective.

**** First Change ****

# Annex A.X. WT#(1.2) Scope: Architecture for FWA

WT 1.2.X - Support for Fixed Wireless Access in 6G

Study and/or enhance the following aspects for fixed wireless access in 6G including

- Whether and how to come up with network architecture specially designed for FWA in 6G.

- Whether and how to come up with FWA architecture which supports ongoing lean architectural design efforts in 6G.

- Whether and how to enable interworking of 5G and 6G FWA services.

**** Second Change ****

# 5.x1. Key Issue #X (1.2.X): Lean System Architecture for FWA

This key issue (KI) aims to study how to support a lean network architecture for FWA in 6G.

# 5.x2. Key Issue #X (1.2.X): Design of Network Functions/interfaces for FWA

This key issue (KI) aims to study the impact on Core Network Functions and RAN-CN interface for FWA.

# 5.X3. Key Issue #X (1.2.X): FWA Interworking across 5G and 6G

This study aims to study mechanisms to support interworking of 5G and 6G FWA services and NFs.

**** End of Changes ****