SA WG2 Meeting #170	S2-2506799

Goteborg, Sweden, 25-29 August, 2025

Source:	Tejas Networks Limited

Title:	WT#4, ISAC – Study proposal for scope of ISAC in 6G

Document for:	Approval

Agenda Item:	20.6.4

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes study considerations relating to 6G sensing described in WT#4 as defined in SP-250806.

# 1. Justifications

The 6G SID, SP-250806 proposes to study the following aspects related to 6G sensing in WT#4:

WT#4: Study the integration of Sensing and Communication over 3GPP access, considering the sensing modes to be supported and other sources of sensing data.

NOTE 5:	The detailed scope of WT#4 will be determined considering the scope and work of R20 FS_Sensing_ARC.

This paper proposes the scope of 6G ISAC for bridging the gaps from Rel-20 5G sensing and defines potential key issues.

Architectural Motivation:

Based on the decision of the RAN#108, the sensing architecture that will be defined as part for Rel-20 aims to define an architecture for supporting gNB-based monostatic sensing for UAV use-cases. Other sensing modes such as bi-static, multi-static as well as impact of choosing different types of SEs are not being investigated in Rel-20.

Gap Analysis:

The resultant procedures and/or architecture for 5G sensing that from release 20 may require enhancements for supporting other sensing modes and usecases. Moreover, the impact of possible SBA enhancements between the 6G RAN and 6G CN need to be studied.

Additionally, the following potential requirements identified by TR 22.870 need to be supported by 6G.

[PR 7.13.6-1] Subject to operator policies, the 6G System shall provide mechanisms for configuring a sensing operation with a single or multiple sensing modes from all sensing modes supported (e.g., bistatic, monostatic, multistatic).

[PR 7.15.6-1] Subject to regulatory requirements and operator’s policy, 6G network should support suitable means to interact with non-3GPP sensors directly or indirectly for triggering and receiving non-3GPP sensing data.

Editor Note: The wording for interact in this requirement is FFS.

[PR 7.15.6-2] The 6G system shall be able to provide sensing service by enabling multi-sensor fusion-based sensing.

[PR 7.13.6-2] The 6G Network shall provide suitable mechanisms for the exposure of sensing results in a synchronised manner with other types of traffic (e.g., audio, video, haptics) to the sensing service consumer.

Technical Impact:

The WT may have the following impacts on

- Overall system architecture: 6G sensing support requirements may need enhancements to system architecture to support newer usecases.

- Interaction between network functions/services:  The interactions between the NFs may be affected due to the need to support for newer sensing modes.

- Interworking and migration: Interworking between 5G sensing and 6G sensing has to be considered due to the potential migration of 6G RAN-core interface to SBA.

**** First Change ****

# Annex A.X. WT#4.X Scope

WT 4.X - Support for integrated sensing and communication in 6G

Study how 6G supports integrated communication and sensing, including

-	Whether and how to support the various sensing modes e.g, monostatic, bi-static, multi-static or a combination of these modes for various sensing entities.

-	Whether and how to aggregate multi-modal inputs including those from non-3GPP sources to obtain sensing results.

-   Whether and how to incorporate sensing results in a synchronised manner with other types of traffic (e.g., audio, video, haptics) to the sensing service consumer.

-   Whether and how to perform interworking between 5G and 6G sensing.

**** Second Change ****

# 4.X. Key Issue #X.1: Support for sensing modes

This key issue (KI) aims to study how to support various sensing modes e.g, monostatic, bi-static, multi-static or a combination of these modes for various sensing entities.

# 4.X. Key Issue #X.2: Support for sensing data collection and aggregation

This key issue (KI) aims to study how to support data collection and aggregation for 6G sensing.

# 4.X. Key Issue #X.3: Support for sensing information/result exposure

This key issue (KI) aims to study how to support sensing associated information and result exposure for 6G.

# 4.X Key Issue #X.4: Support for interworking between 5G and 6G ISAC procedures

This key issue (KI) aims to study whether and how to support interworking mechanisms between 5G and 6G sensing.

**** End of Changes ****