SA WG2 Meeting #170	S2-2506809

25 – 29 Aug, 2025, Goteborg, SE

Source:	NTT DOCOMO

Title:	[WT#2, Migration and Interworking] Study items for migration and interworking

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes scope and key issues of the “WT#2: Study migration and interworking” of the 6G SID.

# 1. Justifications

The WT#2 in SID on migration and interworking includes the following aspects to study:

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

How to support migration to 6GS

5GC was introduced separately from EPC to enable major architectural changes such as the service-based architecture. When a new core network is introduced, backward compatibility with the previous core network does not necessarily need to be considered. This facilitates the adoption of significant innovations. On the other hand, it increases complexity, as interworking between two core networks must be addressed. From this perspective, it is worth discussing whether the 6G core network should be a separate network from 5GC or an evolution of it. If 6GC is an enhanced version of 5GC, it needs to be identified which components require modification to meet 6GS requirements.

How to support interworking with 5GS

Session continuity may differ depending on the scenario. For example, UE/session context can be directly transferred/shared in some deployments. If interfaces exist between 5G NFs and 6G NFs, the UE/session context could be transferred between 5GS and 6GS. If a combined NF (supporting both 5G and 6G) is introduced, the context can be shared. Also, if dual registration for dual-core networks is supported, pre-registration could enable seamless inter-system mobility. To facilitate cross-company discussions on session continuity, it is necessary to identify possible interworking scenarios, considering factors such as single core or dual core, single registration or dual registration, interface existence between 5G NF and 6G NF (or the introduction of combo NF). These scenarios should be analysed in terms of potential throughput, device complexity, and network infrastructure cost.

Whether and how to support interworking with EPS

The necessity of supporting EPS interworking should be evaluated, especially from voice service continuity and coverage perspectives.

-	Many operators today are using VoLTE/EPS to provide a voice service. If it remains prevalent at 6G launch (Day 1), 6G UEs may need to support EPS to access to a voice service. It may be also necessary where 6G coverage is not available for the UE, although the native 6G or 5G voice is supported in the network. In this case, the support of interworking with EPS becomes necessary.

-	In many operator networks, E-UTRA-NR Dual Connectivity (EN-DC) is still widely used. If 6G RAN coverage is insufficient on Day 1, leveraging EPS coverage with co-existence of 6G may be advantageous.

There are two possible approaches for interworking between EPS and 6GS. One approach is direct interworking, in which the EPS interworks directly with the 6GS. The other approach is indirect interworking, where the EPS interworks with the 5GS, and the 5GS in turn interworks with the 6GS. These interworking options should be analysed in terms of feasibility, complexity, and potential deployment scenarios.

How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS

Session continuity over NTN access particularly in cases of Non-Geosynchronous Orbit (NGSO) satellites such as LEO, MEO, etc, may differ depending on the scenario and features. These include, in case of UE-Satellite-UE communication over multi-satellites, if one Satellite is connected to 5GS/EPS via ground gateway while the other satellite is connected to 6GS. Another scenario involves Handover of an ongoing UE-Satellite-UE communication session from a satellite with onboard 5G RAT to a satellite with onboard 6G RAT. Yet another scenario involves store and forward operation of a satellite initially attached to a 5GS/EPS and later attached to 6GS.

Whether the interworking is achieved via interfaces existing between 5G and 6G NFs on ground or onboard or if it is achieved via combined NF (supporting both 5G and 6G) would need to be analysed. These scenarios should be analysed in terms of potential deployment scenarios, packet loss, device complexity, and network infrastructure cost.

**** First Change (all new) ****

## A.X	WT#2: Migration and interworking

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	Identify possible migration paths from 5GS to 6GS, considering the co-existence of EPS

-	Identify the interfaces that can be defined in backward and/or forward compatible manner, and interfaces where backwards/forwards compatibility cannot be achieved.

-	Whether and how 6GC NFs can communicate with 5GC NFs, and vice versa.

-	Whether and how 6G UE can communicate with 5GC, and vice versa.

-	Whether and how 5G UE can communicate with 6GC, and vice versa.

NOTE 1: Migration from 5G NR to 6G NR is to be concluded in RAN WGs.

-	How to support interworking with 5GS:

-	Whether and how to support inter-system handovers and idle-mode mobility between 6GS and 5GS.

-	Whether the UE maintains single registration state with the core network, or independent registrations in 5GS and 6GS.

-	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility, for example whether combined Session Management entities (SMFs) and/or User Plane functions (UPFs) are used.

-	Whether and	how to support session continuity for IMS voice service between 6GS and 5GS.

-	Whether and how to support fallback to 5GS for IMS voice services, considering the voice service may not be deployed/implemented via native 6G on Day 1.

NOTE 2: Interworking and mobility between 5G NR and 6G NR is to be concluded in RAN WGs.

-	How to support interworking with EPS, considering at least the following aspects:

-	Whether and how to support inter-system handovers and idle-mode mobility between 6GS and EPS, for example whether to support direct mobility from/to 6GS and EPS or mobility is provided via 5GS.

-	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility, for example whether a combined Session Management entities (SMF/PGW-C) and/or User Plane functions (UPF/PGW-U) are used.

-	Whether and how to support fallback to EPS for IMS voice services; for example, whether EPS fallback for IMS voice service is to be provided directly from 6GS, or via inter-system mobility to 5GS, considering the voice service may not be deployed/implemented via native 6G or 5G on Day 1.

NOTE 3: Inclusion of LTE/6G interworking/coexistence aspects are to be concluded in RAN WGs.

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS, considering at least the following aspects:

-	How to support handovers across satellites with onboard RATs of different systems.

-	Whether and how to support interworking during mobility of satellites across different systems.

-	Whether and how to ensure IP address preservation in inter-system handovers for the identified scenarios.

NOTE 4:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 5:  The detailed migration study scope will be coordinated and aligned with RAN.

**** Next Change (all new) ****

## 5.X	Key Issue #X: Interworking with 5GS

This KI will study the following aspects with regards to interworking with 5GS:

-	Whether and how to support inter-system handovers and idle-mode mobility between 6GS and 5GS.

-	Whether the UE maintains single registration state with the core network, or independent registrations in 5GS and 6GS.

-	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility, for example whether combined Session Management entities (SMFs) and/or User Plane functions (UPFs) are used.

-	Whether and	how to support session continuity for IMS voice service between 6GS and 5GS.

-	Whether and how to support fallback to 5GS for IMS voice services, considering the voice service may not be deployed/implemented via native 6G on Day 1.

NOTE 1: Interworking and mobility between 5G NR and 6G NR is to be concluded in RAN WGs.

**** Next Change (all new) ****

## 5.X	Key Issue #X: Interworking with EPS

This KI will study the following aspects with regards to interworking with EPS:

- 	Whether and how to support inter-system handovers and idle-mode mobility between 6GS and EPS, for example whether to support direct mobility from/to 6GS and EPS or mobility is provided via 5GS.

-	Whether and how to ensure IP address preservation in inter-system handovers and idle-mode mobility, for example whether a combined Session Management entities (SMF/PGW-C) and/or User Plane functions (UPF/PGW-U) are used.

-	Whether and how to support fallback to EPS for IMS voice services; for example, whether EPS fallback for IMS voice service is to be provided directly from 6GS, or via inter-system mobility to 5GS, considering the voice service may not be deployed/implemented via native 6G or 5G on Day 1.

NOTE 1: Inclusion of LTE/6G interworking/coexistence aspects are to be concluded in RAN WGs.

**** Next Change (all new) ****

## 5.X	Key Issue #X: Migration to 6GS

This KI will study the following aspects with regards to migration to 6GS:

-	Identify possible migration paths from 5GS to 6GS, considering the co-existence of EPS

-	Identify the interfaces that can be defined in backward and/or forward compatible manner, and interfaces where backwards/forwards compatibility cannot be achieved.

-	Whether and how 6GC NFs can communicate with 5GC NFs, and vice versa.

-	Whether and how 6G UE can communicate with 5GC, and vice versa.

-	Whether and how 5G UE can communicate with 6GC, and vice versa.

NOTE 1: Migration from 5G NR to 6G NR is to be concluded in RAN WGs.

**** Next Change (all new) ****

## 5.X	Key Issue #X: Interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS

This KI will study the following aspects with regards to interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS:

-	How to support handovers across satellites with onboard RATs of different systems.

-	Whether and how to support interworking during mobility of satellites across different systems.

-	Whether and how to ensure IP address preservation in inter-system handovers for the identified scenarios.

**** End of Changes ****