3GPP TSG-SA2 Meeting #170	S2-2506812

Gothenburg, Sweden, August 25th-29th, 2025	(Revision of S2-2504898)

Source:	China Mobile

Title:	New Solution for KI#3 Discovery and selection of sensing entities

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Discovery and selection of Sensing functions, sensing entities for FS_Sensing_ARC are proposed for approval.

1. Introduction

This pCR proposes the solution for Key issue#3 Discovery and selection of sensing entities

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

- 3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[x]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X: Discovery and selection of Sensing functions, sensing entities

6.X.1	Description

This solution aims to address key issue#3 on how to perform SCF (Sensing Control Function), SPF(Sensing Processing Function) and Sensing Entities (sensing transmitters and receivers) discovery and selection. Generally, two phases need to be considered during the whole sensing service procedure, one is how NEF selects a SCF and the other one is how SCF selects SPF(s) and Sensing Entitie(s).

For the first phase, it is the NEF to perform SCF discovery and selection based on interaction with NRF or local configuration. The NEF should determine which SCF to serve a sensing request from AF based on serving area of the SCF and Target Sensing Service Area requested by the AF and ensure that the selected SCF can serve the target area of the required sensing service.

-  Before this, Sensing Entities should associate themselves and report their capability, location or serving area information to a SCF. SCF may aggregate the information from Sensing Entities and generate the SCF serving area and capability information, e.g., Supported Sensing Service area, supported sensing service type/scenario (UAV detection or transport traffic monitoring, etc.,), service quality level, etc., and may register itself to the NRF.

-  After the NEF authorizes the sensing service request from the AF, the NEF performs the discovery of SCF as defined in TS 23.502 [x] clause 4.17.4. The NEF determines the SCF instance(s) based on sensing service requirement (e.g., Target Sensing Service Area, service type, service quality requirement, etc.,) from AF and the information received from NRF like supported sensing service area and sensing capability, etc.

For the second phase, it is the selected SCF to selects SPF(s) and Sensing Entitie(s).

- Before this, SPF(s) should associate themselves and reports their serving area and capability, e.g., supported sensing service type/scenario (UAV detection or transport traffic monitoring, etc.,), to a SCF.

- After receiving the sensing service requirement from AF, the SCF selects a or multiple Sensing Procession Function(s) based on their sensing service area, sensing capability, etc., and sensing service requirement (e.g., Target Sensing Service Area, service type, service level requirement, etc.,) from AF, which ensures selected Sensing Procession Function(s) can serve the sensing service.​

- The SCF should also select Sensing Entities based on their sensing service area, sensing capability, etc., and sensing service requirement (e.g., Target Sensing Service Area, service type, service quality requirement, etc.,) from AF, which ensure selected Sensing Entities can serve the sensing service.​

In general, the factors for SCF Selection (performed by NEF), SPF and Sensing Entity (performed by SCF) selection are listed as follows:

-	Supported Sensing Service area.

- 	Sensing Capabilities (e.g., supported service type).

-	Sensing service requirement (e.g., target Sensing Service Area, service type, service quality requirement, etc.,) from AF.

6.X.2	Procedures

Figure 6.X.2: Discovery and selection of Sensing functions, sensing entities

1  Sensing Entities associate themselves and report or register their capability, location or serving area information to the SCF directly or via AMF.

2  SCF aggregates the information from the connected Sensing Entities and generate the SCF serving area and capability information, e.g., Supported Sensing Service area, supported sensing service type/scenario (UAV detection or transport traffic monitoring, etc.,), service quality level, etc., and registers itself to the NRF .

3  The AF requests a sensing service request, the request message contains sensing service requirement parameters(e.g. Sensing service type, Target Sensing Service Area, one-time report/periodical report/event-triggered).

4  The NEF performs SCF discovery and selection based on interaction with NRF or local configuration. The NEF should determine which SCF to serve a sensing request from AF based on serving area of the SCF and Target Sensing Service Area requested by the AF and ensure that the selected SCF can serve the target area of the required sensing service.

5  The NEF sends a sensing service request to the selected SCF, the request message contains sensing service requirement parameters(e.g. Sensing service type, Target Sensing Service Area, one-time report/periodical report/event-triggered).

6  The SCF should select Sensing Entities, SPF and the path of sensing result exposure (SPF-to-AF directly or SPF-SCF/NEF-AF) based on the sensing service area, sensing capability, sensing data type and sensing service requirement (e.g., Target Sensing Service Area, service type, service quality requirement, etc.,) from step5.

7  The SCF coordinates the sensing operation with the selected SPF and sensing entities.

8  Sensing entities provide the collected sensing data to the SPF (via data connection).The information to set up the data connection may be included in step 7 or may be configured after sensing entity registered its capability.

9  The SPF calculates the sensing result based on the collected sensing data.

10 The SPF provides sensing result to AF directly or via SCF and NEF as selected in step6.

6.X.3	Impacts on services, entities and interfaces

NEF:

-	Support SCF discovery and selection

NRF:

-	Support SCF registration.

Sensing Control Function:

-	Support association/registration with Sensing Entities and SPF

-	Support Sensing Entities/SPF discovery and selection,

-	Support registration to NRF.

Sensing Processing Function:

-	Support association/registration with SPF

Sensing Entities:

-	Support association/registration with SCF.

# * * * * End of changes * * * *