3GPP TSG-SA2 Meeting #170	S2-2506813

Goteborg, SE, Aug 25th – 29th 2025	(revision of S2-2504899)

Source:	China Mobile

Title:	KI#4, New Sol: Sensing Data Collection and Transportation

Document for:	Approval

Agenda Item:	20.6.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Sensing data and the associated information collection and transport for FS_Sensing_ARC are proposed.

1. Introduction

This pCR proposes the solution of Sensing Data Collection and Transportation for Key issue#4.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  |  | X |  |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X:Sensing Data Collection and Transportation

6.X.1	Description

This solution aims to address key issue#4 on how to collect sensing data for sensing result generation.

-	A high-level procedure for sensing data collection is proposed with assumptions on the split of the sensing management and SPFality into two different sensing functions.

-	Generally, the SCF requests Sensing Entity for sensing data and the associated information for sensing data collection and transportation. Meanwhile, SCF could get the the associated information for sensing data collection and transportation from Sensing Procession Function and align those information with selected Sensing Entity. Multiple sensing data can be collected by SPF, the SCF determines the type of the Sensing data based on the sensing task requested by the AF.

6.X.2	Procedure for Sensing Data and the Associated Information Collection and Transport

Figure 6.X.2: Sensing Data and the Associated Information Collection and Transport procedure

1.The AF sends Nnef_SensingExposure_Subscribe request message to the NEF, the request message contains sensing service requirement parameters(e.g. Sensing service type, Target Sensing Service Area, one-time report/periodical report/event-triggered).

2.The NEF sends the Nsf_SensingSubscription Request message to the selected SCF. The request message contains the sensing service requirement parameters.

NOTE 1:	How to select SCF should be addressed in KI3.

3.Then the SCF sends a Sensing Task Initiation Request to the selected SPF and SPF replies the data path information (e.g. IP,TEID ) for sensing data transportation.

NOTE 2:	To ensure that the selected SPF suitable for the required sensing service, the SCF determines based on the supported service area of the SPF and Target Sensing Service Area requested by the AF..

4.The SCF sends a StartSenseTask Request to the Sensing Entity, the request message contains sensing service requirement parameters (e.g. sensing data type, one-time report/periodical report/event-triggered).

5.Upon receiving the sensing service request message from the SCF, the Sensing Entity create sensing data path for 3GPP Sensing Data transport with the data path information (e.g. IP,TEID ) received in Step 3.

6.The Sensing Entity/Entities performs the sensing measurement and obtains 3GPP sensing data based on the sensing service requirement. The 3GPP Sensing data may include the following type:

-  I/Q streams of the raw signal,

-  Delay spread spectrum, Doppler spectrum,

-  Point cloud information of the sensing target,

-  Sensing result: Distance, velocity, trajectory, and associated information of the target.

7. The Sensing Entity transmits the 3GPP sensing data to the SPF.

8. After receiving the 3GPP sensing data, the SPF performs the generation or combination of the Sensing result.

9. The SPF Reports Sensing result to SCF.

10. The SCF sends the Sensing result to the NEF via Nsf_Sensing_Notify.

11. The NEF then provides the Sensing result to the AF using Nnef_SensingExposure_Notify.

6.X.3	Impacts on services, entities and interfaces

NEF:

-  Support receive sensing service requirements and expose sensing result.

SPF:

-  Support Sensing Data Collection and Transport.

-  Support Sensing Result generation based on the 3GPP Sensing Data.

SCF:

-  Support Sensing Result Reception and exposure.

# * * * * End of changes * * * *