SA WG2 Meeting #170	S2-2506815

25 – 29 Aug, 2025, Goteborg , SE

Source:	Samsung

Title:	[WT#1.2, Network Slicing] Support of Network Slicing in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Scoping and justifications for Work task 1.2 network slicing

1. Justifications

WT#1.2 of the approved 6G SID in SP-250806 includes

  - Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Network slicing is one of the most promising technologies in mobile networks that enable a new business opportunity. While network slicing features have been evolved continuously in 5G, there exist inefficiency that needs to be improved in 6G. For example, in 5G network slicing, support from UE is required for most of the slicing features and such dependency on UE need to be investigated in 6G.

Architectural Motivation: Complexity of UE configuration

- Dependencies on UE: In the network, the slice ID (i.e., S-NSSAI) is used to identify and manage the resources of the network that is used to serve a specific type of communication service. In the UE, the slice ID is also used to identify the type of communication services (e.g., Real-time streaming, Low latency, etc.) that it is allowed to use. This caused dependencies between the network and UE for network slicing and consequently, the support from UE is required for the evolutions or introduction of a network slicing feature.

- Many alternative options: Some of the network slicing features have defined too many options. It would be valuable to investigate whether all the options are actually needed or not.

Gap Analysis 1: 5G network slicing design is limited to achieve simplified network slicing

Technical Impact

- Any architectural impact can be investigated for simplification of network slicing

- UE, RAN, NFs can be impacted

**** First Change ****

Annex A.X. WT#1.2.X Network Slicing

WT#1.2.X Study how to support network slicing for 6G

- Identify a set of network slicing features to be re-used, enhanced, or re-designed for simplification (e.g., less dependency on UE, reducing options)

- How to support a introduction of additional network slicing features without impacting UE

**** Second Change ****

5.X. Key Issue #X: Simplified Network Slicing for 6G

The key issue will study support for network slicing for 6G:

- Identify a set of network slicing features to be re-used, enhanced, or re-designed for simplification (e.g., less dependency on UE, less options)

- How to support a introduction of additional network slicing features without impacting UE

**** End of Changes ****