3GPP TSG-SA2 Meeting #170	S2-2506818

Goteborg, SE, Aug 25th – 29th 2025

Source:	China Mobile

Title:	KI#2&3, New Sol: Sensing service authorization and revocation based on network condition analysis

Document for:	Approval

Agenda Item:	20.6.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: The Sensing service authorization and revocation based on network condition analysis for FS_Sensing_ARC are proposed for approval.

1. Introduction

This pCR proposes the solution of Sensing service authorization and revocation based on network condition analysis for Key issue#2&3.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X | X |  |  |  |

# * * * * Second change (All Text New) * * * *

6.X	Solution #X:Sensing service authorization and revocation based on network condition analysis

6.X.1	Description

This solution aims to address key issue#2&3.

During the execution of sensing tasks, the scenarios such as unsatisfactory or poor sensing performance may arise due to factors like the mobility of service consumers, sensing entities, or sensing targets, as well as quality of sensing data, malfunctions of sensing devices, and network status. In such cases, how to trigger changes in sensing services, re-select sensing entities, and select appropriate sensing entities is a key issue for ensuring the continuous and high-quality execution of sensing tasks.

It is feasible to consider enabling core network elements to dynamically adjust sensing services and introduce sensing service analysis capabilities. Based on network status and sensing performance indicators, the sensing service analysis network element (like NWDAF) can conduct analysis to dynamically trigger changes in sensing services or re-selection of sensing entities.

6.X.2	Procedure for Sensing service authorization and revocation based on network condition analysis

Figure 6.X.2: Sensing service authorization and revocation based on network condition analysis procedure

1  The AF initiates a sensing service request, which may carry parameters (such as afTaskId, sensing range, sensing accuracy, sensing data type, and sensing data reporting frequency).

2  Upon receiving the sensing task request, NEF initiates a query request for SCF to NRF, carrying information related to the service sensing area. NRF queries the list of SCFs that meet the service sensing area requirement based on the service area of SCF. NEF then initiates a sensing network status analysis request at the sensing network function granularity to NWDAF with the recommended SCFs.

3  NWDAF analyzes the sensing service network status of the target SCFs and provides a recommendation for the optimal SCF.

4  NWDAF initiates a sensing service network status analysis response to NEF, carrying the recommendation for the optimal SCF.

5  NEF sends a request to trigger the sensing task to the optimal SCF, and the sensing request may carry parameters (such as afTaskId, sensing range, sensing accuracy, sensing data type, and sensing data reporting frequency).

6  SCF selects SPF and sensing entities in accordance with the requirements of the sensing service and performs sensing operations.

7  SCF sends a request to NWDAF for analyzing the performance evaluation of sensing results and sensing network status at the sensing entities granularity.

8  NWDAF conducts analysis on the performance evaluation of sensing results and sensing network status at the sensing entities granularity.

9  If the performance at the sensing entity granularity or the network status does not meet the sensing requirements, NWDAF triggers a request for re-selection of sensing entities to SCF, including the reason identifier and information of the sensing entities that do not meet the service requirements.

10 SCF performs the re-selection operation of sensing devices based on the reason identifier for non-compliance.

6.X.3	Impacts on services, entities and interfaces

NEF:

-	Select or re-select SCF based on service conditions.

-	Revoke ongoing sensing operation when the check of condition compliance fails.

NWDAF:

-  Support sensing network or sensing entity status analysis.

SCF:

-	Select or reselect sensing entities based on service conditions.

-	Revoke ongoing sensing operation when the check of condition compliance fails.

# * * * * End of changes * * * *