3GPP TSG-WG SA2 Meeting #170	S2-2506821

Gothenburg, Sweden, August 25 - 29, 2025	(revision of S2-250xxxx)

Source:	NTT DOCOMO

Title:	Discussion on Delayed NAS Message

Document for:	Agreement

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This Discussion Paper discusses the delayed NAS message

1. Introduction

RAN2 sent an LS to CT1 and Cc SA2 (R2-2504697: “LS on delayed A-IoT D2R NAS messages”.

The use case is about:

RAN2 understand that the device may, for example, receive a write command message from the reader, pass the command message to NAS layer, and when the scheduled D2R occasion for the response arrives, transmit the MAC response with an empty SDU if the NAS response has not yet been delivered to MAC as indicated in the agreement above.

[Observation-1] When an AIoT device receives a NAS command request message (e.g., write), the NAS command response message may not be ready when the scheduled D2R occasion arrives.

In the LS, RAN2 mentioned an agreement reached:

- The device is expected to send a MAC response to the reader in the D2R occasion.   The MAC response contains the NAS message if available at the D2R occasion.   If there is no NAS message available to transmit at the D2R occasion then the response contains MAC with 0 SDU and padding as needed.

[Observation-2] According to this agreement, the AIoT device may transmit a MAC response with 0 SDU.

In 3GPP TS 38.391 “more data indication” (MDI) is mentioned, this indicator is used to support segmentation. I.e., in D2R message the “more data indication” can be used to indicate to the reader more data is pending and more D2R resources are required to send all data. This helps the reader to schedule more resources.

In “Draft_MAC running CR_v33_Rappv5_cl” that will be submitted to RAN2#131 and can be found at here. There are more details on where the MDI is placed.

[Observation-3] The MAC (0 SDU) may comprise the MDI and could be used to indicate pending data similar to when segmentation occurs. From a reader point of view there is no significant difference, the reader may continue to allocate D2R resources until device indicates no more pending data by changing the MDI value.

Example procedure below:

[Observation-4] There are currently potential solutions without SA2 involvement.

[Proposal 1] Since this issue can be resolved purely on RAN side, it is proposed to send an LS to RAN2 indicating SA2 considers this issue to be handled outside of SA2. It is proposed to approve the reply LS (S2-2506819).