SA WG2 Meeting #170	S2-2506822

25 – 29 Aug, 2025, Goteborg , SE

Source:	CEWiT

Title:	[WT#1.1] UE-Core Network interaction in 6G

Document for:	Approval

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides detailed scope for WT # 1.1 for supporting  the control signalling for 6G for connectivity services and/or beyond connectivity services

# 1. Justifications

As part of the 6G SID approved during TSG SA #108, the following description was provided for WT #1.1, which focuses on the control signalling for 6G for connectivity services and/or beyond connectivity services to be supported from 6G Day-1:

WT# 1.1. Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a. Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b. Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c. Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

This paper proposes updates to WT #1.1 description of the approved SID to define the scope for the development of a new NAS functionality and generic mechanisms for UE-Core Network interactions. Furthermore, it outlines corresponding Key Issues to be captured in TR 23.801.

Architectural Motivation:

6G system is expected to provide more novel services for connectivity and beyond connectivity features. In order to incorporate the new 6G features, the 6G system is expected to evolve architecturally and protocol vise to support both traditional connectivity services and beyond connectivity services. Architectural transformation is essential to ensure that the network can efficiently support both the traditional connectivity services such as mobility management, session management, and quality of service enforcement as well as the emerging service paradigms enabled by 6G. On the protocol level, enhancements are required across control plane signalling. Protocols such as NAS must be extended to support more context-aware, secured and service-oriented communication between the UE and network. Essential aspects of control plane signalling in 5G have to be maintained in 6G.

Gap Analysis:

- Support of beyond connectivity services.

- 5G NAS basically supports traditional connectivity services, such as registration, authentication, mobility management, and session management to support data and voice services,

- 6G NAS is expected to expand its scope to include novel “beyond connectivity” services.

- 5G NAS signalling procedures are largely predefined limiting the ability to dynamically adapt to emerging new use cases. The static nature makes it more difficult to support emerging use cases in 6G.

- Multiple NAS Variants

- Today, NAS is basically built for mobile phones as a device.

- Different types of UEs having varying requirements , capabilities and service needs, maintain multiple NAS variants. Especially for devices ranging from high-end smart phones to low power IoT devices.

- 5G lacks the identification of basic set of NAS functionality to be supported in all kind of devices.

- Signalling Overhead

- As the service complexity grows, 5G NAS control signalling can lead to increased overhead.

**** First Change (all text are new)****

# Annex A.X. WT#1.1 Scope for NAS and UE-Core network interaction in 6G

WT#5	Study the scope for the development of a new NAS functionality and generic mechanisms for UE-Core Network interactions in 6GS, which include:

- Preserve the access-agnostic NAS model.

- Independent NAS modules for newly introduced NAS functionalities like connectivity and beyond connectivity services.

- Identifying the minimal set of NAS functionalities suitable for all devices.

- Efficient and secured NAS message transmission  reducing signalling overhead.

**** Next Change (all text are new) ****

# 5.x1. Key Issue #1.1.x1: How to enhance NAS for supporting new 6G services

This Key Issue aims to study on enhancements to NAS for newly added functionalities supporting connectivity and beyond connectivity services in 6G without impacting the existing NAS functionalities.

# 5.x2. Key Issue #1.1.x2:  Scope for identification of device- agnostic minimal set of NAS functionalities

This Key Issue aims to identify a minimal set of NAS functionalities suitable for all devices, including those with limited capabilities like low- powered IoT devices.

# 5.x3. Key Issue #1.1.x3: Efficient and secured NAS message transmission

This Key Issue aims to study how efficiently and securely the NAS messages can be transmitted, with reduced complexity and signalling overhead.

NOTE: The detailed study to a secured transmission of NAS messages will be coordinated and aligned with SA3.

**** End of Changes ****