3GPP SA WG2 Meeting #170	S2-2506824

25th – 29th August 2025, Goteborg, SE

Source:	China Telecom

Title:	[WT#1.2, QoS framework] Discussion and proposal for new QoS in 6GS

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes to study the new QoS in 6GS to better support the 6G services, including: how to enhance the connectivity-oriented QoS mechanism, how to provide an integrated QoS mechanism for emerging connectivity/beyond connectivity-oriented services and how to support AI-based QoS mechanism.

1. Justifications

1.1 Architectural Motivation and Gap Analysis

With the introduction of new use cases and requirements in 3GPP R20 TR 22.870, some problems and gaps are identified in the 5G QoS framework:

- For connectivity services:

Only connectivity-oriented QoS parameters are included in 5G, such as bandwidth (GFBR, MFBR, Session-AMBR, UE-AMBR and UE-Slice-MBR), latency (PDB, PSDB), packet loss (PER, PSER), etc. However, more strict performance indicators are required for the 6G connectivity services. For example, as described in clause 9.6 of 3GPP R20 TR 22.870, ‘tighter synchronization between different media components is required’, not only for multi-modal data streams in an application (e.g., holographic telepresence in healthcare), but also for the multiple application media (e.g., 3D remotely controlled repairs or surgery). Both the packet jitter and packet order will affect the synchronization performance. For another example, it is necessary to ensure the MTP (motion-to-photon) latency and audio-video synchronization of XR services to provide immersive experience and avoid dizziness for XR users.

To meet the above new requirements, extended connectivity-oriented QoS parameters related to round-trip delay, jitter, and packet order have been proposed in ‘ITU-T Recommendation Q.5013 Signalling requirements and protocol procedures for two-way QoS mechanism between access networks and core networks in IMT-2020 network and beyond’. For example, round-trip packet delay budget (RPDB) is defined as the upper bound of the sum of end-to-end delay from the source host to destination host and from the destination host to the source host; Maximum cycle delay variation (MCDV) is defined as the upper bound of the difference of end-to-end delay between adjacent cycles among the lifecycle of a service; Maximum packet reordered ratio (MPRR) is defined as the upper bound of the ratio of the total reordered packets to the total successful transmitted packets in a packet flow.

Proposal 1: For connectivity services, it is proposed to extend connectivity-oriented QoS parameters to support end-to-end reliable data transmission.

- For beyond connectivity services:

6G network is evolving deeper into beyond connectivity services with new QoS requirements. For example, as described in clause 6.33 of 3GPP R20 TR 22.870, ‘the 6G network can leverage the overall E2E latency (i.e. communication delay + computing delay) budget of an application service by adjusting communication and computing resource on demand’. Therefore, besides using the existing bandwidth and PDB to constrain communication resources, it may also be necessary to introduce computing resource budget and computing delay budget to constrain computing resources to better support the computing services and artificial intelligence (AI) services. The collaborative decision among these multi-dimension parameters and the integrated scheduling of these various resources are also required. Moreover, for 6G sensing service, the delivery of sensing measurement data to the 6G network, the processing of sensing measurement data to obtain the 6G sensing result, also put forward higher QoS requirements to the 6G network.

Proposal 2: For beyond connectivity services, it is proposed to provide an integrated QoS mechanism to meet the emerging service requirements.

- One-way QoS mechanism:

Existing QoS mechanism is a one-way mechanism controlled by the core network (CN). Specifically, CN is responsible for the configuration/adjustment of QoS policies and parameters without considering the status of RAN and UE, when CN sends the QoS policies to the radio access network (RAN) and user equipment (UE), the RAN and UE can accept or reject the policy but cannot negotiate with the CN. For example, the notification control defined in clause 5.7.2.4 of TS 23.501 is merely a template-based feedback mechanism of RAN without sufficient flexibility, in this one-way QoS mechanism, RAN and UE are excluded from the QoS policy decision, which may lead to the mismatch between the QoS policy and the dynamically changing network environment. Therefore, the efficiency of data transmission in the user plane and the QoE could not be guaranteed.

In addition, it is described in clause 5.8.3 of TR 22.870 that, ‘subject to operator’s policy, regulatory requirements and user consent, the 5G network shall support subscription policies that include alternative (i.e. degraded) service performance (e.g. QoS parameters, maximum bitrate) of services with QoS criteria for energy saving reasons’. Therefore, as we progress towards 6G, some QoS decision assistant information e.g., the QoS information related to the service characteristics or air interface characteristics, can be sent to CN by UE or RAN, to help negotiate the specific degraded QoS parameters.

Proposal 3: Two-way negotiable QoS mechanism is required to improve the reliability and flexibility of QoS control.

- QoS mechanism based on centralized-AI :

For the QoS monitoring, 5G NWDAF is a centralized AI to obtain QoS monitoring report from the UPF. It conducts statistics and predictions only and has no knowledge of the logic of the network functions (NFs) or the service characteristics. If the NFs with QoS awareness/control/enforcement/monitoring capability (e.g., PCF, SMF, UPF and AF) can make decisions based on their native AI capability, such as the embedded intelligent models or algorithms within these NFs, the QoS decision may be more suitable and reliable.

For the industrial internet scenario, the traffic characteristics of multiple industrial terminals are different and the QoS policies for these industrial terminals need to be manually configured one by one. Once the connection is lost, the policy reconfiguration will consume a large amount of manpower and cost. To improve the efficiency, AI-based QoS mechanism can be used to achieve automated traffic awareness, classification, QoS policy decision and configuration.

Proposal 4: It is required to provide a more comprehensive AI-based QoS mechanism, with the collaboration between native-AI and centralized-AI, to improve the reliability and efficiency in the entire QoS lifecycle.

1.2 Technical impact

The potential technical impacts may include the following:

- Interaction between network functions: Information exchange may be required among NFs with QoS awareness/control/enforcement/monitoring capability such as AF, PCF, SMF, UPF, etc. For example, the end-to-end delay, the computing delay and the communication delay will jointly affect the computing service performance. To determine the QoS policy for this services, the end-to-end delay or the computing delay might be sent to PCF from AF or other computing management entities.

- WT dependency: Since it is required to provide the optimal QoS experience for the beyond connectivity services, this WT may have dependency with WT#3 AI in 6G, WT#4 sensing and WT#6 computing service. And the basic architecture design for AI in WT#3 should also consider the native AI within NFs to support the proposed AI-based QoS mechanism.

**** First Change ****

Annex A.X. WT#X Scope

The present document specifies the new QoS in 6GS to better support the 6G services, including:

- Study how to enhance the connectivity-oriented QoS.

- Study how to provide an integrated QoS mechanism for emerging connectivity/beyond connectivity-oriented services.

- Study how to support AI-based QoS mechanism.

**** Second Change ****

5.X. Key Issue #X: Key Issue Title

It is proposed to study the following key issues for WT#1.2 to support New QoS in 6GS:

- Whether and how to enhance connectivity-oriented QoS:

  - Whether and how to provide QoS parameters for time-critical services, such as round-trip delay budget, jitter, and maximum packet reorder ratio.

  - Whether and how to support two-way negotiable QoS mechanism to improve the adaptability of QoS policy and the dynamically changing network environment, e.g., for high mobility scenarios or for energy saving reasons.

- Whether and how to provide an integrated QoS mechanism for emerging connectivity/beyond connectivity-oriented services:

  - Whether and how to define the QoS parameters for computing, AI, sensing, and how to aware and classify them.

  - Whether and how to achieve the collaborative decision of multi-dimension QoS indicators, e.g., bit rate, communication delay, computing delay, etc.

  - Whether and how to achieve the integrated scheduling of various resources, e.g., bandwidth, computing, storage, memory, AI model, etc.

  - Whether and how to monitoring the QoS performance from different granularities, e.g., slice, GTP-U path, QoS flow, etc.

- Whether and how to support AI-based QoS mechanism, such as AI-based QoS awareness and classification, AI-based QoS policy control and AI-based performance monitoring.

**** End of Changes ****