3GPP SA WG2#170	S2-2506828

Goteborg, Sweden, 25-29 August, 2025	(Revision of S2-250xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1657 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Support for authorization, signing and verification of third party user identity information in IMS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Correction to missing description in AF.2.2.2 and remove typos in AF.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to missing description in AF.2.2.2 and remove typos in AF.3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AF.2.2.2, AF.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

AF.2.2.2	IMS AS

Based on the received RCD properties from NEF, IMS AS validates the RCD information and stores the RCD information in HSS repository data by using the existing Nhss_ImsSDM_update service operation.

Originating IMS AS retrieves the RCD URL, RCD Server address or RCD information from the HSS. IMS AS can use the RCD server address or RCD URL to fetch the RCD information from the RCD server. Based on operator policies, IMS AS can invoke signing of the RCD information or the RCD URL.

NOTE:	The interface between IMS AS and RCD server is out of scope of 3GPP.

Terminating IMS AS forwards the RCD URL or RCD information to the terminating UE after successful signature verification.

* * * * Next Change * * * *

AF.3	Procedure for signing and verification of third party user identity information in IMS

Figure AF.3-1 depicts the procedure for signing and verification of third party user identity information in IMS.

Figure AF.3-1: Procedure for signing and verification of third party user identity information in IMS

1-2.	The UE / IP-PBX sends an initial SIP INVITE request. The originating IMS network authorizes the use of received third party user identity information.

NOTE 1:	If the third party user identity information is provided to the IMS network in the incoming initial SIP INVITE request, the originating IMS network can determine to skip steps 3-5 based on local policies and SLA between the third party network and the originating IMS network.

NOTE 2:	UE provided third party user identity information is not supported in this Release of the specification.

3.	The originating IMS AS may retrieve RCD server address, RCD URL, or RCD information from the HSS, based on the originating IMPU or wildcarded IMPU.

4.	The originating IMS AS may receive RCD server address, RCD URL, or RCD information from the HSS.

5.	If the IMS AS has retrieved an RCD URL, the IMS AS, based on configuration, may retrieve the RCD information from the RCD server using the RCD URL. The IMS AS includes either the RCD URL or, if retrieved from the RCD server, the RCD information in the outgoing SIP INVITE request.

If the IMS AS has retrieved an RCD server address, the IMS AS provides the originating IMPU or wildcarded IMPU to the RCD server, to retrieve the RCD information. The IMS AS includes the retrieved RCD information in the outgoing SIP INVITE request.

NOTE 3:	How RCD URL or RCD information are carried in SIP signalling is defined in TS 24.229 [10a].

6-8.	The originating IMS network (S-CSCF, IMS AS or IBCF) invokes signing of the RCD information or RCD URL included in the outgoing SIP INVITE request with a signing AS.

NOTE 4:	Based on operator policies, either the S-CSCF, IMS AS or IBCF in the originating network invoke signing of the RCD information or RCD URL.

9-10.	Based on operator policies, the S-CSCF, IMS AS or IBCF in the terminating IMS network invokes signature verification of the RCD information or RCD URL included in the incoming SIP INVITE request with a verification AS. The S-CSCF, IMS AS or IBCF in the terminating IMS network may reject the call when the signature verification is unsuccessful.

11-12.	The terminating UE may present third party user identity information of the calling-party to the called party either retrieved from RCD information, if present, in the SIP INVITE request or fetched from the RCD server based on the received RCD URL.

* * * * End of Changes * * * *