SA WG2 Meeting #170	S2-2506829

25 – 29 Aug, 2025, Goteborg, SE

Source:	Boost Mobile Network, EchoStar, AT&T, Telecom Italia S.p.A, Rakuten Mobile, KPN N.V., Cisco Systems, SK Telecom

Title:	[General Aspects] 6G Architecture support Cloud Native

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes that 6G system architecture shall support cloud native features of service-based scalability and service-based high availability.

# 1. Justifications

The 6G Study item objective says the following: “The study should consider at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc.” Supporting cloud native has started in 5G where SBA has been supported in addition to restful interfaces. This contribution proposes that 6G system architecture supports the remaining cloud native features which may require architectural consideration.

# 2. Discussion

There are several types of benefits of cloud native technology that 6G architecture and deployment can benefit from. Some of the commercial benefits are reduced cost, faster time to market and enhance customer experience. Other benefits that improve the operational efficiency of the 6G architecture deployment are rapid provisioning of resources, increased agility and flexibility, fully automated and secure CI/CD, and In-service up-grade and downgrade.

On the other hand, there are some of the cloud native benefits that may require to be considered as a 6G architecture principle which can be summarized as service-based high availability and resiliency, service-based automated scalability, service-based high visibility/observability, and fully stateless services.

# 3. Architectural Impact

In order for 6G system to benefit from cloud native technology, the 6G system architecture shall support an architectural principle where standardization at the level of Network Function (NF) of service based interfaces shall be supported. In other words, NFs in 6G architecture shall support standardized service-based interfaces for these benefits to be achieved. This should not contradict the existing 3GPP 5G System architecture consideration of the NF been delivered and deployed with no required standardized interoperability for interactions between different services within the NF. i.e., all services of the NF are assumed to be delivered by the same actor.

# 4. Conclusion

In order for 6G system to garnish the benefits of cloud native where high availability, scalability, and observability are at the level of the NF service, the 6G system architecture shall support standardized service-based interfaces to achieve these benefits while at the same time require no standardized interoperability for interactions between services within the same NF.

# 5. Potential Architectural Requirements

- All 3GPP 6G system defined services of any one Network Function (NF) shall not require any standardized interoperability testing for their interactions amongst them within the same NF.

- 6G system architecture shall support standardized service-based interfaces between NFs for the purpose of making use of cloud native service-based high availability, scalability, and observability.

# 6. Proposal

It is proposed to add the following architecture potential requirements to the TR 23.801 “Study on Architecture for 6G System”.

* * * Start of changes * * * *

# 4	Architectural Assumptions and Requirements

……….

……….

## 4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

- All 3GPP 6G system defined services of any one Network Function (NF) shall not require any standardized interoperability testing for their interactions amongst them within the same NF.

- 6G system architecture shall support standardized service-based interfaces between NFs for the purpose of making use of cloud native service-based high availability, scalability, and observability.

* * * End of changes * * * *