SA WG2 Meeting #S2-170	S2-2506838

25 - 29 August, 2025, Goteborg, Sweden	(revision of S2-250xxxx)

# Title:	Update of solution #15

# Source:	Sony

# Document for:	Approval

Agenda Item:	20.2.1 (FS_Sensing_ARC)

Abstract:	Update of solution#15 to remove EN

# 1. Discussion

This pCR proposes update and removal of Editor’s Note from solution#15

# 2. Proposal

It is proposed to agree the following changes to TR 23.700-14.

# * * * * First change * * * *

## 6.15	Solution #15: Sensing Function selection and Sensing Entity selection based on Sensing Entity Capabilities and Conditions

### 6.15.0	High-level solution principles

In this solution the SF (Sensing Function) keeps inventory of all available Sensing Entities and their capabilities. The SF also performs selection of SE (Sensing Entities) for every sensing service request. The NRF is used for discovery of the SF.

### 6.15.1	Description

This solution addresses Key Issue #3 and Key Issue #6.

This solution proposes a new network node Sensing Function (SF). The SF register/deregister/update itself to the NRF as defined in clause 4.17.1.1, 4.17.2, and 4.17.3 of TS 23.502 [10], respectively. In case of registration or update the Sensing Function include its supported sensing capabilities to the NRF to be stored in the SF profile at the NRF. The NRF is used for the discovery of SF's and may be used by any NF including Sensing Entities, trusted AF or by the NEF for any AF's.

The capabilities in the SF profile may include:

-	Supported area

Sensing service type (e.g. motion monitoring, object detection and tracking as defined in TS 22.137 [2]).The Sensing Entities register/deregister to the SF and the SF keeps an inventory of available Sensing Entities and their capabilities. The Sensing Entities discover the SF via local configuration or use the NRF for that.

NOTE X: A sensing deployment may contain multiple SFs for various purposes (e.g., to support different areas, different types of sensing operations etc.). Hence, a sensing deployment may allow a SE to register to one or multiple SFs.

For every new or changed sensing service request and after authorization and revocation that will be defined in Key Issue#2 the SF decides if any activation, deactivation or configuration changes is needed to any Sensing Entities. The decision logic is up to SF implementation but a set of capability parameters from the sensing entities will be specified and how the SF activate, deactivate or reconfigure the Sensing Entity.

The profile for each Sensing Entity may include the following capabilities:

-	Sensing Transmitter and/or Sensing Receiver.

-	Supported sensing area.

NOTE Y:	Additional capabilities will be decided by RAN.

### 6.15.2	Procedures

#### 6.15.2.1	Sensing entity selection Procedure

The procedure for the discover and selection mechanism:

Figure 6.15.2.1-1: Procedure to support sensing service selection

1.	NEF sends the Nnrf_NFDiscovery_Request to the NRF.

2.	NRF respond with supported SF(s) and their capabilities.

3.	NEF may select a SF based on local configuration or supported SF capabilities and the consumer's request

4.	NEF sends Nsf_Activate_Subscribe to the SF to initiate a sensing session with the based on the sensing consumer's request.

5.	SF verify that the AF is authorized for the requested sensing service request as part of solutions of KI#2

6.	SF decide from its inventory of available Sensing Entities which one to activate/deactivate or reconfigure based on the decided sensing service request.

7.	SF sends Activate to activate all needed Sensing Entities that will be included for the sensing service. This may also include deactivation or reconfiguration of already active Sensing Entities. This may also include prioritization order in performing sensing task(s). The prioritization order information may originally be configured by OAM or by local configuration.

8.	Sensing Entity perform the requested configuration and starts the requested sensing service

9.	Sensing Entity deliver the requested sensing data according to solutions for KI#4 and KI#5.

#### 6.15.2.2	Sensing Function registration procedure

The procedure for Sensing Function Registration:

Figure 6.15.2.2-1: Procedure to support Sensing Entity registration

1.	SF sends Nnrf_NFManagement_NFRegister and include the supported capabilities as defined in clause 6.15.1 that will be stored in the SF profile at the NRF.

#### 6.15.2.3	Sensing Entity registration procedure

The procedure for Sensing Entity Registration:

Figure 6.15.2.3-1: Procedure to support Sensing Entity registration

1.	Sensing Entities sends Register and include supported capabilities to the SF as defined in clause 6.15.1.

Editor's note:	It is FFS if additional configuration or local conditions at the Sensing Entity is included with the capabilities.

### 6.15.3	Impacts on Services, Entities and Interfaces

SF:

-	Inventory of available Sensing entities and corresponding capabilities.

-	Selection logic to select appropriate Sensing Entities and methods based on consumer request.

-	Authorization of sensing service request.

-	Activation/deactivation of Sensing Entities including configuration of Sensing entities.

NRF:

-	Support for storing a profile of the SF capabilities.

NEF:

-	Discovery and selection of SF based on SF profile.

# * * * * End of changes * * * *