SA WG2 Meeting #170	S2-2506852

25 – 29 Aug, 2025, Goteborg , SE

Source:	Nokia

Title:	[WT#1.2] Proposal on SBA framework enhancements

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution suggests SBA framework enhancements as a key issue related to WT#1.2

# 1. Justifications

WT#1.2 includes the following:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G:

…

SBA framework,

…

While the SBA framework in 5GC is being increasingly adopted and is considered a good basis also for 6G by the authors, some specific points could still benefit from enhancements:

- For roaming, NF profiles are supposed to be exchanged between visited and home PLMN, and the originating PLMN is supposed to select NFs in the target PLMN based on NF profiles in the target PLMN. This may violate the desire to hide details of the target PLMN and may also complicate load balancing, error handling, and configurational changes in the target PLMN. While the issue has already been partially resolved via TEI19_NFsel_by_tPLMN, there are also certain restrictions: The solution is optional, not fully applicable for H-SMF and H-PCF selection, and assumes communication models C or D. It is understood that deployments with SBA roaming are still in their infancy, so that the time window for improvements is not yet closed.

- For 5G, 4 communication models (with and without NRF and SCP, and with different SCP capabilities) have been defined in Annex E of TS 23.501. It should be studied, if supported communication models can be simplified, in particular in the light of roaming.

- The interactions between NRF and NFs for the discovery of NF service producers could be optimised: For instance, downloading large NF profiles on-the-fly may delay the service invocation, and relying on cached results may yield suboptimal or outdated results.

# 2. WT Scope

**** First Change (all new text) ****

#### Annex A.1.2.X. WT#1.2.X Service-based architecture framework scope

This Work Task will study whether and how to enhance the service-based architecture framework regarding the following aspects:

- For inter-PLMN scenario, study NF discovery and selection considering network topology hiding.

- Study whether and how to simplify the supported communication models (see clause 7, Annex E in TS 23.501 [2]).

- Study whether and how to optimize the interactions between NRF and NFs for the discovery of NF service producers.

- **** End of First Change

# 3. Potential KI(s) Scope

**** Second Change (all new text) ****

## 5.X. Key Issue X: Service-based architecture framework

This Key Issue will study whether and how to enhance the service-based architecture framework regarding the following aspects:

- For inter-PLMN scenario, study NF discovery and selection considering network topology hiding.

- Study whether and how to simplify the supported communication models (see clause 7, Annex E in TS 23.501 [2]).

- Study whether and how to optimize the interactions between NRF and NFs for the discovery of NF service producers.

- **** End of Second Change