SA WG2 Meeting #170	S2-2506871

25 – 29 Aug, 2025, Goteborg, SE

Source:	Futurewei

Title:	[WT#1.1, Non-Access Stratum] Scope and Key Issue for Non-Access Stratum

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the need for distributing NAS functionality and optimizing paging signalling.

- Justifications

Architectural Motivation:

In addition to connectivity, 6G is expected to support a range of services such as sensing, IoT, and others that are event driven and have sporadic traffic patterns while also requiring high scalability. There is a need for optimal registration, mobility and connectivity handling mechanisms for both connectivity and/or beyond connectivity services that reduce control plane congestion, while minimizing changes to existing functionality.

In 5G system, all NAS signalling is routed via a single point (AMF) that can become a bottleneck during high call volumes and mobility handling. 6G should make the functionality for routing of signalling separate as there is now a need for a wider range of connectivity and beyond connectivity services.

5G system (AMF) can use significant amount of signalling to page idle devices. With the need to support the combined scale of connectivity and beyond connectivity services, and cases like Fixed Wireless Access (FWA) with no mobility, there is a need to study if paging signalling can be further optimized.

Gap Analysis:

- A single function that handles both routing NAS signalling and registration /mobility is not well suited for wider range of connectivity and beyond connectivity services in 6G.

- Significant amount of signalling and capacity at AMF is used for paging idle devices. However, some services like FWA do not need paging and on the other hand, 6G services for connectivity/beyond connectivity that need mobility needs to support a larger number of connections and resulting signalling for paging.

Technical Impact: 6G Control Signalling and Connectivity Services will require:

- Distribution of NAS functionality where routing of NAS signalling is separate from functionality handles basic registration and mobility management, or other functionality for enhanced services.

- Study the enhancement of paging mechanisms and procedures that minimize control plane congestion for connectivity and beyond connectivity services.

**** First Change ****

# Annex A.X. WT#1.1 Scope

WT 1.1 Enhanced NAS:

Study the general architectural aspects for control signaling and connectivity services in 6G that can support a range of connectivity services with connections (PDU sessions) and beyond connectivity (non-session connectivity)

- Investigate new/enhanced functionality for handling routing of NAS signalling that is separate from functionality handles basic registration and mobility management, and other functionality for enhanced services.

- Investigate enhanced mechanisms and procedures for signalling for paging and reduce congestion in the control plane.

**** Second Change ****

# 5.X. Key Issue #X: Enhanced NAS

KI #X (1.1.X) Enhanced NAS

Study whether and how to 6G can support enhanced NAS functionality:

- study whether and how to distribute NAS functionality such that handling routing of NAS signalling is separate from functionality handles basic registration and mobility management, and other functionality for enhanced services.

- study whether and how to enhance mechanisms and procedures for signalling for paging and reduce congestion in the control plane.

**** End of Changes ****