SA WG2 Meeting #170	S2-2506872

25 – 29 Aug, 2025, Goteborg, SE

Source:	Futurewei

Title:	[WT#1.2, Enhanced User Plane and QoS] Scope and Key Issue for Enhanced User Plane and QoS

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses areas to study for enhanced user plane and QoS mechanisms that enable high scalability for user plane transport and functional entities, and mechanism to provide QoS that is adaptable over the life of flows.

- Justifications

Architectural Motivation:

6G is expected to support a range of services such as sensing, IoT, and others that are event driven and have sporadic traffic patterns, in addition to connectivity sessions in 5G. There is a need to study enhanced user plane and QoS mechanisms and procedures in an efficient manner for higher scalability with less state per-user in the user plane.

Connectivity/session handling in 5G system establishes state in the RAN user plane (via NGAP) and core user plane (via PFCP) for each PDU session. This involves maintaining per-user state in the user plane for each session. For better scalability in the user plane as the number of sessions increase, enhancements to minimize state or group into categories should be studied.

In 6G, transport between functional entities of the user plane in RAN and core should support connections that are session oriented and those that are not session oriented. A flexible transport setup can manage the high number of session and non-session-oriented connections. This should be achieved by enhancing the underlying transport in 5G.

Traffic characteristics for new services may include short bursts at random intervals and they may be driven by events and over extended periods of time. QoS in 6G should be able to optimally support the range of user traffic and beyond connectivity traffic that exhibits these traffic patterns. This should be possible even when flows are encrypted E2E, and traffic analysis/DPI is not practical. E2E mechanisms such as L4S cannot respond to rapid changes in radio channel capacity and other means for adaptive QoS that manage classification of packets in the presence of sub-RTT changes in capacity should be studied.

Gap Analysis:

- Due to increased number of connections to support and the sporadic nature of data transmission over extended periods of time that is needed, maintaining connectivity state per- session/non-session in user plane entities is not scalable.

- Transport/N3 interface in 5G maintains state per user, and as the number of users/devices for basic connectivity and beyond connectivity increases, setting up the transport is not flexible or scalable.

- Flow based QoS in 5G depends on classifying the required traffic characteristics. However, encrypted traffic, aggregation of flows into one transport (e.g., WebTransport) and potentially new traffic characteristics for beyond connectivity make the current QoS mechanism not able to adapt to changes.

Technical Impact: 6G User Plane and QoS enhancements will require:

- Enhanced mechanisms and procedures to manage connection state efficiently in user plane for a high number of connections for user and beyond connectivity services (e.g., IP prefix colouring that indicates a common policy)

- Enhanced user plane transport mechanisms between the UP functional entities that contains minimal per-user state in the transport layer for more flexible routing and forwarding of data packets.

- Enhanced mechanisms and procedures for QoS that are adaptive over the life of flows and able to support a wide range of traffic characteristics for encrypted flows, dynamically changing and potentially aggregated application transport.

**** First Change ****

# Annex A.X. WT#1.2 Scope

The WT(s) should cover the following objectives:

WT 1.2.X1 Enhanced User Plane

Study the architectural aspects for an enhanced user plane:

- Investigate enhanced mechanisms and procedures to manage connection state efficiently in user plane for a high number of connections/sessions.

- Investigate enhancements for user plane transport between the UP functional entities that contain minimal per-user/session state in the transport layer for more flexible forwarding of data packets.

WT 1.2.X2 Enhanced QoS Framework

- Investigate mechanisms and procedures for QoS that are adaptive over the life of flows, including flows that are encrypted, to support a wide range of traffic characteristics.

**** Second Change ****

# 5.X. Key Issue #X Enhanced User Plane and QoS

KI #X1 (1.2.X1) Enhanced User Plane

Study whether and how an enhanced 6G user plane can handle connection service state more efficiently and minimize per-user state in user plane transport networks.

KI #X2 (1.2.X2) Enhanced QoS Framework

Study whether and how an enhanced QoS framework can handle changes in traffic patterns /QoS adaptively of flows in the session/connectivity service.

**** End of Changes ****