SA WG2 Meeting #170	S2-2506873

25 – 29 Aug, 2025, Goteborg, SE

Source:	Futurewei

Title:	[WT#7, Service Continuity] Scope and Key Issue for Service Continuity

Document for:	Agreement

Agenda Item:	20.6.7

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution discusses the areas to study for enhanced service and session continuity for TN/NTN accesses.

- Justifications

Architectural Motivation:

In the 6G timeframe, unification between TN and NTN for scenarios such as transportation and public safety, among others, will be an important consideration. Integrating these multilayers of TN and NTN communication can provide higher capacity, ubiquitous coverage, reliability and availability. One aspect that core networks should provide is an efficient means for service continuity including across multiple TN/NTN access connections.

Service continuity in 5G allows the session to be transferred from one access path to another. However, the transfer is complex and there is noticeable impact to the service when the flow has demands for sustained high throughput and/or low latency (managing queue buildup and resulting impact on the connection, etc). Mechanisms and procedures for simple procedures and while sustaining the QoS during this process is needed.

The concept of “edge” when considering TN and NTN is complicated due to the location of ground stations and point-of-presence (PoP) routers. This can result in uneven/long paths, inefficient routing in network and added latency. For multiaccess TN/NTN, further delays can result if e2e flows always must transit a common router (UPF-PSA).

Simplified mechanisms that can allow change of access across multiple accesses of TN and NTN without disrupting the E2E service flow should be studied. Only then can 6G connectivity service deliver on the promise of higher capacity, ubiquitous coverage, reliability and availability.

Gap Analysis:

- An edge location for TN and NTN connections is complex due to the location of ground stations /point-of-presence routers and UPF-PSA. This can result in uneven/long paths for the E2E flows.

- Services that use TN/NTN sessions experience significant changes in capacity that affects the flow during the transition from one access connection to another.

Technical Impact: 6G TN/NTN service continuity should include:

- Enhanced service continuity mechanisms and procedures to provide seamless service continuity and sustain high capacity and provide reliability for the service as TN and NTN connections/sessions change due to user mobility, congestion or poor network conditions.

**** First Change ****

# Annex A.X. WT #7 Scope

These WT(s) should cover the following objectives:

WT #7.X Enhanced Session and Service Continuity:

Study whether and how to enhance session continuity mechanisms and procedures to provide seamless service continuity with support for high bandwidth and reliability for a service as connections/sessions of TN and NTN change due to user mobility or other network conditions.

**** Second Change ****

# 5.X. Key Issue #X: Enhanced Session Continuity

KI #7.X Enhanced Session and Service Continuity:

Study whether and how to enhance session continuity mechanisms and procedures to provide seamless service continuity with support for high bandwidth and reliability for a service as connections/sessions of TN and NTN change due to user mobility or other network conditions.

**** End of Changes ****