SA WG2 Meeting #169	S2-2506886

# Goteborg, Sweden, August 25 – 29rd 2025                                   a revision of S2 2504998

Title:	[DRAFT] LS for a new KPI related to the maintainability of a QoS flow

Response to:

Release:	Rel-19

Work Item:	eNA_Ph3, TEI19

Source:	SA2

To:	SA5

Cc:

Contact Person:

#### Name:	 Adrian Buckley

E-mail Address:	 adrian dot buckley at verizon dot com

Uri Baniel

E-mail Address:	 uri dot baniel at oracle dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-2504996- Addition of QoS Flow Maintainability KPI

- Overall Description:

In TS 23.288 clause 6.9, QoS Sustainability Analytics can provide analytics information regarding the potential QoS

change statistics for an Analytics target period in the past in a certain area or the likelihood of a QoS change for

an Analytics target period in the future in a certain area.   The consumer of this analytics provides reporting

threshold(s) to indicate the conditions on the analytic level for triggering the reporting. In other words, when the

analytics result crosses the threshold of the expected QoS KPI, a report is triggered.

As indicated in TS 23.288 clause 6.9.1, for a GBR flow, the reporting threshold(s) refers to the QoS flow

Retainability KPI. In other words, the sustainability of the flow is determined by a threshold related to how often

the end user abnormally loses the QoS flow during the time the QoS flow is used. The QoS flow Retainability KPI

is defined in TS 28.554 clause 6.5.1.1

The QoS flow Retainability KPI is not sufficient to determine the ability of the wireless system to maintain a flow

with some level of acceptable degradation.

If the OAM system has the ability to monitor and capture statistics of downgraded QoS flows by for example

counting the QoS degradation events per a period of time, then in addition to QoS flow Retainability, a QoS flow

Maintainability should be added providing a KPI that reflects the probability to experience a QoS degradation.

The KPI will serve as a measure to determine the ability of the wireless system to maintain the flow.

If provided, QoS flow Maintainability will be an important information that can a) help

the consumer of the analytics to potentially sustain the flow even when certain level of KPI degradation is

acceptable and/or adoptable b) enable the consumer to activate Alternative QoS Profile and c) provide a good

RAN deployment tool aiming to improve the performance of QoS flow KPIs in a given area of interest.

The attachment provides a conditional CR integrating the QoS flow Maintainability KPI if/when it becomes available.

2. Actions:

To SA5:

ACTION:

- SA2 would like to understand the present or future ability of SA5 to monitor and capture statistics of downgraded QoS flows.

- If the answer to question a is positive, SA2 kindly asks SA5 to consider adding a QoS Maintainability KPI on the basis of IP flow level. The KPI will reflect the probability to experience a QoS degradation of any kind during the time when the QoS flow is used.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#171 | October, 25th-29th | Wuhan | China |
| SA2#172 | November, 17th – 21st | Dallas | Texas |