SA WG2 Meeting #170	S2-2506888

25 – 29 Aug, 2025, Goteborg , SE

Source:	Apple

Title:	[WT#1.2, QoS] Key Issue for 6G QoS framework enhancements

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes a Key issue to study an adaptive and agile QoS Framework for 6G

# 1. Justifications

## Architectural Motivation:

Internet traffic characteristics has changed a lot in recent years. With immersive XR and generative AI applications, traditional traffic patterns (share of UL/DL in the overall flow, nature of uplink traffic i.e. prompts) have undergone changes.

The new services expected in 6GS, AI, sensing and computing may also bring new requirements for traffic handling.

## Gap Analysis:

In the 5G QoS framework, definition of a QoS Flow as the lowest granularity of differentiated traffic is limiting. With evolving application design, network has more flexibility to handle traffic. QoS targets within a QoS Flow are no longer static. An adaptive handling of quality of service providing finer traffic treatment within the QoS flow is required. With such adaptive handling, it is also important to avoid head of line blocking for traffic that can be processed independently,

Adaptive traffic management within a QoS Flow including priority and delay budget

Adaptive Priority

The information on how the user is consuming content in the UE can be useful in determining the priority of a QoS Flow relative to another QoS Flow for the UE. There are also cases where PDUs/ PDU Sets within a QoS flow may need to be treated with different priority

Scalable Video Coding: There are cases when the packets from the same IP flow may need different treatment e.g., with respect to scheduling even when operating within the same QoS Flow boundary. This differential handling is required for IP flows that have enabled PDU Set based QoS handling or PDU based QoS handling. Certain packets within the flow may have a different importance for the application and needs to be expedited in case of congestion. An example of this type of flow is Scalable Video Codec (SVC).  In this case, application encodes videos in hierarchical layers, comprising a foundational base layer that provides a lower resolution (e.g., 360p) but independently decodable stream, and one or more enhancement layers that progressively add data for higher resolutions (e.g., 720p, 1080p). The primary advantage of SVC, especially in congested scenarios with fluctuating packet loss, is its ability to enable graceful degradation, i.e., if network conditions worsen, higher resolution enhancement layers can be selectively dropped or delayed, ensuring a continuous, albeit lower-quality, video stream is maintained without interruption, thereby optimizing bandwidth usage and improving user experience under challenging conditions.

Another example of traffic that may have a need to prioritize data bursts within a QoS Flow are multi-modal applications that may have multiple independent video streams (e.g., multiple camera feeds for a sports event for an immersive application). There are popular video applications that allow users to consume multi-modal content in much diverse ways such that priority of packets within a QoS flow may vary quite dynamically (video in main app window vs video playback inside a large number of video tiles within the app). Depending on how the user is interacting with the application, some data bursts / PDUs /PDU Sets from the QoS Flow may need to be expedited in 6GS.

Adaptive Delay Budgets

Differentiated handling for Generative AI prompts that results in different server processing delays. The delay budget for the prompt traffic in uplink may need to be dynamically adjusted to get a harmonized user experience from the application.

Avoid head of line blocking

With an adaptive QoS flow framework, system enables selective prioritization or expedited handling of certain parts of the flow as per the application’s requirements. Such selective handling imply that these components of the flow can be processed independently at the receiver. Then it becomes important for the QoS framework to have knowledge about the flow to avoid head of line blocking in a QoS Flow for traffic that can be processed independently.

Inputs from UE

UE may be able to provide quick and timely input about the importance of a flow on the device. Such kind of UE inputs can be applied by the 6G system for an adaptive traffic management with an aim to optimize system performance and user experience.

## Technical Impact

The 6G Quality of Service framework is expected to inherit the baseline from 5G/5GA design. There might be dependency from the architecture defined for new services e.g., AI, sensing and computing.

Inter-working/Migration aspect may not be applicable since the enhanced treatment will be available only in 6G.

**** First Change ****

# 5.X. Key Issue #X: Adaptive Quality of Service in 6G

Traffic patterns have changed quite a lot since the definition of 5G Quality of Service framework. Traditional traffic profiles no longer accurately reflect application behaviour (e.g., increased share of uplink traffic compared to downlink traffic). There are popular video applications that allow users to consume multi-modal content in much diverse ways such that priority of packets within a QoS flow may vary quite dynamically (video in main app window vs video playback inside a large number of video tiles within the app).

On the other hand, diverse AI tasks of Generative AI applications may require significantly different processing time by e.g. the large language models in the cloud server, even if they have the same type of AI prompts. Such nature of AI tasks may bring some challenges to maintain a true immersive experience for the end users.

Even for a single flow within an application, traffic needs differentiated treatment. 6G QoS Framework has to be agile and forward-looking considering the needs for future applications.

This key issue aims to study the following improvements in QoS Flow handling

- How to provide differentiated treatment for traffic within a QoS flow (e.g., intra-flow prioritization of PDUs/PDU Sets, differentiated priority handling of data bursts)

- How to avoid head of line blocking in 6GS for traffic that can be processed independently.

- How to enable dynamic QoS adaptation with additional UE input.

**** End of Changes ****