3GPP SA WG2 Meeting #170	S2-2506895

25 - 29 August, 2025, Goteborg, Sweden

# Title:	Reply LS on the path information in 5G ProSe multi-hop discovery procedures

# Response to:	LS on the path information in 5G ProSe multi-hop discovery procedures (S2-2506119 / C1-253724)

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Hong Cheng

Tel. Number:

E-mail Address:	hongc AT qti DOT qualcomm DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	CR 0559 to TS 23.304

1. Overall Description:

SA2 would like to thank CT1 for the LS on the path information in 5G ProSe multi-hop discovery procedures (S2-2506119 / C1-253724).

SA2 had discussed the matter and would like to provide the answers as follows:

Question 1: For both model A and model B discovery, given that in the path to a 5G ProSe multi-hop layer-3 end UE, the 5G ProSe multi-hop layer-3 end UE's first 5G ProSe layer-3 multi-hop UE-to-UE relay inherently stores the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE, why does the path information need to include the source layer-2 ID of the 5G ProSe multi-hop layer-3 end UE?

SA2 Answers: Similar to the single hop 5G ProSe UE-to-UE Relay operations (as specified in TS 23.304 clause 5.8.4), the User Info of the 5G ProSe End UE is expected to be protected in the Direct Discovery set. Therefore, the 5G ProSe multi-hop UE-to-UE Relay may not have visibility to the identifier of the End UE. Therefore, to better support the operations, e.g. the Response in model B discovery (as in clause 6.3.2.6.3.2) or the link establishment (as in clause 6.7.5.2.2), it is beneficial to have the 5G ProSe End UE's Layer-2 ID included explicitly, to allow a unicast transmission of the mesasges.

Question 2: In the 5G ProSe UE-to-UE relay discovery announcement message (for model A discovery), it specifies the path information includes the "source layer-2 ID of the discoverer ProSe end UE (yellow highlighted texts as below)". However, the term "discoverer ProSe end UE" pertains to model B discovery. Why does the announcing relay UE indicate this source layer-2 ID of the discoverer ProSe end UE in the path information, given that model A does not involve a "discoverer" end UE?

SA2 Answers: It is true that "discoverer" may be misleading. SA2 had agreed to attached CR to remove "discoverer" and clarify that it is the source layer-2 ID of the 5G ProSe End UE in the Direct Discovery Set.

Question 3: Why are the Layer-2 IDs included in the Multi-hop path info IE, together with the list of the User info IDs?

SA2 Answers: The Layer-2 IDs were included to facilitate the Response message transmission as in step 8-9 of clause 6.3.2.5.3, as in the Model B discovery the Intermediate Relay(s) does not store any information.

2. Actions:

To CT1:

ACTION: 	SA2 would like to ask CT1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #171		13-17 October 2025	Wuhan, China

TSG-SA2 Meeting #172		17-21 November 2025	Dallas, TX, USA