SA WG2 Meeting #170	S2-2506900

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#1.2, QoS framework] Potential enhancements to QoS framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution reviewed the potential key issues to address the WT1.2 sub-topic on QoS aspect.

# 1. Justifications

QoS framework has been relatively stable from EPS (TS 23.401 clause 4.7) to 5GS (TS 23.501 clause 5.7), with only incremental changes and enhancements.  This enabled the proper system migration and interworking for the important existing services, while allowing some optimization for new services.

The same principle should be kept for the 6G QoS framework. In other words, the 6G QoS framework should be based on the existing 5G QoS framework (as in TS 23.501 clause 5.7). There is no need or justifications to create an entirely new QoS framework.

For the 6G system, the SA1 study (FS_6G_REQ) had the assumption to support all the existing 5G services (TR 22.870 clause 5.1):

The 6G system shall support new 6G services and capabilities and it is assumed that the 6G system shall support existing 5G services and system requirements, unless explicitly identified.

Therefore, the starting point of the 6G System study for QoS aspect should be the 5GS QoS framework, and all the existing 5QIs need to be supported in 6G.

In 6G, there are new services that are very different from the existing data services, e.g. Sensing. However, the QoS for such services should be investigated in the related WTs first, e.g. WT#4, to understand its requirements and impacts to the 6G system architecture. For example, the QoS for Sensing can have two aspects, i.e. the quality of the sensing result (e.g. accuracy, number of trackable objects, etc.). These are related to the sensor capabilities and the sensing methods supported by the system. They do not have relevance to the 6G QoS framework.

On the other hand, in order to support sensing operations, there may be needs to transport large amount of sensing measurement results within a required time window. This would have impacts on the 6G QoS framework, i.e. new QoS characteristics may need to be defined for such traffic pattern generated by sensing operations. However, such enhancement can be achieved with the existing QoS framework and does not have system architecture impacts.

In addition, for some services, e.g. XR, certain optimization to make the RAN to be aware of application layer requirements could be useful. For example, the mapping of QoE or application quality criteria with the QoS parameters may help the RAN to better adapt the radio resources allocation during congestions. Such enhancements can be achieved with some extension of the existing QoS parameters in the QoS Profile or the QoS parameters sent to the UE. It should be studied with the specific needs identified by SA1, SA4, and RAN WGs.

There are also features in the existing QoS framework that is useful but not widely deployed, e.g. UE initiated QoS Flow addition/modifications. In some applications, especially the new uplink centric live streaming or XR services, the UE has more information and could assist the network to adjust the QoS control. However, the support of UE initiated QoS management is not universal. Therefore, it may be helpful to inform the UE of such support in a particular network, so that better coordination or cooperation between the UE and network can be achieved.

**** First Change ****

# Annex A.X. WT#1.2 Scope on QoS framework

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

For WT#1.2, the QoS framework should be handled as part of the general 6G system framework.

It should use the existing 5G System QoS framework as the baseline and focus on the potential adaptation to the 6G CN and RAN architecture.

The QoS support for new services, e.g. Sensing, should be studied after requirements are identified in the study of such services first. In this work task, the focus should be addressing corresponding transport requirements by the QoS framework.

This work task should also address further improvements to the existing 5G QoS framework to better serve certain application needs.

**** Second Change ****

# 5.X. Key Issue #X: QoS Framework for 6G System

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The 6G QoS framework should be based on the 5G QoS framework as defined in TS 23.501 [x] and support all the existing standardized QoS characteristics.

The study on the 6G QoS framework should cover the following aspects:

- 	whether and how to enhance the QoS parameters/profiles to allow better RAN awareness of application layer requirements, e.g. QoE;

-	whether and how to enhance the signalling to allow better UE assistance in QoS management.

**** End of Changes ****