SA WG2 Meeting #170	S2-2506901

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#1.2, policy framework] Potential enhancements to policy framework

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution reviewed the potential key issues to address the WT1.2 sub-topic on policy framework aspect.

# 1. Justifications

The Policy Framework is an integral part of the core network, and therefore it should be addressed as part of the 6G system study. There are two aspects of the policy framework that should be reviewed:

-	the contents and formats of the policies; and

-	the mechanism to manage and deliver the policies to the policy consumers, e.g. NF, RAN, or UE.

For the former, the required policies depend on the services supported by the 6G system. For existing services supported by 5GS, the policies defined in TS 23.503 are sufficient and can be used as the baseline for 6G System. For the new services, e.g. sensing, AI, compute, etc., the corresponding policy enhancements should be decided together with the designs for those services. Therefore, this aspect should be addressed as part of the corresponding work tasks for the new services.

On the latter, the PCF centric policy handling framework can be also used as the baseline for 6G systems. In 5GS, the PCF offers SBI for providing policies to NFs, and NAS based signalling of UE policies towards the UE. In 6GS, similar mechanisms can be reused. In 5GS, the policies to NG-RAN always go through the AMF. For 6G system policy framework, optimizations can be studied on this aspect, i.e. whether a direct access to the SBI of PCF from RAN can be achieved.

**** First Change ****

# Annex A.X. WT#1.2 Scope on Policy framework

For WT#1.2, the Policy framework study should be handled as part of the general 6G system framework study.

The policy framework as defined in TS 23.503 [y] should be taken as the baseline for 6G system policy framework. Specifically, the PCF based policy management schemes should be applied to 6G systems, and the policies defined for the existing services should be also supported in 6G systems.

In WT#1.2, the study can focus on the optimization for the policy management and delivery mechanisms, e.g. potential support of direct access to PCF from RAN.

The study for policy enhancements for individual 6G services, e.g. sensing, should be handled in the corresponding WTs for those services instead of WT#1.2.

**** Second Change ****

# 5.X. Key Issue #X: Policy Framework for 6G System

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The 6G Policy framework should be based on the PCF based framework as defined in TS 23.503 [y]. The policies defined in TS 23.503 [y] should be supported in 6G system when applicable.

The study on the 6G Policy framework should cover the following aspects:

- 	whether and how to enhance the policy framework to allow direct access from RAN;

NOTE: Potential enhancements to the policies for 6G services should be addressed in the study for the corresponding services.

**** End of Changes ****