3GPP TSG-WG SA2 #170	S2-2506902

# Aug 25 – 29, 2025, Gothenburg, Sweden

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6391 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#3: Align the mapping of PSI value to DSPC Drop Precedence setting |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | XRM_Ph2 |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For KI#3: The 2nd ‘NOTE 1’ (Note# to be corrected) of 5.8.2.7 recommends the mapping of the PSI based transport-level marking only impact the DSCP drop precedence. - As stated in the RFC-2597 (Assured Forwarding PHB Group), quoted: Within each AF class IP packets are marked …… with one of three possible drop precedence values…... A congested DS node tries to protect packets with a lower drop precedence value from being lost by preferably discarding packets with a higher drop precedence value. This means ‘for DSCP drop precedence, a lower value is better than a higher one’. - Regarding the ranking of PSI values, according to the caluse 4.2.4 of TS 26.522, quoted: PDU Set Importance [PSI] (4 bits): The PDU Set Importance field indicates the importance of this PDU Set compared to other PDU Sets within the same Multimedia Session…... Lower values shall indicate a higher importance PDU Set, with the highest importance PDU Set indicated by 1 and the lowest importance PDU Set indicated by 15. When the RTP sender cannot define an importance, it shall set the value to 0. Here the clause elucidates explicitly the relationship between the true importance of a PDU Set and its associated PSI value. That is, ‘the lower, the better’. - However, even if the 2nd ‘NOTE 1’ of 23.501 5.8.2.7 does recommend the marking impact only to DSCP DP, it does not explicitly specify the mapping relationship between the PSI value of a PDU and its corresponding ranking importance. This suggests only reading the clause 5.8.2.7 of 23.501 not comprehend what would be the most suitable practice to set the list of transport level packet marking values. In summary, to add a reference to the corresponding clause 4.2.4 of TS 26.522 would serve the purpose better and make the marking clearer. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the 2nd ‘NOTE 1’ of 5.8.2.7 to reflect the better mapping alignment between PSI values and DSPC Drop Precedences - (Editorial) correct the NOTE index# in the clause 5.8.2.7 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mapping between PSI values and DSPC Drop Precedences is not well aligned. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.2.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of changes* * * *

#### 5.8.2.7	PDU Session and QoS Flow Policing

ARP is used for admission control (i.e. retention and pre-emption of the new QoS Flow). The value of ARP is not required to be provided to the UPF.

For every QoS Flow, the SMF shall determine the transport level packet marking value (e.g. the DSCP in the outer IP header) based on the 5QI, the Priority Level (if explicitly signalled) and optionally, the ARP priority level and provide the transport level packet marking value to the UPF.

NOTE 1:	If no transport level packet marking value is provided, the UPF performs transport level marking based on its pre-configured logic.

For QoS Flows that are configured for PDU Set based handling, the SMF may additionally take into account the PDU Set Importance when determining the transport level packet marking values. In this case, the SMF provides a list of transport level packet marking values for the downlink direction to the UPF in a FAR, each of the transport level packet marking values corresponding to one or more PDU Set Importance values.

When an I-SMF/I-UPF is inserted in the PDU Session and the I-SMF is notified by the SMF that Transport Level Marking is being applied for the affected QoS flow, the I-SMF may instruct the I-UPF to derive the transport level packet marking of the outgoing N3 downlink packet based on the transport level packet marking of the incoming N9 downlink packet.

NOTE 2:	It is recommended that the PDU Set Importance based transport level packet marking values applied by the UPF only vary the drop precedence between PDUs in the transport network nodes (e.g. IP routers) on the N3/N9 interfaces. This is needed to avoid causing packet reordering in the transport network when different transport level DSCP marking values are used for a single QoS Flow. Further, when the PDU Set Importance based transport level packet marking values are applied to vary the drop precedence, it is recommended to align the settings of drop precedence with the guidance of mapping PSI values to their true importance as described in the clause 4.2.4 of TS 26.522 [179].

NOTE 3:	The transport level packet marking values are provided on per-QoS Flow basis and it is up to operator deployments to enforce consistency of transport level marking in the transport network.

The SMF shall provide the Session-AMBR values of the PDU Session to the UPF so that the UPF can enforce the Session-AMBR of the PDU Session across all Non-GBR QoS Flows of the PDU Session.

SMF shall provide the GFBR and MFBR value for each GBR QoS Flow of the PDU Session to the UPF. SMF may also provide the Averaging window to the UPF, if Averaging window is not configured at the UPF or if it is different from the default value configured at the UPF.

In the case of 3GPP access, the SMF may decide to activate ECN marking for L4S by PSA UPF for the QoS Flow (see clause 5.37). In this case, the SMF shall send an ECN marking for L4S indicator to PSA UPF.

# * * * * End of changes * * * *