SA WG2 Meeting #170	S2-2506903

25 – 29 Aug, 2025, Goteborg, SE

Source:	Apple

Title:	[WT#1.1] Introduction of NAS functionality without impacting other NAS functionalities

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution makes an analysis of the current NAS design in 5GS, assumes what additions might be made in the Core Network of 6G, and proposes to investigate whether a modification of the NAS protocol is needed in 6G.

# 1. Justifications

The design of NAS protocol in 5GS mandates a connection between the UE and the AMF, a so-called NAS/MM/Signalling Connection, over which the UE can also send and receive NAS messages to/from other Network Functions (NFs) such as SMF, PCF, SMSF, etc., making the communications between the UE and other NFs dependent on the existence of the connection between the UE and AMF.

Some contributions to the 3GPP 6G workshop argued for a new version of NAS protocol in 6G, which would allow for more flexibility and less dependence on the mobility anchor in the core network. As such, a new NAS design has been suggested, which would make the UE – NF communications completely independent of the UE – AMF NAS/MM/Signalling connection. At the same time, existing network deployments of 5G systems have not really shown any major issues with the current NAS design. It is, therefore, not clear or justified why the communication between the UE and legacy NFs such as SMF, PCF, SMSF, etc., should be changed or adopt a new design.

Based on these arguments, the source company thinks the 6G Core Network should inherit the 5G AMF mobility anchor functionality to provide communication between the UE and legacy NFs. However, the current trend of technology points to the assumption that, compared to 5G, there will be more NFs in the 6G Core Network facilitating new services and future use cases. From the perspective of scalability to a higher number of NFs it is, therefore, a reasonable assumption that SA2 needs to investigate how to extend the control plane framework for NF connectivity between UE and CN in 6G and whether an evolved design of NAS protocol could be thought of.

**** First Change (no changes to WT#1.1 in SP-250806) ****

# Annex A.X. WT#1.1 Scope

WT#1: Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity services and/or beyond connectivity services, including at least the following:

a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities.

b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

c.	Whether and how to develop generic mechanisms for UE-Core Network interaction to support operator services.

**** Second Change ****

# 5.X. Key Issue #X: Evolved NAS framework

The following initial KI is with reference to WT#1.1 bullet a):

This key issue will analyse whether and how to maintain connectivity between UE and legacy network functions, such as SMF, PCF, SMSF, etc., while enabling different mechanisms for the UE to have communication with new network functions that will be introduced in 6G.

**** End of Changes ****