3GPP SA WG2 Meeting #170	S2-2506905

# Aug 25 – 29th, 2025; Gothenburg, Sweden. 				(revision of s2-2505023)

Source: 	China Mobile

Title: 	KI#5 Solution: UE-Sat-UE communication for non-IMS service via Onboard I-UPFs

Document for: 	Approval

Agenda Item: 	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This document proposes a solution to address the KI#5 (TR 23.700-19), i.e., UE-SAT-UE communication via UPF only onboard satellite for non-IMS services, e.g. a 5G-LAN service with IP and Ethernet PDU Session type, under the scenario of always available feeder link for the NR NTN case.

## 1	Discussion & Proposals

This solution targets for the KI#5, i.e., UE-Sat-UE communication via UPF only onboard satellite for non-IMS services, e.g., 5G-LAN service. This solution is based on the agreed architecture assumption as in the TR 23.700-19 clause 4.

The term ‘UE-Satellite-UE communication’, as defined in the TR 23.700-19 clause 3.1, indicates the communication, between UEs under the coverage of one or more serving satellites, uses satellite access without the user traffic transiting thru the ground segment. Also, one agreed item in the architecture assumption clause assumes a gNB and a UPF is present onboard of satellite(s). Therefore, we recommend to provision and deploy an I-UPF on board serving satellite.

Another agreed architectural item assumes the feeder links (of serving satellites) are always available. This offers a greater flexibility for PDU sessions supporting the 5G LAN service for US-Sat-UE communication, in which the PSA UPF that is related to UE IP address allocation (for the case of IP PDU session) can be either onboard serving satellite or not. This does reflect various deployment scenarios of PSA UPFs. Since we have suggested above deploying an onboard-satellite I-UPF for a PDU session, to reduce the complexity of the solution (e.g., thanks to the connectivity challenges among serving satellites), we think it would be better to provision and deploy the PSA UPF on the ground.

Because of the extreme dynamics of the service satellites, we want to maintain a minimum necessary set of core network functions (i.e., 5GC) onboard satellites. Also, the pre-condition of always-available feeder links of serving satellites lays the solid foundation to have 5GC on the ground.

## 2	Proposal

It is proposed to capture the solution proposal for the inclusion in the TR 23.700-19 v0.2.0.

# * * * * First change * * * *

## 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions |  |  |
| --- | --- | --- |
|  | <Key Issue #1, #2, #3, #4> | <Key Issue #5> |
| Solution #x: UE-Sat-UE communication for Non-IMS service via Onboard I-UPFs |  | X |
| …… |  |  |

# * * * * next change(all new texts)  * * * *

## 6.x	Solution#x: UE-Sat-UE communication for Non-IMS service via onboard I-UPF(s)

### 6.x.0	High-level solution principles

### This solution is proposed to enhance PDU Session Establishment procedure, including

-  AMF sends the type of satellite backhaul information, along with the UE’s serving satellite ID to the SMF

-  5G VN group data update to add UE-SAT-UE communication indication

-  SMF performs (I-)UPF selection based on satellite related information and new parameters in 5G VN group data

-  I-UPF is enhanced to handle the local-switch and/or N19-based switch for the UE-Sat-UE communication of a 5G VN group.

### 6.x.1	Description

This solution targets for the KI#5, i.e., UE-Sat-UE communication via UPF only onboard satellite for non-IMS services, e.g., 5G-LAN service. This solution follows the scenarios below: for an on-ground UE via satellite access, its PDU sessions supporting the 5G LAN service will have an intermediate UPF (i.e., I-UPF which contains ULCL/BP UPF and local PSA UPF) provisioned and deployed onboard serving satellite, in addition to the central PSA UPF that is provisioned and deployed on the ground for the PDU session. Further, to leverage the always-available feeder links, the 5G core network or 5GC will be deployed on the ground to support the UE-SAT-UE communication via UPF only onboard satellite for non-IMS services (e.g., 5G-LAN services with IP and Ethernet PDU Session type).

The deployment architecture for the proposed solution is shown in the Figure 6.x.1.1.

Figure 6.x.1.1: UE-Sat-UE Deployment Architecture

In the diagram, for the PDU session of a UE supporting 5G VN group via satellite access, we propose to add one I-UPF onboard satellite, and the corresponding (central) PSA UPF of the PDU session is deployed on the ground (to take advantage of the always-available feeder link). Here, the gNB and I-UPF can be on the same or different satellite(s). When they are on different satellites, they are connected via ISL or inter-satellite link(s). Note that the architectural assumption has stated that the existence of ISL(s) has no impact on the connectivity service from the 3GPP point of view.

In the agreed architecture assumption, it assumes the serving satellites supporting the UE-Sat-UE communication service are of NGSO type. As we know, the coverage of a NGSO (LEO or MEO) satellite spanning over a UE’s geographical location is sufficiently long enough for a UE to finish all kinds of signaling procedures, e.g., registration, connection management, PDU session establishment, etc. For example, assuming a LEO satellite orbits at the 500 km altitude, which makes the orbital period be roughly 95 minutes. With a reasonable choice of the minimum elevation angle, a specific spot on Earth could access the satellite approximately for 7 minutes once within the coverage of the satellite. Further, a feeder link is assumed to be always available. With the consideration of NGSO-type satellite and the feeder-link availability, we recommend having 5GC deployed on the ground, which is shown in the diagram.

### 6.x.1.2	UE-Sat-UE communication traffic handling via onboard I-UPF(s)

The user plane handling in 5.29.4 lists three types of traffic forwarding methods allowed for 5G VN group communication:

- N6-based: the UL/DL traffic for the 5G VN communication is forwarded to/from the DN off the centralized PSA UPF of each PDU session.

- N19-based: the UL/DL traffic for the 5G VN group communication is forwarded between PSA UPFs of different PDU sessions via N19 tunnels.

- Local switch: the UL/DL traffic is locally forwarded by a single UPF if this UPF is the common PSA UPF of different PDU Sessions for the same 5G VN group.

In this solution, the centralized PSA UPF of a PDU session is proposed to be deployed on the ground. However, the UE-Sat-UE communication requires the users’ traffic not-transiting thru the ground segment. This means the UE-Sat-UE traffic can only be transported thru the onboard I-UPF(s). Thus, the N6-based forwarding method is not considered in the solution.

If UEs are covered by the same serving satellite, then the local-switch method is considered. Otherwise, if UEs are covered by different serving satellites, then the N19-based forwarding method is considered (note that ISLs, even existing, are of no impact for this case).

In existing 5G VN as in 23.501 clause 5.8.2.13 (Support for 5G VN group communication), for both the local-switch and the N19-based forwarding methods, the traffic forwarding is conducted at the PSA UPF of a PDU session. In comparison, this solution proposes that the traffic forwarding in the UE-Sat-UE scenario will be done via the onboard I-UPF. So, the I-UPF will be enhanced to support the new functionalities, including the setup of N19 tunnels between all participating I-UPFs. Further, all the traffic forwarding schemes of a 5G VN, as shown in 23.501 5.8.2.13, will be conducted at the I-UPF.

### 6.x.1.3	Identification of 5G VN Group Supporting UE-Sat-UE communication

As explained in the clause 5.29.2 of TS 23.501, currently a 5G VN group is 1:1 mapped via the combination of (DNN, S-NSSAI). When the mapping scheme is applied to the UE-Sat-UE communication based 5G VN group, the following two aspects should be supported.

1.	Utilize existing ‘satellite’ information: During the PDU Session Establishment procedure as in the clause 4.3.2.2.1 of TS 23.502, the AMF in the step-3 can determine if a UE is using NR satellite access. If the UE is using satellite access, the AMF sends the type of satellite backhaul information, along with the UE’s serving satellite ID, to the SMF. Then, the SMF can use the combination of the requested (DNN, S-NSSAI), together with the satellite information (e.g., satellite backhaul, serving satellite ID) to identify a 5G VN group supporting the UE-Sat-UE communication service. For this scheme, there is no need to extend the procedure of NEF service operation information flow as in the clause 4.15.6.2 of TS 23.502.

2. 	Add a new indicator in the 5G VN Group Data as in the clause 4.15.6.3b of TS 23.502: There is a caveat in the 1st extension. In some scenarios, a UE accessing a gNB over NR satellite access can establish a PDU session associated with a 5G VN group, but without requesting the UE-Sat-UE communication feature. That is, the PDU session can have its PSA UPF deployed on the ground and the UE may communicate with other UE via the on-ground PSA UPF. Because of this potential ambiguity, we are proposing to add a new indicator in the 5G VN group data to differentiate if a 5G VN group with the combination of (DNN, S-NSSAI) supports the UE-Sat-UE communication service. For this scheme, the extension is added to the 5G VN Group Data that is associated with procedures of NEF service operation information flow as in the clause 4.15.6.2 of TS 23.502.

### 6.x.2	Procedures

Pre-assumptions:

- 5GC deployed on the ground

- Feeder link always available

- Only an individual SMF or a dedicated SMF sets can be responsible for all the PDU sessions for the communication of a certain 5G VN group.

NOTE: 	All the following procedures leverage the existing kinds of 5G VN related procedures with enhancements that are based on the clause 6.x.1. There is no new procedure introduced.

### 6.x.2.1	Enhancement of 5G VN Group Data

While the 1st extension (e.g., utilizing existing ‘satellite’ information) as elucidated in the section 6.x1.3 does not render any procedure enhancement of the NEF service operation information flow, including the create/update/delete of parameter provisioning, the 2nd extension scheme does include the introduction of a new indicator in the 5G VN group data as in 4.15.6.3b of TS 23.502.

In the following table, e.g., the table 4.15.6.3b-1 from 23.502, the new parameter ‘UE-Sat-UE communication indication’ is added to indicate the 5G VN group, which can be mapped via the DNN/S-NSSAI combination, supports the UE-Sat-UE communication as addressed by the solution.

Table 4.15.6.3b-1: Description of 5G VN group data

| Parameters | Description |
| --- | --- |
| DNN | DNN for the 5G VN group. |
| S-NSSAI | S-NSSAI for the 5G VN group. |
| PDU Session Type | PDU Session Types allowed for 5G VN group. |
| UE-Sat-UE communication indication | Indicate the 5G VN group supporting the UE-Sat-UE communication without the user traffic transmitting thru the ground segment. |
| Application descriptor | There may be multiple instances of this information; this information may be used to build URSP sent to 5G VN group members (NOTE 1). |
| Information related with secondary authentication / authorization | This may indicate: - the need for secondary authentication/authorization (as defined in clause 5.6 of TS 23.501 [2]); - the need for SMF to request the UE IP address from the DN-AAA Server. If at least one of secondary authentication/authorization or DN-AAA UE IP address allocation is needed, the AF may provide DN-AAA Server addressing information. |
| 5G VN group communication indication | Indicates that the 5G VN group is associated with 5G VN group communication. |
| Maximum Group Data Rate | This limits the total bit rate that is expected to be provided across all sessions of a 5G VN group. [optional]. |
| User Plane Security Policy | User Plane Security Policy for PDU sessions associated with the 5G VN group. |
| NOTE 1: As described in TS 23.503 [20], the PCF may be configured with a mapping from Application Descriptor to other information required to construct the URSP rules, e.g. IP filters and SSC mode. |  |

### 6.x.2.2	Enhanced procedure of PDU session establishment for UE-Sat-UE communication

The following steps of the existing procedure of PDU session establishment, as shown in the clause 4.3.2.2.1 of TS 23.502, are enhanced to support the UE-Sat-UE communication.

- Step-8: UPF selection: since our solution introduces an I-UPF for the PDU session targeting at the UE-Sat-UE traffic forwarding for a 5G VN group, the following can be added to the step:

SMF may select a UPF, based on the requested DNN, S-NSSAI, and the UE-Sat-UE communication indication in the Table 4.15.6.3b-1 in 23.502, that supports the UE-Sat-UE communication. However, if the AMF has determined in the Step-3 that the UE is not accessing over a gNB using NR satellite access, and the requested (DNN, S-NSSAI) maps to a 5G VN group with the UE-Sat-UE communication indication set, then the SMF rejects the UE request via NAS SM signaling including a relevant SM rejection cause by responding to the AMF with Nsmf_PDUSession_CreateSMContext Response. The SMF also indicates to the AMF that the PDU Session ID is to be considered as released, the SMF proceeds to step 20 and the PDU Session Establishment procedure is stopped.

- Step-10: For N4 session establishment/modification procedures, add logics to handle the 5G VN traffic forwarding scenarios that either the local-switch or the N19-based switch will be applied to the selected I-UPF.

If the UPF selected in the Step-8 acts as an intermediate UPF (I-UPF) between the NG-RAN and the on-ground centralized PSA UPF to support the UE-Sat-UE communication, an N4 Session establishment/modification request message is sent to the I-UPF, providing Packet detection, Data forwarding, enforcement and reporting rules to be installed on the I-UPF for the support of local switching and/or N19-based traffic forwarding. Later, the CN Tunnel Info (on N9) of PSA for this PDU Session is also provided to this I-UPF.

The clause 4.13.8 of TS 23.502 supports various kinds of procedures of the 5G LAN-type service. The sub-clause 4.13.8.2.2 specifies the group-level N4 session management procedure that references the N4 session establishment procedure (4.4.1.2) for N19-based tunnels establishment.

### 6.x.2.3	Procedure of UE Registration for UE-Sat-UE communication

The UE registration procedure needs no modification. In the existing procedure as the clause 4.2.2.2.2 of 23.502, UE provides several satellite access related parameters, e.g., Unavailability Period Duration, Start of Unavailability Period, etc., which could be leveraged for the optimization of the traffic forwarding of the UE-Sat-UE 5G VN group communication.

### 6.x.3	Impacts on services, entities, and interfaces

AF:

-	Provide a new parameter ‘UE-Sat-UE communication indication’ of the 5G VN group data

SMF:

-	Select the I-UPF for handling the UE-Sat-UE communication of a 5G VN group.

-	Enhance the mapping between a 5G VN group supporting UE-Sat-UE communication via the combination of requested DNN, S-NSSAI, and the UE-Sat-UE communication indication.

I-UPF:

-	Enhanced to handle the local-switch and/or N19-based switch for the UE-Sat-UE communication of a 5G VN group.

UDM/UDR:

-	Store the new parameter ‘UE-Sat-UE communication indication’ in the 5G VN group data for a 5G VN group.

# * * * * End of changes * * * *